ZONING ORDINANCE

FOR THE

VILLAGE OF MADISON, OHIO

Approved by the

MADISON VILLAGE PLANNING COMMISSION

Michael L. Gehring, Chairman

David G. Reed, Jr., Mayor

Charles Burgard, Council

Johnna Lynn Kopczak

Ruth B. Kuhn

Robert Densmore, former member

Adopted by the

MADISON VILLAGE COUNCIL

Michael J. Evangelista, President Pro Temp

Sam Britton, Jr.

R.R. Burnheimer

Ronald C. Smith

Charles Burgard

Justin Kopczak

David G. Reed, Jr. Mayor

John D. Sample, Administrator

Richard L. Collins, Jr., Solicitor 1996

Table of Contents

ARTICLE 1 - 	TITLE AND OBJECTIVES 	7

Section 1.1. 	Title 	7

Section 1.2. 	Objectives 	7

ARTICLE 2 - 	DEFINITIONS 	9

Section 2.1. 	Intent 	9

ARTICLE 3 - 	DISTRICTS AND GENERAL PROVISIONS 	17

Section 3.1.	Districts 	17

Section 3.2.	District Map 	17

Section 3.3.	District Boundaries 	18

Section 3.4.	Compliance with Regulations 	18

Section 3.5.	Nonconforming Uses 	19

Section 3.6.	Traffic Visibility Across Corner Lots 	20

Section 3.7.	Pending Application for Building Permits 	20

ARTICLE 4 - 	USE REGULATIONS 	21

Section 4.1.	General 	21

Section 4.2.	Use Regulations 	21

Section 4.3.	Schedule of Uses 	21

4.3.1.	Agricultural District ("A-1") 	21

4.3.2.	Special District ("S-1 ") 	22

4.3.3.	Single Family Residence District ("R-1") 	22

4.3.4.	Single Family Residence District ("R-2") 	22

4.3.5.	Multiple Family Residence District ("R-3") 	23

4.3.6.	General Business District ("B-1") 	23

4.3.7.	Highway Business District ("B-2") 	24

4.3.8.	Planned Commercial Development District ("B-3") 	25

4.3.8.(A)	Interstate Business District ("B-4")	25

4.3.8.(B)	Interstate Business-Industrial District (“B-5”)	25

4.3.9.	Limited Industry District ("M-1") 	26

4.3.10.	General Industry District (''M-2") 	27

4.3.11.	Senior Citizens Residence District ("SC-1") 	27

ARTICLE 5 - 	HEIGHT, LOT AREA AND FLOOR AREA REQUIREMENTS 	29

Section 5.1.	General 	29

Section 5.2.	Height, Lot Area and Floor Area Requirements - Agricultural

and Residential Uses 	29

Section 5.3.	Height and Lot Area Requirements - Special, Business

and Industrial Uses 	32

Section 5.4.	Height and Lot Area Requirements - Shopping

Center Uses 	33

Section 5.5.	Height, Lot Area, Design and Development Requirements -

Senior Citizens Residential Uses 	33

ARTICLE 6 - 	EXCEPTIONS AND MODIFICATIONS 	39

Section 6.1.	General 	39

Section 6.2.	Existing Lots of Record 	39

Section 6.3.	Height 	39

Section 6.4.	Front Yards 	40

Section 6.5.	Side Yards 	40

Section 6.6.	Rear Yards 	41

Section 6.7.	Setback from Publicly Established Drainage Ditches 	41

Section 6.8.	Landscaping and Screening Provisions 	41

ARTICLE 7 - 	OFF-STREET PARKING AND LOADING REQUIREMENTS 	42

Section 7.1. 	Off-Street Loading Requirements 	42

Section 7.2. 	Off-Street Parking Space Requirements 	42

Section 7.3. 	Special Parking Provisions 	45

Section 7.4. 	Development and Maintenance of Parking Areas 	45

ARTICLE 8 - 	SPECIAL PROVISIONS 	47

Section 8.1.	Performance Requirements 	47

Section 8.2.	Trailers and Motor Homes 	47

Section 8.3.	Temporary Buildings 	47

Section 8.4.	Shopping Centers 	48

Section 8.5.	Occupancy Permit in "B-3" Districts 	48

Section 8.6	Transmission Towers 	48

ARTICLE 9 - 	SIGNS 	51

Section 9.1.	Statement of Purpose 	51

Section 9.2.	Definition 	51

Section 9.3.	Permits, Fees and Inspections 	51

Section 9.4.	General Regulations 	51

Section 9.5.	Real Estate Signs 	52

Section 9.6.	Home Occupation Identification Signs 	52

Section 9.7.	Public, Educational, or Religious Institution Identification Signs	52

Section 9.8.	Single Family and Duplex Residence Identification Signs 	53

Section 9.9.	Subdivision and Multi-Family Residence Identification Signs . 	53

Section 9.10.	B-1 District Signs 	53

Section 9.11.	B-2, M-1 and M-2 District Signs 	55

Section 9.12.	B-3 District Signs 	55

Section 9.12.(A)	B-4 District Signs	56

Section 9.12.(B)	B-5 District Signs	56

Section 9.13.	S-1 District Signs 	56

Section 9.14.	Entrance and Exit Signs 	56

Section 9.15.	Service Station Signs 	56

Section 9.16.	Temporary Signs	56

Section 9.17.	Political Signs 	57

Section 9.18.	Removal of Signs 	58

Section 9.19.	Joint Occupancy or Multiple Use 	59

Section 9.20.	Application and Fee 	59

Section 9.21.	Nonconforming Signs 	59

Section 9.22.	Penalty 	60

ARTICLE 10-	FENCES………………………………………………………………….61

Section 10.1.	General Welfare 	61

Section 10.2.	R-1, R-2, R-3, B-1 and SC-1 Zoning Districts 	61

Section 10.3.	B-2, B-3, B-4, B-5, M-1 and M-2 Zoning Districts 	63

Section 10.4.	Permit Fees 	65

Section 10.5.	Compliance 	65

Section 10.6.	Public Grounds 	65

Section 10.7.	Penalty 	65

ARTICLE 11 - 	SATELLITE DISH RECEIVING STATIONS 	66

Section 11.1.	Definition 	66

Section 11.2.	Applicability 	66

Section 11.3.	Location 	66

Section 11.4.	Size 	66

Section 11.5.	Prohibitions 	66

Section 11.6.	Approval	67

Section 11.7.	Penalty	67

ARTICLE 12 - 	POOLS 	68

Section 12.1.	Definitions 	68

Section 12.2.	Zoning Certificate and Approval 	68

Section 12.3.	Fees 	68

Section 12.4.	Polluted Water 	68

Section 12.5.	Shielding Lights 	68

Section 12.6.	Unnecessary Noise 	69

Section 12.7.	Commercial Activities 	69

Section 12.8.	Enclosure of Swimming Pools 	69

Section 12.9.	Location of Pools 	69

ARTICLE 13 - 	GAS WELLS AND OIL WELLS 	70

Section 13.1. 	Location 	70

Section 13.2. 	Requirements 	70

Section 13.3. 	Penalty 	75

ARTICLE 14 - 	GAME ROOMS 	76

Section 14.1. 	Definitions 	76

Section 14.2. 	Requirements 	76

Section 14.3. 	Penalty 	77

ARTICLE 15 - 	PLANNING COMMISSION 	78

Section 15.1.	Organization and Procedures 	78

Section 15.2.	Conditional Use Permits 	79

Section 15.3.	Standards for Evaluating Conditional Use Permits 	79

Section 15.4.	Approval of Conditional Use Permits 	80

Section 15.5.	Revocation of Conditional Use Permits 	80

Section 15.6	Platting Commission  	80

ARTICLE 16 - 	BOARD OF ZONING APPEALS 	81

Section 16.1.	Organization and Procedures 	81

Section 16.2.	Powers of the Board of Zoning Appeals 	82

Section 16.3.	Applications and Appeals 	84

Section 16.4.	Hearings 	85

Section 16.5.	Decisions 	85

Section 16.6.	Appeals from Board of Zoning Appeals to Council 	85

ARTICLE 17 - 	ENFORCEMENT, ZONING CERTIFICATES AND FEES 	87

Section 17.1.	Zoning Inspector 	87

Section 17.2.	Zoning Certificates 	87

Section 17.3.	Conditions under which Zoning Certificates are Required	87

Section 17.4.	Application and Issuance of Zoning Certificates 	88

Section 17.5.	Fees for Zoning Certificates 	88

Section 17.6.	Zoning Certificates for Nonconforming Uses 	89

Section 17.7.	Violations and Penalties 	89

Section 17.8.	Violations and Remedies 	90

ARTICLE 18 - 	DISTRICT CHANGES AND ORDINANCE AMENDMENTS 	91

Section 18.1.	General 	91

Section 18.2.	Initiation of Amendments 	91

Section 18.3.	Action by Planning Commission 	91

Section 18.4.	Action by Council 	92

Section 18.5.	Application Fees 	92

ARTICLE 19 - 	ZONING DISTRICT MAP 	93

ARTICLE 20 - 	INTERPRETATION, PURPOSE AND CONFLICT 	93

Section 20.1. 	General 	93

ARTICLE 21 - 	SEPARABILITY 	93

ARTICLE 22 - 	EFFECTIVE DATE 	93

ARTICLE 23 - 	REPEAL OF ORDINANCE NO. 1-1963, AS AMENDED 	93

ARTICLE 24 - 	OPEN MEETING 	94

ORDINANCE NO. 33 - 1996

AN ORDINANCE ESTABLISHING COMPREHENSIVE ZONING REGULATIONS AND A DISTRICT MAP FOR THE VILLAGE OF MADISON, OHIO, AND PROVIDING FOR THE ADMINISTRATION, ENFORCEMENT AND AMENDMENT THEREOF, ALL FOR THE PURPOSE OF PROMOTING THE PUBLIC HEALTH, SAFETY, MORALS, COMFORT AND GENERAL WELFARE OF THE RESIDENTS OF THE VILLAGE, THROUGH THE REGULATION OF THE USE OF LAND AND BUILDINGS, THE LOCATION, BULK AND HEIGHT OF BUILDINGS AND STRUCTURES BY ESTABLISHING VARIOUS DISTRICTS WITHIN WHICH CERTAIN REGULATIONS SHALL APPLY, AND REPEALING ORDINANCE NO. 1-1963 AND ALL AMENDMENTS THERETO.

WHEREAS, Ohio enabling legislation for Planning and Zoning within municipal corporations is provided in Sections 713.01 through 713.27 of the Ohio Revised Code; and,

WHEREAS, the Madison Village Planning Commission conducted a comprehensive study and review of the 1963 zoning ordinance, map and all amendments thereto, examined the current land use and development within the Village and considered general planning goals and objectives for the future of the Village; and,

WHEREAS, as a result of said study and review, the Planning Commission, after public hearing, recommended that Ordinance No. 1 -1963 and all amendments thereto be repealed and, that a new Codified Zoning Ordinance for Madison Village be adopted; and,

WHEREAS, Madison Village Council, after proper legal public notice and public hearing, desires to accept the recommendation of the Planning Commission and repeal Ordinance No. 1-1963 and all amendments thereto and create a new Codified Zoning Ordinance for Madison Village.

NOW, THEREFORE, BE IT ORDAINED BY THE COUNCIL OF THE VILLAGE OF MADISON, COUNTY OF LAKE AND STATE OF OHIO, THAT:

ARTICLE 1 - TITLE AND OBJECTIVES

Section 1.1. Title

These rules, regulations, procedures and maps shall be known and cited as the Madison Village Zoning Ordinance.

Section 1.2. Objectives

For the purpose of promoting the public health, safety, morals, comfort and general welfare of said village and its residents the provisions of this Ordinance shall be applied as follows:

to protect the character and values of the residential, business, service, industrial, agricultural, institutional and public uses and to assure their orderly and beneficial development; and to improve the public safety and to lessen congestion by locating buildings and uses in relation to streets so as to cause the least interference with and the least damage by traffic movements; and to provide for public facilities and utilities such as water

supply, a sewer system and transportation systems as well as developments for recreation, schools, and other public facilities; and to encourage the most appropriate uses of the land and guide the future development of the Village in accordance with a Comprehensive Plan.

ARTICLE 2 - DEFINITIONS

Section 2.1. Intent

For the purpose of this Ordinance the following terms shall have, throughout this text, the meaning given herein; words used in the present tense shall include the future; the singular shall include the plural and the plural the singular; the word "shall" is mandatory and not directive; the word "may" is permissive; the word "Village" shall mean the Village of Madison, Ohio; the term "Commission" shall mean the Planning Commission of said Village; the term "Board" shall mean the Board of Zoning Appeals of said Village; and, the term "Council" shall mean the Council of said Village.

2.1.1.	Accessory Use: A use or building located on the same lot occupied by the main use or buildings which is naturally and normally incidental to, subordinate to and devoted exclusively to the main use of the land or buildings. Accessory uses include garages and home occupations, as herein defined.

2.1.2.	Adult Care Facility: An adult family home or adult group home that provides accommodations and supervision for three (3) to sixteen (16) unrelated adults, at least 3 of whom are provided personal care services. It does not include a nursing home, rest home for the aged, an alcohol treatment center, a rehabilitation center, a residential facility for the mentally ill, a drug treatment facility, a facility licensed by the Department of Mental Retardation and Developmental Disabilities, an assisted living facility, a Veterans' Administration facility or a facility that provides only housing, housekeeping, laundry, meal preparation, social and recreational activities and similar services that are not personal care services or skilled nursing care.

2.1.3.	Adult Living Facility: A home for three (3) to sixteen (16) unrelated adults that provides supervision and accommodations such as housing, housekeeping, laundry, meal preparation, social and recreational activities and similar services that are not personal care services or skilled nursing care.

2.1.4.	Agriculture: Any agricultural use, such as and including farming, dairying, pasturage, animal and poultry husbandry, agriculture, horticulture, floriculture, viticulture and greenhouses for the propagation of nursery stock for the owner's personal uses. Agricultural purposes as used herein shall not include the operation of a commercial greenhouse, retail or wholesale, or commercial plant cultivation or specialized animal raising and care.

2.1.5.	Apartment: A room or suite of rooms intended, designed or used as a residence by a single family.

2.1.6.	Apartment Building: A building designed for or containing apartments or suites of rooms for residence.

2.1.7.	Assisted Living Facility: A multiple-unit residential facility, as defined and limited by Ohio Revised Code Chapter 3726, that provides or arranges for skilled nursing care for one (1) or more individuals who reside in the facility and are not related to the owner or operator of the facility or his spouse as a parent, grandparent, child, sibling, niece, nephew, aunt, uncle, or child of an aunt or uncle.

2.1.8.	Automotive Repair: The repair, rebuilding or reconditioning of motor vehicles, or parts thereof, including collision service, painting and steam cleaning.

2.1.9.	Automotive Sales: The sale or rental of new or used vehicles or trailers.

2.1.10. 	Automotive Salvage: The dismantling, storage, sale or dumping of used motor vehicles, trailers or parts thereof.

2.1.11. 	Basement: A story all or partly below ground, but having at least one half of its height below average grade of the adjoining ground. A basement shall be considered a story if used for dwelling purposes.

2.1. 12. 	Bed and Breakfast: A single family dwelling within which individual guest rooms are available for meals and lodging for compensation and by prior arrangement for not more than four (4) transient guests for not more than seventy-two (72) consecutive hours. Said use shall not allow a sign or display visible from the outside of the dwelling indicating the existence of such use, shall not generate traffic in greater volume than would normally be expected in a residential neighborhood and shall comply with all off-street parking requirements.

2.1.13. 	Building: Building shall include the word "structure" unless otherwise noted, and is a structure, permanently affixed to or permanently located on the land, having one or more floors and a roof, being bounded by either open spaces or lot lines and used as a shelter or enclosure for persons, animals and/or property.

2.1.14. 	Building. Front Line of: The line of that face or front of the building nearest the front line of the lot. This face includes sun parlors, decks, and covered porches, whether enclosed or unenclosed, but does not include steps.

2.1.15. 	Building. Height of: The vertical distance from the grade to the highest point of the coping of a flat roof or to the deck line of a mansard roof or to the mean height level between eaves and ridge for gable, hip and gambrel roofs.

2.1.16. 	Building. Principal: A building in which is conducted the main or principal use of the lot on which said building is situated.

2.1.17. 	Business Services: Any activity conducted for gain which renders services primarily to other commercial or industrial enterprises or which services and repairs appliances, equipment and machines used in homes or businesses.

2.1.18. 	Child Daycare Services: A service administering to the needs of three (3) or more children under the age of fifteen (15), or under the age of eighteen (18) if the child is mentally retarded or emotionally handicapped, for part of the 24 hour day by a person or persons other than his caretaker, parent or guardian.

2.1.19. 	Clinic: An establishment operated or occupied by four (4) or more individuals engaged in professional services.

2.1.20. 	Commercial Plant Cultivation: The cultivation of crops, horticulture, floriculture and viticulture including fruit trees, nursery stock, truck garden products and similar plant materials for commercial purposes, including the operation of a commercial greenhouse, retail or wholesale.

2.1.21. 	Conditional Use: An uncommon, unique or infrequent use which shall not be permitted by right but may be permitted in certain districts under specific conditions or prohibited by the Commission after making a determination for conformance with the procedures and standards established in Article 15. All such uses shall require a Conditional Use Permit.

2.1.22. 	Dwelling: Any non-moveable building or portion thereof which is designed for or used for residential purposes.

2. 1.23. 	Dwelling. Single Family: A building designed for or occupied exclusively by one (1) family.

2.1.24. 	Dwelling. Two-Family: A building designed for or occupied exclusively by two (2) families.

2. 1.25. 	Dwelling. Multiple: A building designed for or occupied exclusively by three (3) or more families.

2.1.26. 	Entertainment Facilities: A facility for an activity conducted for gain which is generally related to the entertainment field, such as motion picture theaters, bowling alleys, roller skating rinks, miniature golf, golf driving ranges, commercial swimming pools, carnivals and related uses.

2.1.27. 	Essential Services: The erection, construction, alteration or maintenance, by public utilities or municipal or other governmental agencies, of underground or overhead gas, electrical, steam or water transmission or distribution systems, collection, communication, supply or disposal systems, including poles, wires, drains, sewers, pipes, conduits, cables, fire alarm boxes, police call boxes, traffic signals, hydrants and other similar equipment and accessories in connection therewith reasonably necessary for the furnishing of adequate service by such public utilities or municipal or other governmental agencies or for the public health, safety and welfare, but not including buildings other than structures for the purpose of housing the essential services named herein.

2.1.28. 	Family: One (1) or more persons occupying a dwelling and living as a single housekeeping unit.

2.1.29. 	Financial Services: Banks, savings and loan companies, savings and loan associations, credit unions and small loan companies.

2.1.30.	Frontage. The size of a lot measured along the side of a street except a lot on a cul de sac which shall be measured at the building line.

2.1.31.	Garage: An accessory building designed or used for the storage of motor vehicles owned and used by the occupants of the building to which it is accessory. Not more than one (1) of the vehicles may be a commercial vehicle of not more than two (2) ton capacity.

2.1.31(A)	Gasoline Sales/Convenience Stores:  Any building, structure or land used for the purposes of dispensing, selling or offering for sale at retail any automobile fuels, oils or accessories, not including repair work, and/or selling or offering for sale at retail any food, beverage, or commodity commonly found at a convenience store.  (Amended by Ordinance No. 07-1998, Passed March 16, 1998:  An Ordinance amending Ordinance No. 33-1996, by amending Section 2.1as it relates to said New Zoning District (B-4).)

2.1.32. 	Gasoline Service Station: Any building, structure or land used for the dispensing, sale or offering for sale at retail of any automobile fuels, oils, or accessories, including lubrication of automobiles and replacement or installation of minor parts and accessories but not including major repair work, such as motor replacement, body and fender repair or spray painting.

2.1.33. 	General Manufacturing: The process of manufacturing, compiling, fabricating assembling or otherwise producing a product from materials or components such as asphalt, concrete and cement making, bleaching, cleaning and dyeing plants; boiler, machine and structural steel fabricating shops; brewing or distilling of liquors; brick, pottery, tile manufacturing; candle manufacturing; coal, coke and tar products, fertilizers and gelatin; making of paint, lacquer and enamel; foundry works, freight truck yards; lime products; meat packing, melting and alloying of metals; plaster and drywall; reduction, refining smelting of ferrous metals or metal ore; refining petroleum products; rolling mills; rubber; soaps, sodium compounds; and, similar uses and operations.

2.1.34.	Home Occupation: Any occupation, trade, profession or business carried on by a member of the immediate family, residing on the premises, provided there is no commodity sold upon the premises and no mechanical equipment is used except of a type that is similar in character to that normally used for purely domestic or household purposes; and provided that no display will indicate from the exterior that the building or land is being utilized in part for any purpose other than that of the dwelling except as otherwise provided in Section 9.6. of this ordinance.

2.1.35.	Junk Salvage Yard: Any building and/or open space where waste or discarded materials are stored, processed or sold.

2.1.36. 	Level: Any floor of a building, all of which lies in the same horizontal plane throughout.

2.1.37.	Light Manufacturing: The process of manufacturing, compiling, fabricating, assembling or otherwise producing a product from materials or components such as boat building; book bindery, bottling plant; box factory; builders supply and storage of building materials; bus storage; cabinet making; canned; carpenter shop; cesspool pumping, cleaning and draining; contractor's equipment; dainty; distributing plants; dry goods; electric appliance assembly; electrical parts; engines; engraving; fixtures; food processing; furniture and glass; industrial design; lumber yard; machine shops; metals; motors; plumbing; printing and engraving; roofing contractor; sheet metal shop; shooting, rifle and pistol ranges; textiles; tools and dies; toys; trailers; truck storage, rental or terminal; valves, storage warehouse; repair welding; wood; and, similar uses and operations.

2. 1.38. 	Loading Space: A space within the main building, or on the same lot therewith, providing for the standing, loading or unloading of trucks and having a minimum dimension of twelve (12) by fifty-five (55) feet and a vertical clearance of at least fourteen feet (14').

2.1.39. 	Lot: A parcel of land having frontage upon a street which individually or as a part of a subdivision has been duly recorded in the Office of the Recorder of Lake County.

2.1.40. 	Lot. Minimum Area of: The area of a lot computed exclusive of any portion of the right-of-way of any public thoroughfare.

2.1.41.	Manufacturing: The process of manufacturing, compiling, fabricating, assembling or otherwise providing a product from materials or components.

2.1.42. 	Motel: A building or buildings in which temporary, lodging or temporary boarding and lodging are provided and offered to at least twenty (20) members of the transient public for compensation and supervised by a person in charge at all hours. As such, it is open to the public in contradistinction to a boarding house, a lodging house or an apartment.

2.1.43. 	Non-Conforming Use: The use of land or a building, or portion thereof, which use does not conform with the use regulations of the district in which it is situated.

2. 1.44. 	Nursing Home: A home used for the reception and care of individuals, who by reason of illness or physical or mental impairment require skilled nursing care, and of individuals who require personal assistance but not skilled nursing care. A nursing home is licensed to provide personal assistance and skilled nursing care.

2.1.44(A)	Offices:  Any building or structure used for the directing headquarters of an enterprise or organization or for the rendering of professional services.  (Amended by Ordinance No. 07-1998, Passed March 16, 1998:  An Ordinance amending Ordinance No. 33-1996, by amending Section 2.1 as it relates to said new zoning district (B-4).)

2. 1.45. 	Off-Street Parking Space: Any parking space located wholly off any street, alley or sidewalk, either in an enclosed building or on an open lot and where each parking space has an area of not less than two hundred (200) square feet exclusive of access drives or aisles.

2.1.46.	Park: A piece of ground set apart and maintained for public use, and laid out in such a way as to afford pleasure to the eye as well as opportunity for open air recreations.

2.1.47. 	Personal Services: Any enterprise conducted for gain which primarily offers services to the general public such as shoe repair, valet service, watch repair, barber shops, beauty parlors and related activities.

2. 1.48. 	Plant Cultivation: The non-commercial cultivation of crops, horticulture, floriculture and viticulture including fruit trees, nursery stock, garden products and similar plant materials for the personal use of the property owner.

2. 1.49. 	Professional Services: The use of offices and related spaces for such professional services as are provided by doctors, dentists, lawyers, accountants, architects, insurance agents, engineers and realtors.

2. 1.50. 	Public Uses: Any use conducted by a governmental entity for public purposes.

2.1.51. 	Public Service Facility: A facility for any service operating under authority granted by a governmental body.

2.1.52. 	Recreational Facilities: Any building, area or use where members of the public may engage in sports or similar recreational activities.

2.1.53. 	Residential Floor Area: The interior floor area of a dwelling including stairways, halls and closets but not including basements, porches, garages or carports.

2.1.53(A)	Restaurants:  Any building or structure where, in consideration of the payment of money, meals are prepared, sold and served to the public on a regular basis.  (Amended by Ordinance No. 07-1998, Passed March 16, 1998:  An Ordinance amending Ordinance No. 33-1996, by amending Section 2.1 as it relates to said new zoning district (B-4).)

2.1.54. 	Retail Business: Any business normally found in a central business district.

2.1.55. 	Right-of-Way: A strip of land taken or dedicated for use as a public way. In addition to the roadway, it normally incorporates the curbs, lawn strips, sidewalks, lighting and drainage facilities, and may include special features (required by the topography or treatment) such as grade separation, landscaped areas, viaducts and bridges, usually under the control of the State or local government authority.

2.1.56. 	Right-of-Way Line: A dividing line between a lot, tract or parcel of land and a contiguous street.

2.1.57. 	Semipublic Uses: Churches, Sunday schools, parochial schools, colleges, hospitals and other institutions of an educational, religious, charitable or philanthropic nature.

2. 1.58. 	Sign: Any advertisement, announcement, mark of identification, plaque, historical marker, graphic or symbol attached to, placed on or painted upon any building, structure, or land, which display is visible beyond the boundaries of the lot. Such displays attached to movable trailers, or frames, shall be construed to be signs for the purposes of this ordinance.

2.1.59. 	Social Activities: A use in a building or portion thereof, or on premises owned or operated by a corporation, association, person or persons for a social, educational or recreational purpose, but not primarily for profit, or to render a service which is customarily carried on as a business.

2.1.60. 	Specialized Animal Raising and Care: The use of land and buildings for the raising and care of fur-bearing animals such as rabbits and domestic pets; the stabling and care of horses; animal kennels; pigeon raising; and, the raising of any other domestic animals or birds of a similar nature.

2.1.61. 	Storage Building: A detached portable or permanent structure designed or constructed for the storage of seasonal tools and implements normally found necessary in conjunction with the care and upkeep of property surrounding a dwelling, exceeding one hundred (100) sq. feet in area.

2.1.62. 	Storage Shed: A detached portable or permanent structure designed or constructed for the storage of seasonal tools and implements found necessary in conjunction with the care and upkeep of property surrounding a dwelling, but not exceeding one hundred (100) sq. feet in area.

2.1.63. 	Story: That portion of a building, other than a basement, not used for dwelling purposes, included between the surface of any floor and the surface of the floor next above it, or, if there be no floor above it, then the space between the floor and the ceiling next above it.

2.1.64. 	Structure: Anything constructed or erected, the use of which requires more or less a permanent location on the ground or attached to something having a permanent location on the ground.

2.1.65. 	Swimming Pool. Private: Any pool, pond, lake or open tank, where swimming is permitted, not located within a completely enclosed building and containing or normally capable of containing water to a depth at any point greater than twenty-four inches (24") and having at least one hundred fifty (150) square feet of surface.

2.1.66. 	Tavern: A saloon or place primarily and predominately used for the retail sale of beer, wine and liquor, though food is usually served.

2.1.67. 	Transmission Tower: A tower used in connection with the operation of the transmission or receiving of radio, television, mobile telephone, facsimile, or siren broadcasting by a commercially operated broadcasting station licensed by the Federal Communications Commission or other governmental agency.

2.1.68. 	Warehouse: A building used entirely or mainly for the storage of goods in bulk or large quantities for persons other than the owner of the warehouse.

2.1.69. 	Yard: An open space at grade between a building and the adjoining lot lines, unoccupied and unobstructed by any portion of a structure from the ground upward, except as otherwise provided herein. In measuring a yard for the purpose of determining the width of a side yard, the depth of a front yard or the depth of the rear yard, the minimum horizontal distance between the lot lines and the main building shall be used.

2.1.70. 	Yard. Front: A yard extending across the front of a lot between the side lot lines and being the minimum horizontal distance between the right of way line and the main building or any projections thereof. On corner lots, the front yard shall be considered as parallel to the street upon which the lot has its least dimension.

2.1.71. 	Yard. Rear: A yard extending across the rear of a lot and being the required minimum horizontal distance between the rear lot line and the rear of the main building or any projections thereof.

2.1.72. 	Yard. Side: A yard between the main building and the side line of the lot and extending from the required front yard to the required rear yard and being the minimum horizontal distance between a side lot and the side of the main building or any projections thereof.

2.1.73. 	Zoning Certificate: The document issued by the Zoning Inspector authorizing the use of land or buildings.

2.1.74. 	Zoning District Map: The zoning district map or maps of the Village together with all amendments thereto subsequently adopted.

2.1.75. 	Zoning Inspector: The Zoning Inspector or his authorized representative appointed by the Council of the Village.

ARTICLE 3 - DISTRICTS AND GENERAL PROVISIONS

Section 3.1. Districts

The incorporated area of the Village of Madison, Ohio, is hereby divided into districts, known as:

"A-1"	Agricultural District

"S-1"	Special District

"R-1"	Single Family Residence District

"R-2"	Single Family Residence District

"R-3"	Multiple Family Residence District

"B-1"	General Business District

"B-2"	Highway Business District

"B-3"	Planned Commercial Development District

"B-4"	Interstate Business District

“B-5”                                                                        Interstate Business-Industrial District

"M-1"	Limited Industry District

"M-2"	General Industry District

"SC-1" 	Senior Citizens Residence District

(Amended by Ordinance No. 54-1999, Passed January 3, 2000:  An Ordinance amending Ordinance

No. 33-1996, by amending Section 3.1 so as to add a “B-5” Interstate Business-Industrial District.)

Section 3.2. District Map

The boundaries of the districts are shown upon the map which is made a part of this

Ordinance, which map is designated as the "Zoning District Map". The district map and all the

notations, references and other information shown thereon are a part of this Ordinance and have the

same force and effect as if the district map and all the notations, references and other information

shown thereon were all fully set forth or described herein, the original of which district map is

properly attested to and is on file with the Clerk-Treasurer of Council.

No amendment to this Ordinance which involves matters portrayed on the Official

Zoning District Map shall become effective until after such change and entry has been made on said

map, at the direction of Council.

No changes of any nature shall be made on the Official Zoning District Map or matters

shown thereon except in conformity with the procedures set forth in this Ordinance.

Regardless of the existence of purported copies of the Official Zoning District Map which

may from time to time be made or published, the Official Zoning District Map, which shall be

located in the office of the Clerk-Treasurer of Council, shall be the final authority as to the current

zoning status of land and water areas, buildings and other structures.

In the event that the Official Zoning District Map becomes damaged, destroyed or lost,

the Council may, by resolution, adopt a new Official Zoning District Map which shall supersede the

prior Official Zoning District Map. The new Official Zoning District Map may correct drafting or

other errors or omissions in the prior Official Zoning District Map, but no such corrections shall have the effect of amending the original Ordinance or subsequent amendments thereof unless the procedures for amendment prescribed herein shall have been followed.

Section 3.3. District Boundaries

3.3.1. 	The district boundary lines on said map are intended to follow either streets or alleys or lot lines; and, where the districts designated on the map are bounded approximately by such street, alley or lot lines, the street or alley or lot shall be construed to be the boundary of the district, unless such boundary is otherwise indicated on the map. In the case of unsubdivided property, the district boundary lines shall be determined by the use of the dimensions appearing on the Zoning District Map or the district boundary shall follow property lines.

3.3.2. 	Where the boundary of a district follows a railroad line, such boundary shall be deemed to be located on the right-of-way line of said railroad line.

3.3.3. 	Whenever any street, alley or other public way is vacated by official action of the Council, the zoning district adjoining each side of such street, alley or public way shall be automatically extended to the center of such area vacated, and all such area shall then and henceforth be subject to all appropriate regulations of the extended districts.

Section 3.4. Compliance with Regulations

The regulations set by this Ordinance within each district shall be minimum regulations

and shall apply uniformly to each class or kind of structure or land, and particularly, except as

hereinafter provided:

3.4.1.	No building, structure or land shall hereafter be used or occupied, and no building or structure or part thereof shall hereafter be erected, constructed, reconstructed, moved or structurally altered unless in conformity with all of the regulations herein specified for the district in which it is located.

3.4.2. 	No building or other structure shall hereafter be erected or altered:

(a) to exceed the height;

(b) to accommodate or house a greater number of families;

(c) to occupy a greater percentage of lot area:

(d) to have narrower or smaller rear yards, front yards, side yards or other open spaces; than herein required or in any other manner contrary to the provisions of this Ordinance.

3.4.3. 	No part of a yard or other open space or off-street parking or loading space required about or in connection with any building for the purpose of complying with this Ordinance shall be included as part of a yard, open space or off-street parking or loading space similarly required for any other building.

3.4.4. 	No yard or lot existing at the time of passage of this Ordinance shall be reduced in dimension or area below the minimum requirements set forth herein. Yards or lots created after the effective date of this Ordinance shall meet at least the minimum requirements established by this Ordinance.

Section 3.5. Nonconforming Uses

3.5.1. 	Any lawful use of any dwelling, building, structure or land existing at the effective date of this Ordinance may be continued, even though such use does not conform to the provisions hereof. A non-conforming use of a building may only be changed to a conforming use. Whenever a non-conforming use has been changed to a conforming use, such use shall not thereafter be changed to a less restricted use. The non-conforming use of a building may be extended throughout those parts thereof which were manifestly arranged or designed for such use at the time of adoption of this Ordinance.

3.5.2. 	Whenever the use of a building or land shall become non-conforming through a change in the Zoning Ordinance or in the district boundaries, such use may be continued.

3.5.3. 	A non-conforming use of a building or land or portions thereof which are hereafter voluntarily discontinued for a continuous period of one (1) year shall not again be used except in conformity with the regulations of the district in which such building or land is located.

3.5.4. 	Any building arranged, intended or designed for a non-conforming use, the construction of which has been started at the time of the passage of this Ordinance, but not completed, may be completed and put into such non-conforming use, provided it is done within one (1) year after this Ordinance takes effect.

3.5.5. 	A non-conforming building which has been damaged by fire, explosion, act of God or the public enemy may be restored to its prior non-conforming use and condition, provided such repairs or reconstruction are completed within one (1) year of the date of such damage.

3.5.6. 	Any dwelling, building, structure or land in a General Business District (B-1) which is used as a lawful nonconforming residential (R-1 or R-2) use pursuant to this section may also be used for General Business (B-1) permitted uses, accessory uses and conditional uses in conformity with the regulations of the General Business District (B-1) without forfeiting the right to continue the nonconforming use PROVIDED, HOWEVER, that such conforming permitted 	uses, accessory uses and conditional uses shall first be approved by the Planning Commission and shall be in compliance with all regulations and requirements applicable to General Business District laws and ordinances. Such approval shall be in the form of a conditional use permit and the procedure, standards and regulations for obtaining and maintaining said conditional use permit shall be in accordance with Article 15 of this Ordinance.

Section 3.6. Traffic Visibility Across Corner Lots

In any district on any corner lot, no fence, structure or planting shall be erected or

maintained within twenty (20) feet of the "corner" of intersecting right-of-way lines at a height of

more than three (3) feet above curb or street grade or so as to interfere with traffic visibility across

the corner.  No fence, structure or planting shall be erected or maintained within the public right

of-way.

Section 3.7. Pending Application for Building Permits

Nothing set forth herein shall require any change in the overall layout, plans,

construction, size or designated use of any development, building, structure or part thereof for which

official approvals and required building permits have been granted before the enactment of this

Ordinance, the construction of which, conforming with such plans, shall have been started prior to

the effective date of this Ordinance and completion thereof carried on in a normal manner within the

subsequent six (6) month period and not discontinued until completion, except for reasons beyond

the builder's control.

ARTICLE 4 - USE REGULATIONS

Section 4.1. General

The permitted and conditional uses for each district are shown in Article 4, Section 4.3.

Uses not specifically listed or interpreted by the Board of Zoning Appeals to be included under this

Article shall not be permitted except by amendment to the Zoning Ordinance.

Section 4.2. Use Regulations

Buildings and land shall be used, and buildings shall be designed, erected, altered, moved

or maintained, in whole or in part, in all zoning districts only for the uses set forth in the following

schedule and regulations of this Ordinance:

4.2.1. 	The Main Building and Uses set forth as permitted uses in the schedule, Section 4.3., shall be permitted by right as the principal building or use of a lot only in a district in which it is specifically permitted.

4.2.2. 	The Accessory Buildings and Uses set forth as permitted accessory uses in the schedule, Section 4.3., and further regulated in subsequent sections, shall be permitted as a subordinate building or use if it is clearly incident to and if located on the same lot as the main building or use. If set forth as a conditional accessory use, it shall be permitted only under the conditions upon which it is approved.

4.2.3.	 The Conditional Uses set forth as such in the schedule, Section 4.3., shall not be permitted by right. Such uses may be permitted under specific conditions or prohibited in certain locations by the Commission after making a determination for conformance with standards set forth in Section 15 of this Ordinance. If approved, a Conditional Use Permit shall be granted.

Section 4.3. Schedule of Uses

4.3.1. Agricultural District ("A-1")

Permitted Uses

1. Agriculture

2. Plant Cultivation

3. Essential Services

Accessory Uses

1. Single Family Dwellings when used by the operators of the agricultural use

2. Barns

3. Garages

4. Storage Sheds

5. Storage Buildings

Conditional Uses Requiring Commission Approval

1. Specialized animal raising and care

2. Oil and Gas wells

3. Sand, gravel and topsoil extraction

4. Commercial Plant Cultivation

4.3.2. Special District ("S-1")

Permitted Uses

1. Parks

2. Public uses

3. Semipublic uses

4. Essential Services

Accessory Uses

None

Conditional Uses Requiring Commission Approval

1. Public Service facilities

2. Recreational facilities

3. Cemeteries

4.3.3. Single Family Residence District ("R-1")

Permitted Uses

1. Single Family Dwellings

2. Essential Services

Accessory Uses

1. Garages

2. Private Swimming Pools

3. Storage Sheds

4. Storage Buildings

Conditional Uses Requiring Commission Approval

1. Home Occupations

2. Child Day-Care

4.3.4. Single Family Residence District ("R-2")

Permitted Uses

1. Single Family Dwellings

2. Essential Services

Accessory Uses

1. Garages

2. Private Swimming Pools

3. Storage Sheds

4. Storage Buildings

Conditional Uses Requiring Commission Approval

1. Home Occupations

2. Nursing Homes

3. Semipublic Uses

4. Agriculture

5. Two-Family Dwellings

6. Specialized Animal Raising and Care

7. Child Daycare

8. Bed and Breakfast

4.3.5. Multiple Family Residence District ("R-3")

Permitted Uses

1. Single Family Dwellings

2. Two Family Dwellings

3. Multiple-Family Dwellings

4. Essential Services

Accessory Uses

1. Garages

2. Private Swimming Pools

3. Storage Sheds

4. Storage Buildings

Conditional Uses Requiring Commission Approval

1. Child Day-Care

2. Home Occupations

3. Nursing Homes

4. Public Service Facilities

5. Bed and Breakfast

4.3.6. General Business District ("B-1")

Permitted Uses

1. Retail Businesses

2. Personal Services

3. Business Services

4. Professional Activities

5. Offices

6. Restaurants

7. Financial Services

8. Public Uses

9. Semipublic Uses

10. Essential Services

11. Public Service Facilities

12. Funeral Homes

Accessory Uses

None

Conditional Uses Requiring Commission Approval

1. Taverns

2. Wholesale Businesses

3. Sale or Storage of Building Material

4. Motels

5. Recreational Facilities

6. Gasoline Service Stations

7. Clinics

8. Social Activities

9. Entertainment Facilities

10. Bed and Breakfast  ( ORD # 34-2002 )

4.3.7. Highway Business District ("B-2")

Permitted Uses

1. Gasoline Service Stations

2. Restaurants

3. Motels

4. Public Uses

5. Essential Services

6. Automotive Repairs

7. Garden Centers

8. Public Service Facilities

Accessory Uses

None

Conditional Uses Requiring Commission Approval

1. Semipublic Uses					7.  Offices

2. Taverns						8.  Social Activities

3. Wholesale Businesses				9.  Entertainment Facilities

4. Sale or Storage of Building Materials		10.  Automotive Sales

5. Animal Hospitals or Clinics

6. Retail Businesses

4.3.8. Planned Commercial Development District ("B-3")

(Commonly known as Shopping Center)

Permitted Uses

1. Retail Businesses				7. Restaurants

2. Personal Services				8. Public Uses

3. Business Services				9. Essential Services

4. Professional Activities			10. Public Service Facilities

5. Commercial Schools			11. Financial Services

6. Offices					12. Clinics

Accessory Uses

None

Conditional Uses Requiring Commission Approval

1. Sale or Storage of Building Material

2. Recreational Facilities

3. Semipublic Uses

4. Social Activities

5. Entertainment Facilities

4.3.8.(A).  Interstate Business District ("B-4")

Permitted Uses

1.  Gasoline Sales/Convenience Stores

2.  Restaurants

3.  Motels

4.  Public Users

5.  Essential Services

6.  Public Service Facilities

Accessory Uses

None

Conditional Uses Requiring Commission Approval

1.  Retail Business

2.  Offices

(Amended by Ordinance No. 07-1998, Passed March 16, 1998:  An Ordinance amending Ordinance No. 33-1996, by adding a New Section 4.3.8(A) so as to create a New Zoning District to be known as the "B-4" Interstate Business District.)

4.3.8.(B). Interstate Business-Industrial District (“B-5”)

Permit Uses

- Gasoline Sales/Convenience Stores

- Restaurants

- Motels

- Public Users

- Essential Services

Accessory Uses

None

Contional Uses Requiring Commission Approval

- Retail Business

- Office

- Light Manufacturing

- Pharmaceutical, Electrical Equipment, Plastics and Similar Uses

- Research and Development Facilities

- Public Service Facilities

- Sale and Storage of Building Materials

- Assembly

- Wholesale Businesses

- Warehousing

(Amended by Ordinance No. 54-1999, Passed January 3, 2000:  An Ordinance amending Ordinance No. 33-1996 so as to create a new Zoning District to be known as the Interstate Business-Industrial District “B-5”.)

4.3.9. Limited Industry District ("M-1")

Permitted Uses

1. Light Manufacturing

2. Pharmaceutical, Electrical Equipment, Plastics and Similar Uses

3. Contracting Firms

4. Research and Development Facilities

5. Public Service Facilities

6. Sale and Storage of Building Materials

7. Assembly

Accessory Uses

1. Warehouses

2. Offices

Conditional Uses Requiring Commission Approval

1. Wholesale Businesses

2. Warehousing

3. Oil and Gas Wells

4. Sand, Gravel and Topsoil Extraction

5. Mineral Extraction, Storage and Processing

6. Food Processing

7. Transport and Trucking Terminals

8. Retail Businesses

9. Offices

10. Commercial Plant Cultivation

11. Excavation Contracting Firms

4.3.10. General Industry District ("M-2")

Permitted Uses

1. General Manufacturing

2. Assembly

3. Research and Development Facilities

4. Public Service Facilities

5. Sale and Storage of Building Materials

6. Food Processing

7. Transport and Trucking Terminals

8. Warehousing

9. Wholesale Businesses

Accessory Uses

1. Warehouses

2. Offices

Conditional Uses Requiring Commission Approval

1. Oil and Gas Wells

2. Sand, Gravel and Topsoil Extraction

3. Mineral Extraction, Storage and Processing

4. Excavation Contracting Firms

4.3.11. Senior Citizens Residence District ("SC-1")

Permitted Uses

1. Apartment Buildings designed specifically for and used by persons of retirement age.

Accessory Uses

1. Multi-Purpose Community Centers

2. Health Centers

3. Eating Facilities

4. Counseling Offices

5. Craft and Meeting Rooms

6. Lounge Areas

7. Passive and Active Recreation Areas

8. Service and Maintenance Buildings

9. Storage Garages

10. Off-street parking areas all primarily for the use of residents of the development, their guests

and employees.

Conditional Uses Requiring Commission Approval

1. Day Care Centers

ARTICLE 5 - HEIGHT. LOT AREA. FLOOR AREA AND MISCELLANEOUS REQUIREMENTS

Section 5.1. General

The height, lot area and floor area requirements for the location and erection of buildings on any lot or tract of land are established and set forth in Section 5.2.

Section 5.2. Height. Lot Area and Floor Area Requirements for Agricultural and

Residential Uses

District Symbol	"A-1"               "R-1"              "R-2"

Maximum height of buildings,

in stories	2.5	     2.5	       2.5

Maximum height of buildings,

in feet	35	     35	       35

Minimum depth of front yard 1/

from right-of-way, in feet	75	     40	       40

Minimum width of either side yard,

in feet                                                            50                          15                      	15

Minimum depth of rear yard, in feet             50                         40	      45

Minimum lot area per family, in

square feet                                           1 acre                       20,000               20,000

Minimum lot frontage, in feet                   175                         100	    100

Minimum residential floor area,

one (1) level building only, in

square feet                                               1,500                      1,500                 1,500 ( ORD #38-2001)

Minimum residential floor area,

more than one (1) level, but

no more than three (3), in sq. ft.               1,800                     1,800                 1,800 ( ORD #38-2001)

1. A front yard of a minimum 75-foot depth shall be required for all lots fronting on Federal or State highways.

Section 5.2. (continued)

"R-3"	"R-3"	"R-3"

Single	Two

District Symbol	Family	Family

Maximum height of buildings,

in stories	2.5	 2.5

Maximum height of buildings,

in feet	 35	 35

Minimum depth of front yard 1/

from right-of-way, in feet	 40	 40

Minimum width of either side yard,

in feet	 15	 15

Minimum depth of rear yard, in feet	 40	 40

Minimum lot area per family,

dwelling unit, in square feet                     20,000              12,350

Minimum lot frontage, in feet	 100	100

Minimum residential floor area, per

dwelling, in square feet                               1,400               1,400

Minimum distance between buildings,

in feet	   30	  30

(Amended by Ordinance No. 63-2000, Passed December 4, 2000:  An Ordinance amending Ordinance

No. 33-1996, by amending Section 5.2 relating to minimum lot area per family dwelling unit, minimum

residential floor area per dwelling and multiple family requirements.)

Section 5.3. Height and Lot Area Requirements for Special. Business and Industrial Uses

Interstate

General    	Highway       Interstate     Business-           Limited           	General

District           Special   Business	    Business       Business      Industrial          Industry                  Industry

Symbol          "S-1" 2/   "B-1" 2/	   "B-2" 2/        "B-4" 2/        “B-5	“ 2/           "M-1" 2/	                 “M-2”12/

Max. height

of bldgs.,

in stories                 2	        2	2	              N/A                 N/A                   2	                                  2

Max. height

of bldgs.,

in feet                    35	      60	45	               35                     35	60                      	         60

Min. depth of front

yard, in feet (row)  50	       50		50 		 50                   80                  100                            100

Min. width of either

side yard, in feet     20	       20		20 		20	  20                 20 4/	                          20 4/

Min. depth of rear

yard, in feet            40	       40	       40	 	40		 40                50 3/	                           50 3/

Max. percentage

of lot coverage      25%     None	  40%	   40%	      40%              40%               40%

(Amended by Ordinance No. 07-1998, Passed March 16, 1998:  An Ordinance amending Ordinance No. 33-1996, by amending Section 5.3 as it relates to said new zoning district. B-4.)

(Amended by Ordinance No. 54-1999, Passed January 3, 2000:  An Ordinance amending Ordinance No. 33-1996, by amending Section 5.3 so as to add height and lot area requirements for the Interstate Business-Industrial District B-5.)

2.   No residence or dwelling unit permitted.

3.   A rear yard depth of not less than 50 feet in a "B-1" District and not less than 50 feet in a

"B-2" District and not less than 100 feet in an "M-1" District is required where a lot abuts upon

a residential district. See Article 6, Section 6.8., for exceptions.

4.   A side yard width of not less than 100 feet is required where a lot adjoins a residential

district. See Article 6, Section 6.8., for exceptions.

Section 5.4. Height and Lot Area Requirements for "B-3" Shopping Center Uses

No residence of dwelling unit permitted

Maximum height of buildings	2.5 stories above grade

Minimum depth of building set back

from right of way	100 feet

Minimum depth of paved area set back	50 feet

Minimum width of building set back,

on either side	50 feet

Minimum depth of building set back,

at rear	50 feet

Minimum feet between buildings	100 feet

Minimum lot area	2.5 acres

Minimum street frontage on a street or highway	200 feet

Minimum floor area for first phase

of a new development, grade level	7,500 square feet

Section 5.5. Height, Lot Area, Design and Development Requirements for "SC-1" Senior Citizens

Residence District.

The following height, lot area, design and development criteria are hereby established to

control the development and use of land and buildings in a Senior Citizens Residence District:

5.5.1.	Ancillary Facilities. All buildings designed for senior citizens in a Senior Citizens Residence District 	with ten (10) or less dwelling units may contain a multi purpose community center, health center, 	counseling offices, craft and meeting rooms, a lounge area, areas for passive and active recreation, and 	any and all other requirements for buildings qualifying for housing assistance for the elderly in new 	construction designed for the elderly as set forth in the regulations promulgated by the Federal 	Department of Housing and Urban Development.

All buildings designed for senior citizens in a Senior Citizens Residence District with more

than ten (10) dwelling units shall contain a multi-purpose community center, and may contain

a health center, counseling offices, craft and meeting rooms, a lounge area, areas for passive

and active recreation, and any and all other requirements for buildings qualifying for housing

assistance for the elderly in new construction designed for the elderly as set forth in the

regulations promulgated by the Federal Department of Housing and Urban Development.

5.5.2.  Special Safety and Convenience Features. All buildings designed for senior citizens in a

Senior Citizens Residence District shall have smoke detectors and/or sprinkler systems and

shall include any and all special safety and convenience features required in buildings qualifying for 	housing assistance for the elderly in new construction designed for the elderly as set forth in the 	regulations promulgated by the Federal Department of Housing and Urban Development.

5.5.3. Area and Yard Regulations. In a Senior Citizens Residence District, all buildings and land

shall be used, designed, erected, altered, moved or maintained, in whole or in part, only in

accordance with the following regulations:

a. Minimum width of lot at street right-of-way: 40 ft.

b. Minimum front yard depth: 50 ft.

c. Minimum side yard depth: 20 ft.

d. Minimum rear yard depth: 40 ft.

e. Minimum distance between main walls of separate buildings: 30 ft.

f. Minimum distance between main walls of buildings and lot line: 20 ft.

5.5.4.  Maximum Dwelling Units Per Acre. There shall be a maximum of seven (7) dwelling units

per acre in a Senior Citizens Residence District.

5.5.5.  Maximum Land Coverage. The ground floor area of all main and accessory buildings shall not

exceed twenty percent (20%) of the area of the zoned lot in a Senior Citizens Residence

District.

5.5.6.  Height Regulation. The maximum height of any main or accessory building in a Senior

Citizens Residence District shall not exceed one (1) story or eighteen feet (18') exclusive of an

elevator penthouse, chimney, a radio tower or other permitted appurtenance located upon or

constituted as an integral part of a main building.

5.5.7.	Dwelling Unit Size. The minimum area of each dwelling unit in a Senior Citizens Residence

District shall not be less than:

a. No bedroom unit: 390 sq. ft.

b. One bedroom unit: 500 sq. ft.

c. Two bedroom unit: 650 sq. ft.

5.5.8.	Building Wall. The maximum length of any building wall shall be one hundred feet (100') provided, however, that building length may extend a maximum of two hundred feet (200') if the walls are offset or aligned at angles as approved by the Planning Commission.

5.5.9.	Dwelling Units. In order to enhance privacy and encourage attractive building arrangements, the alignment of attached dwelling units shall be varied and the facades of not more than every two (2) dwelling units shall be offset by a least sixteen inches (16"). No more than eight (8) dwelling units shall be located in one (1) building.

5.5.10.	Design. Attractive variations in such building elements as facade, width, color, exterior materials and roof lines shall be deemed desirable. Parallel arrangements of buildings shall be avoided. However, uniformity in design is not expressly prohibited.

5.5.11. 	Landscape. The development shall be designed to compliment the topography of the land in order to utilize natural contours, economize the construction of utilities, reduce required grading and maximize the conservation of trees, water course and other natural features. Natural features and other distinctive characteristics of the site shall be integrated into the plan to create functional variations in the arrangements of residences, open spaces and accessory uses.

5.5.12.	Open Space. There shall be a minimum of five hundred square feet (500 sq.ft.) of open lot area per dwelling unit in a Senior Citizens Residence District.

5.5.13.	Pedestrian Circulation. There shall be a comprehensive walkway system adequately separated from vehicular traffic connecting residential buildings, parking areas, recreation areas and health center buildings in a Senior Citizens Residence District.

5.5.14.	Lighting. Exterior lighting shall be provided in a Senior Citizens Residence District for the safety and convenience of residents, their guests and employees, but shall not have excessive brightness and shall be placed in such a way so as not to cause annoying glare or reflections on neighboring premises or public streets.

5.5.15.	Parking. Parking in a Senior Citizens Residence District shall be subject to the following regulations:

a. There shall be a minimum of one (1) parking space per dwelling unit.

b. There shall be a minimum of one (1) parking space for each employee.

c. Parking spaces shall be located not less than twenty feet (20') from any building on the premises.

5.5.16.	Screening. The Planning Commission may require that a wall, solid fence and/or hedge or trees, at least five and one-half feet (5-1/2') high, shall be constructed or planted in a Senior Citizens Residence District along a lot line adjoining a Single Family Residence District.

5.5.17.	Utility Equipment. All utility lines serving a Senior Citizens Residence District shall be located underground. All rooftop mechanical equipment shall be enclosed in a manner which compliments the architectural style of the building on which it is located.

5.5.18.	Swimming Pools. Any swimming pool in a Senior Citizens Residence District in which water may collect to a minimum depth of one and one-half feet (1-1/2') shall be enclosed by a wall or fence not less than five feet (5') in height with access provided from a controlled point.

5.5.19.	Mail Boxes. There shall be no rural mail boxes. All mail boxes shall conform to federal laws and regulations, shall be approved by the United States Postal office and shall have a lock and access panel.

5.5.20.	Variation from Regulations. With respect to properties of irregular shape, unusual topography or limited size, the Planning Commission may modify requirements regarding areas, yards, buffers, and distances between uses if compliance with these requirements is either clearly not feasible or contrary to compliance with the design regulations of this Zoning Ordinance. However, the maximum permitted density and height shall not be increased by reason of this Section.

5.5.21.	Submittal of Development Plans. Four (4) copies of the proposed development plans for buildings and land in a Senior Citizens Residence District at a scale of one inch equals one hundred feet (1" = 100'), shall be submitted to the Planning Commission for review. The Planning Commission shall forward one (1) copy to the Village Engineer for review. The proposed development plans shall indicate the following:

a. Property lines and dimensions of the parcel or parcels proposed for development and the locations of existing main buildings on adjacent parcels;

b. Topography showing existing contours at not less than two foot intervals;

c. All proposed main and accessory buildings, their location, elevations, height and use;

d. The arrangement, number and size of proposed parking and loading space, the width and location of private driveways and walks;

e. The location, width, cross section and centerline profile of all access roads, private and public;

f. The location and type of proposed outdoor lighting fixtures;

g. Other site improvements, including grading and drainage, preliminary design of landscaped yards and yards adjoining a Single Family Residence District;

h. The size, location and content of proposed signs;

i. Plans for sanitary sewage and storm drainage facilities and all other utility installations and connections; and,

j. The location and size of all streams, ponds, lakes and waterways.

5.5.22. 	Fees. At the time of submitting the development plans the owner or developer shall pay a filing fee in accordance with the following schedule:

Number of Dwelling Units

in Proposed Development			Amount of Fee

1- 10			$  50.00

11-20			$ 100.00

21-30			$ 150.00

31- 40			$ 200.00

41-50			$ 250.00

over 51			$ 300.00

In addition to the above filing fee, the owner or developer shall pay any fee charged by the Village Engineer for the cost of review of the development plans and the cost of supervision of all improvements in the proposed development.

5.5.23.	Approval of Plans. The Village Engineer shall review all development plans for buildings and land in a Senior Citizens Residence District regarding location and construction of sanitary sewers, storm drains and all of the utilities, improvements of access roads, driveways, parking and loading areas, walks and all other proposed site improvements including grading and drainage and shall report his recommendations to the Planning Commission.

The Planning Commission shall review all development plans to determine if they are in accord with all provisions of this Zoning Ordinance and all other Village Ordinances. Within sixty (60) days from the date of receipt of the proposed development plans, the Planning Commission shall either approve the proposed development plans or shall notify the developer, in writing, that the proposed development plans are not in accord with this Zoning Ordinance or other Village Ordinances and shall indicate the particulars in which said plans do not conform.

5.5.24. 	Construction and Permits. Construction shall not be commenced and building and zoning permits shall not be issued until the Planning Commission has approved the proposed development plans, and has notified the Village Administrator, in writing, of such approval

ARTICLE 6 - EXCEPTIONS AND MODIFICATIONS

Section 6.1. General

Requirements and regulations specified in this Ordinance shall be subject to the exceptions, modifications, and interpretations set forth in this Article.

Section 6.2. Existing Lots of Record

In any district where dwellings are permitted, a single-family detached dwelling may be erected on any lot of official record as of the effective date of this Ordinance, irrespective of its area or width, provided the owner of such lot does not own any adjoining property, except that no lot shall be deemed to be less than forty (40) feet wide for the calculation of yard requirements, and provided further:

6.2.1. 	The sum of the side yard widths on any such lot shall not be less than twenty (20) percent of the width of the lot, but in no case shall the width of any side yard be less than ten (10) percent of the width of the lot or less than eight (8) feet, whichever is greater.

6.2.2. 	On a corner lot the width of the side yard adjoining the side street lot line shall not be less than eight (8) feet or twenty (20) percent of the frontage, whichever is greater and shall not project beyond the average building line so established.

6.2.3. 	The depth of the rear yard of any such lot need not exceed twenty (20) percent of the depth of the lot, but in no case shall it be less than twenty (20) feet.

6.2.4. 	Where three (3) or more contiguous unimproved lots of record with less than the required area and width are held by one (1) owner, the Planning Commission may require replatting to fewer lots to permit compliance with minimum yard requirements.

6.2.5. 	For the purpose of computing lot area per family, a depth of only two (2) times the lot width shall be used.

Section 6.3. Height

6.3.1. 	The height regulations prescribed herein shall not apply to television and radio towers so long as they do not exceed thirty-five feet (35') in height unless authorized by Section 8.6 herein, church spires, belfries, monuments, tanks, water and fire towers, stage towers or scenery lofts, cooling towers, ornamental towers and spires, chimneys, elevator bulkheads, smokestacks, conveyors and flagpoles except where the height of such structures will constitute a hazard to the safe landing and take-off of military, public commercial and private aircraft at an established airport.

6.3.2. 	Public, semipublic or public service buildings, hospitals, institutions or schools, where permitted, may be erected to a height not exceeding sixty (60) feet, and churches and temples may be erected to a height not exceeding seventy-five (75) feet when the required side and rear yards are each increased by one (1) foot for each foot of additional building height above the height regulations for the district in which the building is located.

Section 6.4. Front Yards

6.4.1. 	When fifty (50) percent or more of the frontage on one side of the street between two (2) intersecting streets is improved with buildings that have a front yard which is greater or less than the required front yard in the district, no building shall project beyond the average front yard so established, provided, however, that a front yard depth shall not be required to exceed fifty (50) percent in excess of the front yard otherwise required in the district in which the lot is located.

6.4.2. 	On lots between two streets having double frontage, the required front yard shall be provided on both streets.

6.4.3. 	Gasoline service station pumps and pump islands may be located within a required yard, provided they are not less than fifteen (15) feet from any right-of-way line and not less than one hundred (100) feet from the boundary of any residence and residential district.

6.4.4. 	Off-street parking facilities may be located within the required front yard of any "B" District or "M" District but shall not be nearer than fifty (50) feet to any "A" or "R" District.

Section 6.5. Side Yards

6.5.1. 	On a corner lot, the width of the yard along the side street shall not be less than one-half of any required front yard on such street, provided that the buildable width of a lot of record shall not be reduced to less than forty (40) feet.

6.5.2. 	No accessory building shall project beyond a required yard line along any street or be closer than six (6) feet to any side lot line.

6.5.3. 	A canopy may project into a required side yard provided every part of such canopy is unenclosed and not less than seven (7) feet from any side lot line.

6.5.4. 	For the purpose of side yard regulation, a two-family dwelling or multiple-family dwelling shall be considered as one building occupying one lot.

6.5.5. 	An owner of a dwelling erected prior to the effective date of this Ordinance shall be permitted to enlarge or structurally alter such dwelling to provide additional closed space for living or garage purposes, provided that no side yard shall be reduced to less than fifteen (15) percent of the lot width.

Section 6.6. Rear Yards

6.6.1 	An accessory building, not exceeding twenty (20) feet in height, may not occupy more than twenty (20) percent of the area of a required rear yard. Unenclosed parking spaces may not occupy more than twenty (20) percent of the area of a required rear yard. No accessory building shall be closer than fifteen (15) feet to the main building nor closer than six (6) feet to any rear lot line.

6.6.2. 	The ordinary projections of sills, belt courses, cornices and ornamental features may extend to a distance not to exceed eighteen (18) inches into a required yard.

6.6.3. 	Open or lattice-enclosed fire escapes, outside stairways and balconies opening upon fire towers and the ordinary projections of chimneys and flues into a rear yard may be permitted by the Zoning Inspector for a distance not to exceed five (5) feet when these are so placed as not to obstruct light and ventilation.

Section 6.7. Setback from Publicly Established Drainage Ditches

In all districts a setback of forty (40) feet from the center line of a publicly established drainage ditch shall be provided for all buildings or structures erected along such ditch.

Section 6.8 Landscaping and Screening Provisions

For business and industrial uses abutting "A," "S," or "R" Districts, screening shall be required. For agricultural uses abutting "A," "S," or "R" Districts, screening may be required with approval of the Planning Commission after public notice to adjoining property owners. The screening shall be composed of a masonry or solid fence six (6) feet to eight (8) feet in height, maintained in good condition and free of all advertising or other signs, landscaping that shall consist of a strip of land not less than fifteen (15) feet in width and all located between the fence and property boundary, maintaining the original trees and topography, and planted with an additional evergreen hedge or dense planting of evergreen shrubs not less than four (4) feet in height.

ARTICLE 7 - OFF-STREET PARKING AND LOADING REQUIREMENTS

Section 7.1. Off-Street Loading Requirements

7.1.1.	In any district, in connection with every building or part thereof hereafter erected and having a gross floor area of ten thousand (10,000) square feet or more, which is to be occupied by manufacturing, storage, warehouse, goods display, retail store, wholesale store, market, motel, restaurant, hospital, mortuary, laundry, dry cleaning or other uses similarly requiring the receipt or distribution by vehicles of material or merchandise, there shall be provided and maintained, on the same lot with such building, at least one (1) off-street loading space plus one (1) additional such loading space for each twenty thousand (20,000) square feet or major fraction thereof of floor area so used in excess of twenty thousand (20,000) square feet.

7.1.2. 	Each loading space shall be not less than twelve (12) feet in width, fifty-five (55) feet in length, and fourteen (14) feet in height.

7.1.3. 	Subject to the limitations in Section 7.4. of this Article, such space may occupy all or any part of any required yard space.

7.1.4. 	No such space shall be located closer than one hundred (100) feet to any lot in any "R" District.

7.1.5. 	Off-Street Loading Requirements for "B-3" Districts, in lieu of the above, shall be as follows: There shall be provided at least one off-street loading space for each 10,000 square feet of floor area.

Section 7.2. Off-Street Parking Space Requirements

7.2.1. 	General Requirements. In all districts, in connection with every industrial, business, institutional, recreational, residential or any other use, there shall be provided, at any time any building or structure is erected, enlarged, increased in capacity or structurally altered, off-street parking spaces for motor vehicles in accordance with the following requirements:

a. Each off-street parking space shall have an area of not less than two hundred (200) square feet in addition to access drives or aisles, and shall be of usable shape and condition.

b. There shall be adequate provision for ingress and egress to all parking spaces. Where a parking lot does not abut on a public or private alley or easement of access, there shall be provided an access drive not less than eight (8) feet in width in the case of a dwelling and not less than eighteen (18) feet in width in all other cases, leading to the parking or storage areas or loading or unloading spaces required hereunder.

7.2.2. Number of Spaces to be Provided. In all districts there shall be provided at the time any building or structure is erected, enlarged, increased in capacity or structurally altered off-street parking spaces in accordance with the following requirements:

Use	Parking Spaces Required

"B-3" District	In "B-3" District in lieu of the above require-

ments, one parking space shall be provided

for each 200 feet of gross floor area.

Bowling alley	5 for each alley.

Church or temple	1 for each five (5) seats in main auditorium.

College or high school	1 for each eight (8) seats in main auditori-

um or three (3) for each classroom, which

ever is greater.

Community center, library, museum or	10 plus one (1) additional for each museum

art gallery	or art gallery or each three hundred (300)

sq. ft. of floor area in excess of two thou

sand (2,000) square feet.

Dwellings, including 1, 2, and 3 fami-	Two for each dwelling unit.

lies, multiple dwellings and summer

cottages

Furniture or appliance store, hardware	2 plus one (1) additional for each two hun-

store, wholesale establishments, ma-	dred (200) sq. ft. of area over one thou

chinery or equipment sales and service	sand (1,000) sq. ft.

Hospital, sanitarium, convalescent	1 for each two (2) beds.

home, home for the aged or similar

institution

Manufacturing or industrial establish-	2 for each three (3) employees on the max-

ment, research or testing laboratory,	imum working shift plus space to accom-

creamery, bottling plant, warehouse or	modate all trucks and other vehicles used

similar establishment	in connection therewith.

Mortuary or funeral home	1 for each fifty (50) sq. ft. of floor space in

sitting rooms, parlors or individual funeral

service rooms.

Motel	1 for each sleeping room or suite.

Office or Clinic	1 for each employee and two (2) for each

examining room or office.

Printing or plumbing shop or similar	1 for each person employed therein.

service establishment

Restaurants, nightclubs, cafes or similar 	1 for each one hundred (100) square feet

recreation or amusement establishments, 	of floor area.

dance halls, assembly or exhibition halls

without fixed seats

Retail store or personal service                    	1 for each two hundred (200 square feet of gross

establishment except as otherwise                 	floor area.

provided herein

School (except high school or college) 	1 for each ten (10) seats in auditorium or main assembly room, or one for each classroom, whichever is greater.

Social Activities		1 for each five (5) members.

Theater or auditorium (except school		1 for each three (3) seats or bench

auditorium), sports arena, stadium or		seating spaces.

gymnasium

7.2.3. Rules Governing the Determination of the Number of Spaces. In computing the number of spaces required in Subsection 7.2.2. of Section 7.2. of this Article, the following rules shall govern:

a. "Floor area" shall mean the gross floor area of the specified use.

b. Where fractional spaces result, the parking spaces required shall be construed to be the nearest whole number.

c. The parking space requirement for a use not specifically mentioned herein shall be the same as required for a use of similar nature.

d. Whenever a building or use constructed or established after the effective date of this Ordinance is changed or enlarged in floor area, number of employees, number of dwelling units, seating capacity or otherwise, to create a need for an increase of ten (10) percent or more in the number of existing parking spaces, such spaces shall be provided on the basis of the enlargement or change. Whenever a building or use existing prior to the effective date of this Zoning Ordinance is changed or enlarged to the extent of fifty (50) percent or more in floor area or in the area used, said building or use shall then and thereafter comply with the parking requirements set forth herein.

Section 7.3. Special Parking Provisions.

7.3.1. 	The parking of a disabled vehicle within a residential district for a period of more than 48 hours shall be prohibited; except that such vehicles may be stored in an enclosed garage or other accessory building provided that no business shall be conducted in connection therewith while such vehicles are parked or stored.

7.3.2. 	All parking spaces required herein shall be located on the same lot with the building or use served, except that where an increase in the number of spaces is required by a change or enlargement of use or where such spaces are provided collectively or used jointly by two (2) or more buildings or establishments, the required spaces may be located not to exceed four hundred (400) feet from the building or use served.

7.3.3. 	In any case where the required parking spaces are not located on the same lot with the building or use served, or where such spaces are collectively or jointly provided and used, a written agreement thereby assuring their retention for such purposes shall be properly drawn and executed by the parties concerned, approved as to form by the Village Solicitor and approved by the Planning Commission and shall be filed with the application for a zoning permit.

Section 7.4. Development and Maintenance of Parking Areas.

Every parcel of land hereafter used as a public, commercial or private parking area shall be developed and maintained in accordance with the following requirements:

7.4.1. 	Screening and Landscaping. Off-street parking areas situated in any "B" District or "M" District shall be effectively screened on each side which adjoins premises situated in any "R" District by a solid fence as approved by the Planning Commission. Such fence shall not be less than four (4) feet or more than six (6) feet in height and shall be maintained in good condition without any advertising thereon. The space between such fence and the lot line of the adjoining premises in any "R" District shall be landscaped with grass, hardy shrubs, or evergreen ground cover and maintained in good condition. In lieu of such wall or fence, a strip of land not less than fifteen (15) feet in width and planted and maintained with an evergreen hedge or dense planting of evergreen shrubs not less than four (4) feet in height may be substituted if approved by the Planning Commission.

7.4.2. 	Minimum Distances and Setbacks. No part of any parking area for more than five (5) vehicles shall be closer than ten (10) feet to any dwelling, school, hospital or other institution for human care located on an adjoining lot unless screened by an unpierced masonry wall of acceptable design. If on the same lot with a main building, the parking area shall not be located within the front yard or side street side yard required for such building. In no case shall any part of a parking area be closer than five (5) feet to any established street or alley right-of-way.

7.4.3. 	Surfacing. Any off-street parking area for more than five (5) vehicles shall be graded for proper drainage and surfaced with an asphaltic or Portland cement binder pavement so as to provide a durable and dustless surface and shall be so arranged and marked as to provide for orderly and safe parking and storage of motor vehicles. The foregoing surfacing requirements shall not apply to a parking area in an "M" District if more than five hundred (500) feet distant from any "R" District, except that a dustless surface shall be provided in any case. Any expansion or enlargement of 50% or more of existing, non-conforming parking area shall require the entire parking area, old and new, to conform to this section.

7.4.4. 	Lighting. Any lighting used to illuminate any off-street parking area shall be so arranged as to reflect the light away from adjoining premises in any "R" District so as not to interfere in any way with traffic movements on any adjoining street or highway.

ARTICLE 8 - SPECIAL PROVISIONS

Section 8.1. Performance Requirements

8.1.1. 	No land or building in any district shall be used or occupied in any manner so as to create any dangerous, injurious, noxious, or otherwise objectionable fire, explosive, or other hazard; noise or vibration; smoke, dust, odor or other form of air pollution; heat, cold, dampness; electrical or other disturbance; glare; liquid or solid or hazardous refuse or wastes; or other substance, condition or element in such a manner or in such amount as to adversely affect the surrounding area or adjoining premises provided that any use permitted or not expressly prohibited by this Ordinance may be undertaken and maintained if acceptable measures and safeguards are employed to limit dangerous and objectionable elements to acceptable limits and tolerances at the following points of observation:

a. In any "R" District and "B-1 " and "B-2" Districts, 25 feet from the establishment or use, or at the lot line if closer to the establishment or use.

b. In any "M" Districts, at the boundary or boundaries of the District or at any point within an adjacent "R" District.

8. 1.2. 	The Zoning Inspector or Board of Zoning Appeals, prior to the issuance of a zoning certificate, may require the submission of statements and plans indicating the manner in which dangerous and objectionable elements involved in processing and in equipment operations are to be eliminated or reduced to acceptable limits and tolerances.

Section 8.2. Trailers and Motor Homes.

8.2.1. 	Parking of a trailer in any district for 48 hours or a longer period of time shall be prohibited, provided that the owner thereof may park one small utility trailer, vacation trailer or motor home in an enclosed garage or other accessory building, or in the open elsewhere on his property if parked within thirty feet (30') of the dwelling or building in the front yard or anywhere in a side yard or rear yard so long as it is at least ten feet (10') from all lot lines. In all cases no living quarters shall be maintained or any business conducted in connection therewith while such trailer or motor home is parked or stored. The parking of a trailer, other than utility or vacation trailer, for less than 48 hours outside of an enclosed garage or another accessory building shall be permissible only after the Zoning Inspector has been notified of such intention.

Section 8.3. Temporary Buildings.

Temporary buildings used in conjunction with on-site construction work only may be permitted in any district during the period that the construction work is in progress, but such temporary buildings shall be removed upon completion of the construction work.

Section 8.4. Shopping Centers ("B-3").

Prior to the issuance of a zoning certificate the Zoning Inspector shall require that the preliminary plan for the use and development of a shopping center be submitted to the Planning Commission. It shall thereupon be the duty of the Planning Commission to investigate and ascertain that the plan of the project provides for integrated and harmonious design of buildings and for adequate and properly arranged facilities for internal traffic circulation, entrances and exits, off-street parking and loading,

landscaping and such other features and facilities as may be necessary to make the project attractive and efficient from the stand point of the developer as well as adjoining and surrounding property owners.

The plans shall provide for the installation of adequate storm drainage, sanitary sewer and water and the extension of other public utilities.

The Planning Commission shall either approve said plan or disapprove same giving its reasons in writing, within forty-five (45) days after the plan is submitted to it. Upon approval of the plan by the Planning Commission, the Zoning Inspector shall issue the Zoning Certificate.

Section 8.5. Occupancy Permit in "B-3" Districts.

In the event that any person desires to occupy a portion of the building or buildings in a Shopping Center prior to the completion of such project including building, planting, loading area, parking facilities and driveways, then such party shall apply to the Planning Commission for an Occupancy Permit. If, in the opinion of the Planning Commission, the project is sufficiently completed so that it will be for the best interests of the community to allow such occupancy prior to the completion, an Occupancy Permit may be granted upon the owner or owners of the project furnishing a completion bond satisfactory to the Planning Commission, guaranteeing completion of the planting, landscaping and building in conformance with the plans previously approved by the Planning Commission and for completion of sufficient parking, loading and driveway areas together with the entrance and exit roads, for the portion of the project then under development.

Section 8.6. Transmission Towers.

8.6.1.	A "transmission tower," as used in this Section, is a tower used in connection with the operation of the transmission or receiving of radio, television, mobile telephone, facsimile, or siren broadcasting by a commercially operated broadcasting station licensed by the Federal Communications Commission or other governmental agency.

8.6.2.	Transmission towers, and accessory buildings used only in connection with the operation thereof, may be constructed, operated and maintained only in an Agricultural District (A-1) and only in compliance with the provisions of this Section.

8.6.3.	A transmission tower shall be located on an unoccupied parcel of land having an area of sufficient size so that no part of the tower, as determined by its height, will fall on any neighboring property should the structure fall or collapse unless a certified structural engineer determines to the satisfaction of the Planning Commission, that a transmission tower upon fall or collapse will not encroach upon any neighboring property. Costs incurred for any engineering study shall be paid by the property owner. The minimum size of said parcel shall be five (5) acres. Only one (1) transmission tower may be located on said parcel.

8.6.4.		A tower guy anchor or accessory building used in connection with the operation of said transmission tower shall not be located closer than fifty feet 	(50') from any lot line or public right-of-way. A transmission tower shall not be located closer than four hundred fifty feet (450') from any public right-away unless a certified structural engineer determines, to the satisfaction of the Planning Commission, that a transmission tower upon fall or collapse will not encroach upon any public right-of-way. Costs incurred for any engineering study shall be paid by the property owner.

8.6.5.	A transmission tower, and accessory buildings used only in connection with the operation thereof, shall not cause objectionable odors, noise, glare, vibration or electrical disturbance which materially affects the use, enjoyment, health, safety or welfare of the owners or occupants of adjoining or nearby properties.

8.6.6.	The owner of said transmission tower shall maintain adequate liability insurance in an amount not less than One Million Dollars ($1,000,000.00) to protect against loss of life, injury or damage to property as a result of the failure, fall or collapse or said transmission tower. Madison Village shall be named as an additional insured on said liability insurance policy.

    - Applicants for a permit to erect, construct, alter, modify, relocate or rebuild a transmission tower shall provide evidence of written contact with all wireless service providers who supply service within a quarter mile of the proposed transmission tower.  Said written contact shall inquire about collocation opportunities at all technically feasible locations.  The contacted providers shall be requested to respond in writing to the inquiry within thirty (30) days.  The Applicant’s letter, as well as responses, shall be presented to the Planning Commission as a means of demonstrating the need for a new transmission tower.  Whenever technically feasible, transmission or receiving equipment shall be located on an existing transmission tower or on an existing structure or building.

    - Unless technically not feasible, a transmission tower shall consist of a support structure constructed of a single, self-supporting hollow metal tube securely anchored to a foundation, commonly known as a “monopole.”

    - No transmission tower shall be erected, constructed, altered, modified, relocated or rebuilt until approved by the Planning Commission and a permit therefor has been issued by the Zoning Inspector.  Such permit shall become void if such transmission tower is not erected, constructed, altered, modified, relocated or rebuilt within one hundred twenty (120) days after such permit is issued.

    - Applicants for a permit to erect, construct, alter, modify, relocate or rebuild a transmission tower shall file with the Zoning Inspector an application which shall contain such information and documents as the Zoning Inspector and the Planning Commission shall require, including but no limited to, a development plan and copies of all documents submitted by the applicant to the Federal Communications Commission or other governmental agency having jurisdiction.

8.6.11.	An application for a transmission tower permit shall be accompanied by an application fee in the amount of Fifty Dollars ($50.00).

8.6.12.	The owner of said transmission tower shall notify the Zoning Inspector when any such tower and related facilities use will be discontinued and the date this use will cease.  Within thirty (30) days of said use being discontinued, said transmission tower and related facilitites shall be dismantled and removed.  If at any time the use of said transmission tower and related facilities is discontinued for one hundred eighty (180) days, the Zoning Inspector may declare the tower and related facilities abandoned.  The owner of the tower and related facilities shall then be instructed in writing to either reactivate the tower’s use within one hundred eighty (180) days or dismantle and remove the tower and related facilities.  If reactivation or dismantling does not occur, either under the voluntary discontinuation of use or abandonment, the Village shall remove, or shall have removed, the tower and related facilities and shall assess the owner the cost of same.

8.6.13.	Any person, firm or corporation who violates any of the provisions of this Section shall be deemed guilty of a minor misdemeanor and shall be fined not more than One Hundred Dollars ($100.00) for each such offense. A separate offense shall be deemed to have been committed for each day during which a violation continues.  (Amended by Ordinance No. 20-1999, Passed May 17, 1999:  An Ordinance amending Ordinance No. 33-1996, as amended, the Zoning Code of the Village of Madison, by amending Section 8.6 thereof, relating to transmission towers.)

ARTICLE 9 - SIGNS

Section 9.1. Statement of Purpose.

9.1.1. 	The purpose of this section is to create a comprehensive set of sign regulations and to authorize the use of signs which:

a. facilitate optimum communication between people and the environment;

b. are compatible with the surroundings;

c. are appropriate to the type of activity to which they pertain;

d. express the identity of individual proprietors or of the community as a whole;

e. are legible in the circumstances in which they are seen; and,

f. promote public safety.

Section 9.2. Definition.

"Sign," as referred to herein, shall mean any advertisement, announcement, mark of identification, plaque, historical marker, graphic or symbol attached to or painted upon any building, structure or land, which display is visible beyond the boundaries of the lot. Such displays attached to movable trailers, or frames, shall be construed to be signs for the purposes of this Ordinance.

Section 9.3. Permits, Fees, and Inspections.

9.3.1.	 A sign permit issued by the Zoning Inspector shall be required prior to the erection, display, relocation, or alteration of any sign except as otherwise provided in this Section. When any sign is erected, displayed, relocated or altered prior to obtaining a permit, the required fees shall be doubled but the payment of such double fees shall not relieve any person from complying with all provisions of this Section or from any penalties prescribed by law.

9.3.2. 	No sign permit issued hereunder shall be deemed to constitute permission or authorization to maintain an unlawful sign, nor shall any permit issued hereunder constitute a defense in an action to abate an unlawful sign.

9.3.3.	Signs shall be inspected by the Zoning Inspector for compliance with all applicable codes.

Section 9.4. General Regulations.

9.4.1. 	Except as provided in Section 9.10.4., every sign shall be located on the same lot of the business or use to which such sign pertains.

9.4.2. 	No signs except those erected and maintained by public agencies for purposes of traffic control shall be erected in the public right-of-way or on public property unless approved by the Zoning Inspector.

9.4.3. 	No sign shall be constructed or located so as to conflict with traffic control devices.

9.4.4. 	No sign shall have more than two (2) faces.

9.4.5. 	No sign affixed to a building shall extend above the highest point of the roof line or project further than forty eight (48) inches in front of the surface to which it is affixed.

9.4.6. 	Illumination used on, for, or in any sign shall not be so arranged or used so as to cause reflection or glare. No sign shall be illuminated by flashing or moving lights. No sign in a residential district shall be illuminated.

9.4.7. 	No sign shall be revolving, oscillating, or otherwise moving. Flags, banners, pennants, or balloons shall not be attached to any sign.

9.4.8. 	Signs shall be maintained in a safe condition and in a reasonable state of repair. The Zoning Inspector shall have the authority to inspect and order the painting, repair, alteration, or removal of any sign which becomes dilapidated or is abandoned or which constitutes a hazard to public safety.

9.4.9. 	No sign shall be located, placed, illuminated or arranged so as to impair traffic visibility at any intersection or at any entrance or exit to any property.

Section 9.5. Real Estate Signs.

Non-illuminated real estate signs advertising the sale, rental, or lease of the premises on which they are maintained shall not exceed nine (9) square feet in area per side, shall have a minimum front setback of ten feet (10'), shall have a minimum side setback of ten feet (10'), and shall have a maximum height of six feet (6'). Larger signs shall be permitted for two (2) or more lots in single ownership or for properties with frontages in excess of two hundred feet (200'), provided that such signs shall not exceed a maximum area of fifty (50) square feet per side, provided that such signs shall be setback from every street line at least a distance in feet equal to the number of square feet in the area of the sign. N o permit and no fee shall be required for real estate signs .

Section 9.6. Home Occupation Identification Signs.

Small announcement or home occupation identification signs, where permitted, shall not exceed four (4) square feet in area per side, shall have a minimum front setback of ten feet (10'), shall have a minimum side setback of ten feet (10'), and shall have a maximum height of six feet (6').

Section 9.7. Public, Educational, or Religious Institution Identification Signs.

Public, educational, or religious institution identification signs shall not exceed twenty (20) square feet in area per side, shall have a minimum front setback of ten feet (10'), shall have a minimum side setback of ten feet (10'), and shall have a maximum height of six feet (6').

Section 9.8. Single Family and Duplex Residence Identification Signs.

Single family and duplex residence identification signs shall not exceed two (2) square feet per side, shall have a minimum front setback of ten feet (10'), shall have a minimum side setback of ten feet (10'), and shall not exceed a maximum height of six feet (6'). No permit and no fee shall be required for single family and duplex residence identification signs.

Section 9.9. Subdivision and Multi-Family Residence Identification Signs.

Subdivision and multi-family residence identification signs shall be permitted only after prior approval and authorization by the Planning Commission.  Subdivision and multi-family residence identification signs shall be required when the subdivision or multi-family residence consists of more than five (5) parcels or lots and a final plat is required or consists of more than five (5) dwelling units.  Such signs shall not exceed twenty (20) square feet in area per side, shall have a minimum front setback of ten feet (10'), shall have a minimum side setback of ten feet (10') and shall have a maximum height of six feet (6').  Such signs shall be compatible with their surroundings and shall conform to and comply with all applicable sign regulations, including, but not limited to, Section 9.4 of this Article.  The Applicant/Owner/Subdivider shall provide, to the satisfaction of the Planning Commission, adequate and sufficient assurance that such signs will be continuously own, maintain and be responsible for such signs.  The assurances required herein may include, but are not limited to, (1) deposit of sufficient cash; (2) posting of adequate bond; (3) creation of homeowner or condominium association; (4) trust agreement appointing trustee to act for all owners; or, (5) common easements and deed restrictions.  (Amended by Ordinance No. 03-1999, Passed February 15, 1999:  An Ordinance amending Section 9.9 of Ordinance No. 33-1996, as amended, the zoning code of the Village of Madison, relating to Subdivision and Multi-Family Residence Identification Signs.)

Section 9.10. B-1 District Signs.

9.10.1. 	A business located in a "B-1" District may have one (1) sign per building advertising the nature of the business, merchandise, products, or service. The total aggregate area of said sign shall not exceed one and one-half (1.5) square feet of sign for each lineal foot of frontage of said building, provided, however, that no sign shall exceed one hundred (100) square feet and no sign shall be located within five percent (5%) of said frontage at each end of said building. Excepted as provided in Section 9.10.4(a), said sign shall be located on the same lot as said building. Said sign shall have a minimum front setback of five feet (5'), shall have a minimum side setback of five feet (5'), and shall have a minimum height of eight feet (8') to the bottom of the sign.

9.10.2. 	The sign referred to in this section may be a hanging sign or a free-standing sign provided, however, that said sign shall first be approved and authorized by the Planning Commission. Said sign shall conform to the sign area and setback requirements applicable for the "B-1" District and shall be located no nearer than five feet (5') from the traveled portion of any roadway.  The minimum height of said sign shall be as approved and authorized by the Planning Commission.  (Amended by Ordinance No. 24-1997, Passed May 19, 1997:  An Ordinance No. 33-1996, The Madison Village Zoning Code, Sections 9.10.2, 9.10.3 and 9.11.2. so as to require Planning Commission approval of the minimum height of hanging or free-standing signs in "B-1", "B-2", "M-1" and "M-2" districts.)

9.10.3. 	In addition to the sign referred to in this section, a business located in a "B-1" District which has a minimum of one hundred fifty feet (150') of frontage on a public street may have a free-standing sign provided that said sign shall first be approved and authorized by the Planning Commission, subject to the following requirements: no portion of said sign shall be located nearer than five feet (5') from the public right-of-way or a public walkway; said sign shall not exceed a maximum area of fifty (50) square feet per side; and, said sign shall not exceed a maximum height of twenty feet (20').  The minimum height of said sign shall be as approved and authorized by the Planning Commission.  (Amended by Ordinance No. 24-1997, Passed May 19, 1997:  An Ordinance No. 33-1996, The Madison Village Zoning Code, Sections 9.10.2, 9.10.3. and 9.11.2. so as to require Planning Commission approval of the minimum height of hanging or free-standing signs in "B-1", "B-2", "M-1" and "M-2" districts.)

9.10.4. 	Every sign shall be located on the same lot of the business or use to which such sign pertains unless all of the following apply:

a. There has been prior approval and authorization by the Planning Commission for a sign to be located on a different lot subject to such terms and conditions as the Planning Commission deems necessary to protect the public interest, health, safety and welfare.

b. The location of the business or use to which such sign pertains is on a dead end or non-through street or road.

c. The sign is to be located within five hundred feet (500') of the lot where the business or use to which such sign pertains is located or the distance to the nearest major through street or road at the point where it intersects with the street or road upon which the business or use is located if more than five hundred feet (500') away.

d. The owner of the lot where the sign is to be located has given written permission for the sign's location.

e. There has been demonstrated that all the following apply: that an undue and unnecessary hardship will result upon the owner of the business or use to which such sign pertains should the sign be required on the same lot as said business or use; that there are exceptional or extraordinary circumstances or conditions which apply to the owner of the business or use to which such sign pertains which justify location of the sign on a different lot; that preservation and enjoyment of substantial property rights can only be protected by permitting the sign to be on a different lot than the one where the business or use to which such sign pertains is located; that there will be no substantial detriment to adjacent property; and, that there will be no material impairment of the public interest or the spirit, intent and purposes of this Zoning Ordinance by permitting said sign on a different lot.

Section 9.11. B-2. M-1. and M-2 District Signs.

9.11.1.	A business located in a "B-2", "M-1", or "M-2" District may have signs advertising the nature of the business, merchandise, products, or service subject to the following regulations. The combined aggregate area of all signs per building shall not exceed two (2) square feet of sign for each lineal foot of frontage of said building provided that the combined area of all signs per building shall not exceed two hundred (200) square feet and no signs shall be located within five percent (5%) of said frontage at each end of said building. The total allowable area per building may be divided between signs. Said signs shall be located on the same lot as said building. Said signs shall have a minimum front setback of five feet (5'), shall have a minimum side setback of five feet (5'), and shall have a minimum height of ten feet (10') to the bottom of said signs.

    - In addition to the signs referred to in this section, a business located in a "B-2", "M-1", or "M-2" District may have one hanging sign or one free-standing sign provided that said  hanging sign shall first be approved and authorized by the Zoning Inspectorand said free standing signshall first be approved and authorized by the Planning Commission. Said sign shall conform to the size area and setback requirements applicable for the appropriate "B-2", "M-1", or "M-2" District and shall be located no nearer than five feet (5~) from the traveled portion of any roadway.  The minimum height of said hanging sign shall be eight feet (8’) and minimum height of said free standing sign shall be as approved and authorized by the  Planning Commission.  (ORD # 13-2002)

Section 9.12. B-3 District Signs.

Signs in "B-3" Districts announcing the name of the shopping area and/or each individual business in the shopping area shall be permitted only after prior approval and authorization by the Planning Commission.

Section 9.12(A).  B-4 District Signs.

Signs in "B-4" Districts shall be permitted only after prior approval and authorization by the Planning Commission.  (Amended by Ordinance No. 07-1998, Passed March 16, 1998:  An Ordinance amending Ordinance No. 33-1996, by adding a New Section 9.12(A) so as to regulate "B-4" District Signs.)

Section 9.12(B).  B-5 District Signs

Signs in “B-5” Districts shall be permitted only after prior approval and authorization by the Planning Commission.  (Amended by Ordinance No. 54-1999, Passed January 3, 2000:  An Ordinance amending Ordinance No. 33-1996, by adding a New Section 9.12(B) so as to regulate “B-5” District Signs.)

Section 9.13. S-1 District Signs.

Signs in S-1 Districts shall be permitted only after prior approval and authorization by the Planning Commission.

Section 9.14. Entrance and Exit Signs.

All parking lots may have entrance and exit signs. Such signs shall be limited to the words "entrance", "in", "exit", "out", "shipping", "receiving", and "parking," or other similar directional words specifically permitted by the Zoning Inspector and may also have arrows or other appropriate directional indicators. Corporate names, logos, or other nondirectional identification may be included but shall not exceed a maximum area equal to twenty-five percent (25%) of the sign face. Such signs shall not exceed five (5) square feet in total aggregate area, nor two and one-half (2.5) square feet in area on one side, nor forty-two inches (42") in height, nor be located closer than three feet (3') to any property line or traveled portion of any roadway. No permit and no fee shall be required for entrance and exit signs.

Section 9.15. Service Station Signs.

Service stations may erect one (I) permanent sign in addition to those permitted herein for the purpose of identifying fuel prices only. Said signs shall not exceed six (6) square feet in area per side and shall not be located closer than five feet (5' ) from the traveled portion of any roadway.

Section 9.16. Temporary Signs.

Temporary signs not in conflict with the purposes or regulations of this ordinance may be displayed only as follows:

9. 16.1. 	Special Event Signs: Temporary signs advertising special events may be displayed for not more than two (2) two week periods per year in the business districts only. Such signs shall conform to all

requirements for free-standing signs established herein except that they shall not exceed thirty-two (32) square feet in total aggregate area and shall not be illuminated. A permit, but no fee, shall be required if a temporary sign is displayed in excess of seventy-two (72) hours. Otherwise, no permit and no fee shall be required for said signs.

9.16.2. 	Signs Over Roadways: Banners, streamers, cloth, canvas or similar signs shall not be suspended over or across any public sidewalk, street or roadway unless approved by the Planning Commission and then only for a temporary period not to exceed thirty (30) days. No permit and no fee shall be required for said signs.

9.16.3. 	Construction Signs: Temporary signs identifying future occupants, architects, engineers, developers or contractors engaged in construction of any subdivision or individual site may be displayed in the industrial and commercial districts during the period of construction. Such signs shall conform to all requirements for freestanding signs established herein except that they shall not exceed thirty-two (32) square feet in total aggregate area and shall not be illuminated. No permit and no fee shall be required for said signs.

9.16.4. 	Residential Subdivision Development Signs: Temporary signs identifying and advertising residential subdivision developments may be displayed as permitted by the Zoning Inspector except that such signs shall not exceed thirty two (32) square feet in total aggregate area. Said signs may be displayed for a period of one (I) year or to the end of development whichever comes first unless otherwise extended by the Zoning Inspector for good cause shown.

Section 9.17. Political Signs.

9.17.1. 	A political sign is hereby defined as being one concerning, by word or words, and/or picture, and/or design, and/or symbol, any one candidate or political party, issue, levy, referendum, or other matter whatsoever eligible to be voted upon in any general, primary or special election and/or one advocating by any word or words, and/or picture, and/or designs, and/or symbol any type of political action to be taken or already taken by the electorate and/or its representative.

9. 17.2. 	Political signs are hereby permitted in any zoning district in the Village of Madison provided that no owner and/or occupier of premises, or other person or persons, shall place, erect, maintain or give permission therefor, or suffer to remain on or about premises or parcels of land, political signs that do not comply with or conform to the following restrictions and conditions:

a. A political sign shall not exceed nine (9) square feet in total aggregate area, all sides included, and shall not be artificially illuminated.

b. No political signs shall be permitted on vacant and/or unoccupied and/or unimproved parcels or lots or premises.

c.	   Up to three (3) political signs shall be permitted on each parcel or lot improved with a building or other structure in lawful use and occupancy, provided, however, that no two (2) political signs shall be less than ten feet (10’) apart.

d. Political signs shall not be permitted within the right-of-way boundary lines of any street, road, or alley.

e. Political signs shall not be located closer than fifteen feet (15’) from the edge of any pavement or traveled surface of any street, road or alley.

f. Political signs shall not be placed, erected, and/or maintained without first securing the permission of the owner or occupier of the property where the sign is to be located.

g. Political signs shall not be placed or erected and/or maintained without first posting a Fifty Dollar ($50.00) bond with the Village Administrator. Said bond shall be refundable if all political signs are removed from the premises within five (5) days after any such election. Said bond shall be forfeited if a sign is erected, placed or maintained in violation of this Article unless the violation is corrected within twenty-four (24) hours of notice of same. Only one (1) bond shall be required of any person seeking to place, erect, or maintain a political sign regardless of the number of political signs so erected, placed, or maintained.

h. Political signs shall not be erected, placed, or maintained, or permission therefor granted, or suffered to remain on or about the premises, by any 	owner or occupier of land, or any other person, more than thirty (30) days prior to any general, primary, or special election, nor shall any owner or occupier of land, or other person, permit, suffer, or cause any political sign to remain on or about the premises longer than five (5) days after any such election.

- Political signs shall be firmly affixed to the ground or lawful structures and shall not be permitted to become loose or detached from any support structure.

- No signs shall be placed on utility poles, street signs or traffic control poles.

- No signs shall obstruct sight distances for vehicles entering or exiting any property or traveling on a public street.

- No signs shall obstruct sight distances for pedestrians entering or exiting any property or walking on or along a public street.

9.17.3. 	This Section shall not apply to political signs displayed from inside any building or other structure if the same is in lawful use and occupancy.

9.17.4. 	Political headquarters for any candidate, political party, issue, levy, referendum or any other matter whatsoever eligible to be voted upon in any election shall be permitted only in "B-1" and "B-2" Districts.

9.17.5. 	No permit and no fee shall be required for political signs.

Section 9.18. Removal of Signs.

Whenever an occupant vacates a building, the owner shall remove all outside signs pertaining to said business or profession within two (2) weeks after such vacation.

Section 9.19. Joint Occupancy or Multiple Use

When a building is occupied by a single occupant who carries on more than one (I) activity or business within the building, or when a building contains two (2) or more activities or businesses carried on by different occupants, the sign regulations provided for herein shall apply to the building as a whole, not the separate occupants or uses therein. However, additional signs may be permitted for joint occupants or multiple uses after prior approval and authorization by the Planning Commission.

Section 9.20. Application and Fee

9.20.1. 	All required applications for sign permits pursuant to this ordinance shall be in writing and shall include a drawing or diagram of the proposed sign, the name and address of the owner of the building where said sign is to be located, the name and address of the occupant of the building where the sign is to be located, the area, size, and description of said sign, the location, and such other information as may be requested by the Zoning Inspector or the Planning Commission. Unless otherwise provided for herein, each application shall be accompanied by a fee of Ten Dollars ($10.00).

9.20.2. 	No permit and no fee shall be required for:

a. Signs erected by the Village of Madison or other governmental entities for the regulation of traffic, identification of streets, and similar designations or instructions; and,

b. Legal notices, house numbers, resident's identification, memorial or inaugural inscriptions, or historical plaques and markers.

9.20.3. 	In considering applications for sign permits the Zoning Inspector, the Board of Zoning Appeals, or the Planning Commission shall consider the impact each sign will have on adjoining properties and the safety of users of adjoining roadways. The Zoning Inspector, the Board of Zoning Appeals, and the Planning Commission may establish such conditions and standards for approval as they deem necessary or appropriate to preserve the intent of this Zoning Code.

Section 9.21. Non-Conforming Signs.

A non-conforming sign is defined as a sign existing prior to the passage of this Zoning Ordinance which does not conform to one or more of the requirements of this Zoning Ordinance. Normal maintenance by way of painting, cleaning, and minor repair is permitted for the life of the allowed non-conforming period.

Section 9.22. Penalty.

9.22.1. 	No person, firm, or corporation, or his or her agents, including owners and occupants of businesses and buildings in the Village of Madison and subject to this Section, shall violate or refuse to comply with all applicable provisions of this Zoning Code or a lawful order of the Zoning Inspector or Planning Commission issued pursuant to this Zoning Code.

9.22.2. 	Any person, firm, or corporation, or his or her agents, including owners and occupants of businesses and buildings in the Village of Madison and subject to this Zoning Code, who violates any of the provisions of this Section or who fails or refuses to obey a lawful order of the Zoning Inspector or Planning Commission issued pursuant to this Zoning Code, shall be deemed guilty of a minor misdemeanor and shall be fined not more than One Hundred Dollars ($100.00) for each such offense. A separate offense shall be deemed to have been committed each day during which a violation continues.

ARTICLE 10 - FENCES

Section 10.1. General Welfare.

Fences shall not be permitted, erected or in any way maintained in any district in a front, rear or side yard which are contrary or detrimental to the public health, safety, convenience, comfort, prosperity or general welfare of the residents of the Village.

Section 10.2. R-1. R-2. R-3. B-1 and SC-1 Zoning Districts

Fences in a Single Family Residence District (R-1), Single Family Residence District (R-2), Multiple Family Residence District (R-3), General Business District, (B-1) and Senior Citizens Residence District (SC-1) shall conform to the following rules and regulations:

10.2.1. 	Front Yards and Portions of Side Yards Abutting a Street.

a. Fences are permitted in front yards and portions of side yards abutting a street subject to the following restrictions: In a front yard and/or that portion of a side yard which abuts a street, no fence shall be permitted, erected or in any way maintained that is within two feet (2') of the right-of-way of any street or exceeds the height of three feet (3') above the ground surface along which said fence is erected.  (Amended by Ordinance No. 25-2000, Passed April 17, 2000:  An Ordinance amending Ordinance No. 33-1996, Article 10, Section 10.2.1, so as to modify the requirements of fences abutting a street.)

b. Decorative fences generally referred to as "split rail," "post and rail," "ornamental iron" and "picket" fences may exceed three feet (3') in height so long as they conform to the conditions hereinafter set forth and are not located within two feet (2') of the right-of-way of any street.

c.  No "rail," which is hereby defined to be a horizontal structure erected as a component of a fence, shall be constructed nearer than six inches (6") to any other rail unless said rails crisscross. No more than three (3) rails, not to exceed eight inches (8") in width shall be constructed between adjacent posts.

d. No "post," which is hereby defined as a vertical structure erected as a component part of a fence, shall be constructed nearer than four feet (4') to any other post.

e. An "ornamental iron" fence may be constructed provided that the design is such that in the opinion of the Zoning Inspector it will not obstruct the view of pedestrian or vehicular traffic or obstruct the view to the detriment of the public safety.

f. A "picket" fence may be constructed provided that no "picket," which is hereby defined to be a vertical structure erected as a component part of a fence, shall be constructed nearer than four inches (4") to any other "picket" and further provided that if a "picket" is wider than four inches (4") then the space between pickets shall be the same width of said pickets.

g. No vegetation shall be planted or in any way maintained or allowed to exist along any fence permitted by this section which will obstruct	the view of pedestrians or vehicular traffic or obstruct the view to the detriment of the public safety.

10.2.2.	Rear Yards and Portions of Side Yards Not Abutting a Street.

a. Fences are permitted in a rear yard and portions of a side yard which do not abut a street; provided, however, said fences shall not at any point exceed six feet (6’) in height from the ground to the top of the fence and shall have at all points a sufficient open space from the ground to the bottom of the fence to properly maintain grass, weeds, vegetation and similar plant growth. (Amended by Ordinance No. 25-2000, Passed April 17, 2000:  An Ordinance amending Ordinance No. 33-1996, Article 10, Section 10.2.2., so as to modify the maximum height requirement of fences in side and rear yards.)

10.2.3. 	Permits.

a. No fences shall be erected, constructed, altered, relocated or rebuilt until a permit therefor has been issued by the Zoning Inspector. Such permit shall become void if such fence is not erected, constructed, altered, relocated or rebuilt within one hundred twenty (120) days after said permit is issued.

10.2.4.  	Application for Permits.

a. Applicants for a permit to erect, construct, alter, relocate or rebuild a fence shall file with the Zoning Inspector an application which shall contain the following:

(1) A plat plan of the lot or parcel drawn by a registered surveyor, showing:

(a) The exact location of all structures located on the subject lot or parcel; and,

(b) The exact location of the proposed fence in relation to:

1. Structures on the subject lot or parcel.

2. The boundary lines of subject lot or parcel.

3. The right-of-way of any street abutting subject lot or parcel.

4. The distance from any other fence or structure on the property abutting

the subject lot or parcel.

(2) The complete plans and specifications for the proposed fence, including:

(a) Material to be used;

(b) The design thereof;

(c) The exact percentage of open space between the ground surface and top of

fence in front yards or portions of side yards abutting streets; and,

(d) The exact height of said fences from the ground surface along which the fence

is erected.

(3) In lieu of the plat plan drawn by a registered surveyor referred to in Section 10.2.4.a(1), the applicant may submit a scaled plat plan containing all other items required by this Section, provided that: (a) said plat plan is approved in writing by all owners of property contiguous to the proposed fence, or (b) sufficient proof is presented to the satisfaction of the Planning Commission that the proposed fence is to be located in its entirety upon the Applicant’s property (Amended by Ordinance No. 25-2000, Passed April 17, 2000:  An Ordinance amending Ordinance No. 33-1996, Article 10, Section 10.2.4.a(3) related to rules and regulations regarding fences.)

The plat plans referred to in Section 10.2.4.a(1),(2) and (3) shall not be necessary if the fence is to be located more than five feet (5') from any lot line.

10.2.5. 	Prohibitions.

- Barbed wire or electrically charged fences shall be prohibited in a Single Family Residence District (R-1), Single Family Residence District (R-2), Multiple Family Residence District (R-3), General Business District (B-1) and Senior Citizens Residence District (SC1).

10.2.6.	Finished Side Facing Out.

a.  All fences shall be erected and maintained so that their finished side faces out and the unfinished side faces the property of the person who has caused said fence to be erected or maintained.  The posts or support structures of the fence shall be on the inside of the fence.  (Amended by Ordinance No. 25-2000, Passed April 17, 2000: An Ordinance amending Ordinance No. 33-1996, by adding a New Section, 10.2.6, related to rules and regulations regarding fences.)

10.3.	B-2, B-3, B-4, B-5, M-1 and M-2 Zoning Districts.

Fences in a Highway Business District (B-2), Planned Commercial Development District (B-3), Interstate Business District (B-4), Interstate Business-Industrial District (B-5), Limited Industry District (M-1) and General Industry District (M-2) shall conform to the following rules and regulations: (Amended by Ordinance No. 54-1999, Passed January 3, 2000:  An Ordinance amending Ordinance No. 33-1996, Article 10, Section 10.3  so as to add fences in “B-4” and “B-5” Zoning District subject to the fence rules and regulations).

10.3.1.	Permits.

a. No fences shall be erected, constructed, altered, relocated or rebuilt until a permit therefor has been issued by the Planning Commission. Such permit shall become void if such fence is not erected, constructed, altered, relocated or rebuilt within one hundred and twenty (120) days after said permit is issued.

10.3.2. 	Application of Permits.

a. Applicants for a permit to erect, construct, alter, relocate or rebuild a fence shall file with the Planning Commission an application which shall contain the following:

(1) A plat plan of the lot or parcel drawn by a registered surveyor, showing:

(a) The exact location of all structures located on the subject lot or parcel; and

(b) The exact location of the proposed fence in relation to:

1. Structures on the subject lot or parcel;

2. The boundary lines of subject lot or parcel;

3. The right-of-way of any street abutting subject lot or parcel; and,

4. The distance from any other fence or structure on the property abutting the

subject lot or parcel.

(2) The complete plans and specification for the proposed fence, including:

(a) Material to be used;

(b) The design thereof;

(c) The exact percentage of open space between the ground surface and top of fence

in front yards or portions of side yards abutting streets; and,

(d) The exact height of said fence from the ground surface along which the fence is

erected.

(3) In lieu of the plat plan drawn by a registered surveyor referred to in Section 10.3.2.a(1), the applicant may submit a scaled plat plan containing all other items required by this Section, provided that: (a) said plat plan is approved in writing by all owners of property contiguous to the proposed fence, or (b) sufficient proof is presented to the satisfaction of the Planning Commission that the proposed fence is to be located in its entirety upon the Applicant’s property. (Amended by Ordinance No. 25-2000, Passed April 17, 2000:  An Ordinance amending Ordinance No. 33-1996, Article 10, Section 10.3.2.a(3) related to rules and regulations regarding fences.)

(4) The plat plans referred to in Section 10.3.2.a(1), (2) and (3) shall not be necessary if the fence is to be located more than five feet (5') from any lot line.

    - Finished Side Facing Out.

a.  All fences shall be erected and maintained so that their finished side faces out and the unfinished side faces the property of the person who has caused said fence to be erected or maintained.  The posts or support structures of the fence shall be on the inside of the fence.  (Amended by Ordinance No. 25-2000, Passed April 17, 2000:  An Ordinance amending Ordinance No. 33-1996, by adding a New Section, 10.3.3 related to rules and regulations regarding fences.)

Section 10.4. Permit Fees.

An application for a fence permit shall be accompanied by an application fee in the amount of Ten Dollars ($10.00).

Section 10.5. Compliance.

Except for repainting and minor repairs, no fence shall hereafter be altered, rebuilt, enlarged, extended or relocated except in conformity with the rules and regulations set forth in this Article.

Section 10.6. Public Grounds.

Notwithstanding any provisions set forth herein, fences may be erected by Madison Village on or around any Village owned property or utilities as may be necessary to protect and provide for the public health, safety and welfare.

Section 10.7. Penalty.

No person, firm, or corporation, or his or her agents, including owners and occupants of businesses and buildings in the Village of Madison and subject to this Zoning Ordinance, shall violate or refuse to comply with all applicable provisions of this Zoning Code or a lawful order of the Zoning Inspector or Planning Commission issued pursuant to this Zoning Code.

Any person, firm, or corporation, or his or her agents, including owners and occupants of businesses and buildings in the Village of Madison and subject to this Zoning Ordinance, who violates any of the provisions of this Section or who fails or refuses to obey a lawful order of the Zoning Inspector or Planning Commission issued pursuant to this Zoning Code, shall be deemed guilty of a minor misdemeanor and shall be fined not more than One Hundred Dollars ($100.00) for each such offense. A separate offense shall be deemed to have been committed each day during which a violation continues.

ARTICLE 11 - SATELLITE DISH RECEIVING STATIONS

Section 11.1. Definition.

11.1.1.	A "satellite dish receiving station" shall be defined as an antenna designed to receive direct broadcast satellite service, including direct-to-home satellite service or to receive video programming service by way of multi-point distribution service, including multi-channel, multi-point distribution service, instructional television fixed service and local multi-point distribution service.

Section 11.2.  Applicability.

11.2.1.	This Article shall apply to all satellite dish receiving stations of a size greater than one meter in diameter.

Section 11.3.  Location.

11.3.1. 	Dish type satellite signal receiving stations may be located within the Village, subject to the following regulations and if not otherwise prohibited by law.

11.3.2. 	Dish type satellite signal receiving stations shall be located only in a "rear yard" and behind the principal dwelling or structure located on the lot.

11.3.3. 	Dish type satellite signal receiving stations shall be located so that however turned or otherwise used, all parts of said station shall be set back at least five feet (5') from all side yard lines and shall be set back from the rear lot at least twenty feet (20') or one-half (1/2) of the depth of the rear yard, whichever is greater.

Section 11.4. Size.

11.4.1. 	The height of the dish type satellite signal receiving station, should the dish antenna be turned perpendicular to the ground, shall not extend above eighteen feet (18') and the maximum diameter of said station shall not exceed fourteen feet (14').

Section 11.5. Prohibitions

11.5.1. 	Dish type satellite signal receiving stations shall not be placed on rooftops, buildings or similar structures.

11.5.2. 	Installation or erection of a dish type satellite signal receiving station shall not commence before installation drawings and plans, including all intended hookups to said station, are approved by the Planning Commission.

Section 11.6. Approval.

11.6.1. 	All receivers to said dish type satellite receiving station which are not located on the same lot as said station shall be approved in advance by the Planning Commission.

Section 11.7. Penalty.

11.7.1. 	Any person, firm, or corporation who violates any of the provisions of this Section shall be found guilty of a minor misdemeanor and shall be fine not more than One Hundred Dollars ($100.00) for each such offense. A separate offense shall be deemed to have been committed each day during which a violation continues.

(Amended by Ordinance No. 30-1997, Passed July 21, 1997:  An Ordinance amending Ordinance No. 33-1996, the Madison Village Zoning Code, Article 11-"Satellite Dish Receiving Stations" and to limit the applicability of said article.)

ARTICLE 12 - PRIVATE SWIMMING POOLS

Section 12.1. Definitions.

As used in this section:

a. 	"Inspector" means the Zoning Inspector of the Village of Madison or his authorized representative.

b.	"Swimming pool" shall be any pool, pond, lake or open tank where swimming is permitted, not located within a completely enclosed building and containing or normally capable of containing water to a depth at any point greater than 24" and having at least 150 square feet of surface.

c.	"Rear Yard" is a yard extending across the rear of a lot and being the required minimum horizontal distance between the rear lot line and the rear of the main building or any projections thereof.

Section 12.2. Zoning Certificate and Approval.

Before any work is commenced on the construction of a swimming pool, or any alteration, addition, remodeling or other improvement thereto, an application for a zoning certificate to construct same, and the plans and specifications and pertinent explanatory data hereinafter required, shall be submitted to the Zoning Inspector for approval, and no part of the work shall be commenced until the Zoning Inspector has granted a zoning certificate therefore.

Section 12.3. Fees.

The fee for the zoning certificate to construct, alter, add to or relocate a swimming pool shall be $20.00. Such fee shall be in addition to building permit fees and fees applicable to other structures which may be incidental to the pool.

Section 12.4. Polluted Water.

No swimming pool within the Village which contains sewage, waste or other contaminating or polluting ingredients which renders the water hazardous to health, which fails to meet the requirements of this Section, shall be used for swimming or bathing purposes by any person.

Section 12.5. Shielding Lights.

Lights used to illuminate any swimming pool shall be so arranged and shaded as to reflect light away from adjoining premises and public streets.

Section 12.6. Unnecessary Noise.

No person shall make, continue or cause to be made or continued at any pool, any loud, unnecessary or unusual noise or any noise which annoys, disturbs, injures, or endangers the comfort, repose, health, peace, or safety of others. In the operation of a pool, no person shall use or permit the use of operation of any radio, receiving set, musical instrument, phonograph or other machine or device for the producing or reproducing of sound in such manner as to disturb the peace, quiet and comfort of the neighboring 	inhabitants, or at any time with louder volume than is necessary for convenient hearing of the persons who are in the pool premises.

Section 12.7. Commercial Activities.

The carrying on of any commercial undertaking entailing sales of food, drinks or other merchandise within the enclosure of any swimming pool is prohibited.

Section 12.8. Enclosure of Swimming Pools.

Every swimming pool, as defined in Section 12.1(b), shall be completely enclosed at all points by an aesthetically acceptable fence of sturdy construction not less than four feet in height. Such fence or barrier must effectively prevent a child from crawling or otherwise passing through, over or under it and shall effectively prevent a child from free and unauthorized access to the pool water. Such fence or barrier shall be maintained in good condition and repair and shall include a gate, retractable steps or ladder and/or locking device which shall be kept lifted and/or locked when the pool is not in actual use or is left unattended. Such fence or barrier shall conform to all provisions of Village Ordinances and laws regulating same.

Every swimming pool, including walks, paved areas or accessory structures adjacent thereto, shall be located at least fifteen feet from any property line or the property where located.  (Amended by Ordinance No. 15-1998, Passed April 20, 1998:  An Ordinance amending Ordinance No. 33-1996, Section 12.8, so as to modify the enclosure requirements for Swimming Pools.)

Section 12.9. Location of Pools.

All pools shall be located in the rear yard, not side yards or front yards.

ARTICLE 13 - GAS WELLS AND OIL WELLS

Section 13.1. Location.

13.1.1.	Gas wells and oil wells shall be permitted in the Village only in the Limited Industry District (M-1), General Industry District (M-2) and Agricultural District (A1) and then only as a conditional use requiring Planning Commission approval subject to the rules, requirements, regulations and conditions hereinafter set forth.

Section 13.2. Requirements.

13.2.1. 	No person shall drill, operate or maintain a gas well or oil well without first obtaining a conditional use permit from the Planning Commission. A fee of Two Hundred Fifty Dollars ($250.00) per gas well or oil well shall be paid to the Village of Madison by the driller, producer or owner thereof prior to issuance of a conditional use permit. A separate conditional use permit shall be required for each gas well and oil well.

13.2.2. 	The application for a conditional use permit shall be filed by the property owner, or by another with the owner's written consent and shall include a surveyed site plan drawn to scale of the well, its depth, all buildings and improvements within a radius of three hundred feet (300') of the proposed well, proposed access roads, transmission lines and storage tanks, landscaping and grade changes, proposed drilling dates, a truck route within the Village to and from the site after completion of drilling. Said application shall also include the name, address and telephone number of the person responsible for the operation and maintenance of the site and available for emergency purpose and plans for the removal and disposal of all waste products and any other information required by the Planning Commission. Said application shall also include a pre-drill survey, written report and recommendation from the Madison Fire District and the Village Engineer regarding the site and surrounding area.

13.2.3. 	Prior to commencement of drilling operations, proof of a comprehensive liability insurance policy in the amount of Five Million Dollars ($5,000,000.00) for property damage and Five Million Dollars ($5,000,000.00) for bodily injury and death shall be submitted to the Village of Madison. Said policy shall provide for continuous coverage and remain in full force from commencement of drilling operations until said well is plugged and all storage tanks are emptied and taken out of use. Said policy shall include coverage for blowout and cratering and shall also include underground resources and equipment coverage. The permit holder shall see to it that all acts and omissions of all contractors, subcontractors or other persons drilling or operating the well, or otherwise, engaged in the process, are covered by said policy. The Village of Madison shall be named as an additional insured and shall be notified in writing sixty (60) days prior to cancellation.

13.2.4. 	Prior to commencement of drilling operations, a complete copy of the application for a permit to drill a gas or oil well, and additional information, required by and filed with the State of Ohio, shall be filed with the Village of Madison and the Madison Fire District.

13.2.5. 	Prior to commencement of drilling operations, a construction deposit of Two Thousand Five Hundred Dollars ($2,500.00) shall be paid to the Village of Madison to insure the strict compliance with this Article and restoration of the 	drilling site at the completion of drilling operations. If at the completion of drilling operations the Village or Administrator is satisfied that the site has been restored and that the obligations of this Article have been fully met, then said construction deposit shall be returned. In the event the site is not restored or certain obligations have not been met, then

said construction deposit shall be forfeited to the extent that said funds are necessary to complete restoration or cure nonconformities.

13.2.6. 	Prior to commencement of drilling operations, a performance bond in the form approved by the Solicitor in the amount of Twenty Thousand Dollars ($20,000.00) shall be deposited with the Village of Madison to insure the strict compliance with this Article, all other Village Ordinances and all state and federal laws. Said performance bond shall not only be conditioned upon strict compliance by the permit holder, but also by any assignee, employee, contractor, subcontractor or other person or entity performing services required by this Article, any Village Ordinances or state and federal laws. Said performance bond shall be maintained until the Village Administrator and Village Engineer have approved final abandonment, restoration and compliance with this Article.

13.2.7. 	The minimum size of a gas or oil well site shall be twenty (20) acres and all of said twenty (20) acres shall be under one ownership.

13.2.8. 	No gas well or oil well, storage tank or separator shall be located within three hundred feet (300') of a property line, electrical transmission line, dedicated public street, highway or right-of-way, or nearest rail of a railway. No gas well or oil well shall be located within five hundred feet (500') of any dwelling, structure or building which is not necessary to the operation of said well or any school, church, hospital, theatre, assembly hall or other public building.

13.2.9. 	All drilling operations shall be muffled with appropriate muffling devices so as to prevent unreasonable noise from disturbing neighboring residents or property owners and all lights shall be shielded so as to minimize the effect of drilling on neighboring residents or property owners.

13.2.10. 	Each gas well or oil well shall be sealed to protect fresh ground water wells and natural aquifers from salt water or other pollution or contamination. Both the permit holder and well driller shall establish contingency plans and provide for the immediate furnishing of potable water to affected residents for such period as may be required to reestablish proper potability of any polluted or contaminated well or wells. The permit holder and well driller shall be jointly and severally responsible for the obligation to provide potable water without cost to the residents receiving same.

13.2.11. 	Each gas well or oil well location shall have a legible sign with the name, address and telephone number of the owner and operator, name or number of the lease, and a number of the well printing thereon posted in a conspicuous place.

13.2.12. 	The driller, producer or owner of a gas well or oil well shall construct and maintain a service road to the gas well or oil well site at a level and in a manner which will not permit water to accumulate or pond, with a minimum width of fourteen feet (14'), and with a minimum thickness of six inches (6") of thick stone or gravel base. Said service road shall be properly maintained to keep dust levels down 	during and after drilling operations. A barricade of cable or chain shall be erected and maintained to restrict the use and access of said service road.

13.2.13. 	Drilling rigs or other heavy equipment shall not be permitted to pass over any sewer or water line, except on public roads or highways, unless precautionary measures to prevent damage as prescribed by the Village Engineer are met.

13.2.14. 	At least two (2) fire extinguishers shall be kept and maintained at all gas well and oil well sites where drilling, servicing or repair work is being conducted. Each extinguisher shall have a minimum classification of 40 BC and travel distance shall not be more than twenty-five feet (25').

13.2.15. 	Smoking shall not be permitted at any gas well or oil well site, except as designated and posted in areas approved by the Madison Fire District.

13.2.16. 	The Madison Fire District shall be notified prior to any burn off operations at the gas well or oil well site.

13.2.17. 	In the event the services of any fire protection agency or law enforcement agency are required at the site of an oil well or gas well, the owner, permit holder and well driller shall be jointly and severally responsible for the total cost of said services, including but not limited to wages, fringe benefits, mileage, equipment use, and material cost.

13.2.18. 	Adequate protection shall be provided to the satisfaction of the Madison Fire District to control and prevent the blowout of a gas well and oil well. Al1 protection equipment shall meet the minimum requirements of all federal and state laws and regulations.

13.2.19. 	Only rotary tools using a liquid drilling medium and which are capable of sealing off each oil, gas, brine or fresh water stratum above the producing horizon or objective formation shall be used. When drilling with rotary tools using liquid as a drilling medium, a blowout preventer in good working condition of sufficient size and working pressure rating to control normal hydrostatic pressure for the deepest pool to be penetrated must be installed on the casing through which drilling is being performed. Blowout preventers must be capable of closing off the annulus between casing and drill pipe as well as completely closing off the casing when drill pipe is not in the hole. Blowout preventer systems must include a device which will permit bleed off of hydrostatic pressure.

13.2.20. 	Accumulations of mud and debris shall not be permitted on public highways and roads at any time. The Village Administrator shall have the authority to require mud and debris to be removed from any public highway and road.

13.2.21. 	Open pits shall not be used for the storage of any by-products of drilling operation.

13.2.22. 	Where oil or brine are encountered, they shall be stored in closed steel tanks or cement tanks and shall be periodically emptied and disposed of off the site by legal means. No oil or brine shall be permitted to be dumped on the ground, allowed to percolate through the soil or allowed to be discharged into any stream, ditch or other waterway. No oil or brine shall be permitted to contaminate any water system. All spills of oil or brine shall be cleaned up immediately at the cost 	of the owner, permit holder, and well driller. No tank in which oil or brine is stored shall be located within one hundred feet (100') of any dedicated public street, highway or nearest rail of a railway nor shall it be located within two hundred fifty feet (250') of any dwelling or building not necessary to the operation of said well.

13.2.23. 	Disposal of brine waste into any annular space of any well shall be prohibited. Dumping, discharging or disposing of brine waste on public or private property in the Village of Madison shall be prohibited.

13.2.24. 	Any gas well, oil well or drilling operation which may affect a private or public water supply shall comply with all rules, regulations and recommendations of the Environmental Protection Agency.

13.2.25.	All land within twenty-five feet (25') of any gas well or oil well, flammable or combustible liquid tank or other appurtenance to any such well shall be at all times kept free and clear of dry weeds, grass, rubbish or other combustible material.

13.2.26. 	Natural drainage patterns shall not be altered or disturbed.

13.2.27. 	Functional landscaping shall be provided to adequately screen both the gas well or oil well site and meter house.

13.2.28. 	A six foot (6') high chain link fence with three (3) strands of barbed wire on top shall be installed around each gas well, oil well, pump house, storage pit, tank, separator and other permanent producing facility within thirty (30) days after the well is placed into production.

13.2.29 	Lightning rods shall be properly installed and maintained on all storage tanks.

13.2.30. 	Each gas or oil storage or processing tank shall have a legible sign posted thereon with the name, address and telephone number of the owner or operator, the name, address and telephone number of the lessee and the name, address and telephone number of a responsible person who is available on a twenty-four (24) hour basis. Each such tank shall be equipped with child-proof safety hatch covers to prevent children from falling into any such tank.

13.2.31. 	All buildings, facilities, tanks and appurtenances shall be maintained in a neat and orderly fashion and shall be painted so as to maintain a neat appearance. No debris or accumulations of unused parts or equipment shall be permitted on any gas well or oil well site.

13.2.32. 	Within thirty (30) days after drilling has been completed, all pits required for drilling shall be filled in and the ground restored to its natural grade and contours.

13.2.33. 	Within thirty (30) days after a well is in operation, a site plan shall be delivered to the Village Administrator and the Madison Fire District showing the exact location of the well, all permanent storage facilities, fences, property lines, roads, landscaping, transmission lines and shut-off valves for emergency purposes.

13.2.34. 	A gas well or oil well which is voluntarily abandoned shall be permanently plugged and capped in accordance with state law within thirty (30) days thereafter. Prior 	to plugging or capping a gas well or oil well, a permit shall be obtained from the Village Administrator. In the event an abandoned gas well or oil well is not plugged and capped as provided herein, the Village of Madison shall have the right to plug and cap said well at the expense of the driller, producer or owner of said well.

13.2.35. 	If a gas well or oil well has not produced for eighteen (18) months, it shall be considered abandoned and shall be permanently plugged and capped in accordance with state law within thirty (30) days thereafter. Prior to plugging or capping a gas well or oil well, a permit shall be obtained from the Village Administrator. In the event an abandoned gas well or oil well is not plugged or capped as provided herein, the Village of Madison shall have the right to plug and cap said well at the expense of the driller, producer or owner of said well.

13.2.36. 	An abandoned gas well or oil well shall be plugged and capped above ground and shall be visible above the surface grade.

13.2.37. 	After an abandoned gas well or oil well is plugged and capped, the area shall be restored in accordance with the restoration plan filed with the conditional use permit application, and the Village Administrator shall be notified for his acceptance. The owner, occupier, user or driller of said well shall not be freed of his obligations hereunder until the Village Administrator has given his written approval of acceptance of said plugging and capping.

13.2.38. 	All buildings, facilities, service tanks, fences, gates, access roads and appurtenances shall be removed from the gas well or oil well site within six (6) months after a gas well or oil well is plugged and capped. In the event they are not removed as provided herein, the Village of Madison shall have the right to remove them at the expense of the owner, occupier, user or driller of said well.

13.2.39. 	No person, firm, corporation or other entity shall discharge waste, sludge, water, oil or other effluent into any storm or sanitary sewer or upon any public street, highway or right-of-way in the Village of Madison.

13.2.40. 	All other applicable federal, state and local rules, regulations, laws and ordinances relating to gas well or oil wells and any additional conditions set forth in the conditional use permit shall be strictly complied with.

13.2.41. 	Monthly production reports shall be submitted to the Village of Madison for each gas well or oil well.

13.2.42. 	The Village Administrator, the Police Chief or his authorized representative, and the Fire Chief or his authorized representative, shall have the authority at any time to enter upon the property where a drilling site is contemplated, upon the property where a well is being drilled, or upon the property where a well is located for the purpose of inspecting the site, equipment and all other things necessary to insure strict compliance with this Ordinance, other Village Ordinances or state laws.

13.2.43. 	The approval of the conditional use permit shall become null and void if drilling operations are not commenced within a six (6) month period after the date of approval.

13.2.44. 	The conditional use permit provided for herein may be revoked by the Planning Commission at any time the rules, requirements, regulations and conditions set forth herein, or in the conditional use permit, are not complied with or are violated. Upon revocation of the conditional use permit, all drilling and production operations shall immediately cease.

Section 13.3. Penalty

13.3.1	 Any person, firm or corporation who violates any of the provisions of this Ordinance shall be deemed guilty of a misdemeanor of the third degree and shall be fined not more than Five Hundred Dollars ($500.00), or imprisoned for a term of not more than sixty (60) days, or both, for each such offense. A separate offense shall be deemed to have been committed each day during which a violation continues.

ARTICLE 14 - GAME ROOMS

Section 14.1. Definitions.

14.1.1. 	"Game Room" means a building, room, enclosure, premises, place, or establishment in the Village which contains more than four (4) mechanical amusement devices, including bowling machines or pool tables, other than juke boxes, which require a cost or coins or tokens for the operation of said mechanical amusement device.

14.1.2. 	"Mechanical Amusement Device" means a machine or device which upon insertion of a coin or slug operates or may be operated for use as a game, contest, or amusement of any description or which may be used for any such game, contest, or amusement, other than juke boxes.

Section 14.2. Requirements.

14.2.1. 	No person, firm, corporation or association shall engage in the business of operating a game room within the corporate limits of the Village of Madison without first having applied for and obtained from the Village Administrator a license as provided for in this Article.

14.2.2. 	Application for the license shall be made to the Village Administrator on forms to be prescribed by him and shall state the name and address of the applicant, if individuals; if a firm or partnership, the names, residences and occupations of the manager and each member of the firm or partnership; and, if a corporation, the names of its officers and manager, and such other information as the Administrator may require. The Village Administrator shall transmit such application for inspection and investigation to the Fire Chief and Police Chief. The Fire Chief and Police Chief each shall, after such inspection and investigation as they may deem necessary, make a report with their recommendations for approval or disapproval of such application. At the time the application is made there shall also be submitted a plan showing parking and exits from the game room and this plan shall be submitted to the Planning Commission for their approval. If the application and the plan are approved by the Planning Commission, Fire Chief, and Police Chief, the Village Administrator shall issue a license upon payment of the proper fee.

The application for a license shall be rejected if the reports submitted thereon shows that any person named in the application is not of good moral character or that the game room sought to be licensed does not comply with the health and fire regulations applicable thereto.

14.2.3. 	Any person, firm, corporation or association to which a license is granted shall pay an annual fee of Fifty Dollars ($50.00).

All licenses granted under the provisions of this Article shall expire on July 1. The license shall be displayed in a conspicuous place in the game room for which it is issued. No transfer of any license shall be permitted.

14.2.4. 	The interior of the game room shall provide that there shall be a minimum area for mechanical amusement device equal to the size of the mechanical amusement device plus one (1) foot of area on each side of the mechanical amusement 	device plus an area of four (4) feet in front of each mechanical amusement device.

The interior of the game room shall also contain sufficient toilet facilities as required and defined by the provisions of the Ohio Basic Building Code which are in effect at the time that application for a license is made.

14.2.5. 	Each operator of a game room shall provide one (1) responsible adult supervisor having attained the age of twenty-one (21) years for each ten (10) games or a fraction thereof that may be located on said game room premises. Said supervisor shall maintain the decorum of the game room by keeping the aisleways free for passage throughout the game room and shall further control the amount of noise and general conduct of the users of said game room.

14.2.6.	The hours of operation of said game room shall not commence earlier than 1:00 p.m. nor shall they extend beyond the hour of 10:00 p.m.

14.2.7. 	The Mayor, upon recommendation of the Village Administrator, Fire Chief, Police Chief or any of them, may at any time revoke or suspend licenses granted under authority of this Article for the failure to comply with the terms of this Article or any rules, regulations, statutes or Ordinances of the State of Ohio or the Village of Madison, Ohio. In addition thereto, the Mayor shall have the full power and authority to revoke the license of any firm, person, corporation or association which is convicted of a violation of any criminal law of the State of Ohio or of the Village of Madison.

Section 14.3. Penalty.

14.3.1. 	Any firm, person, corporation or association who violates any of the provisions of this Article shall be deemed guilty of a minor misdemeanor and shall be fined not more than One Hundred Dollars ($100.00) for each offense. A separate offense shall be deemed to have been committed each day during which a violation continues.

ARTICLE 15 - PLANNING COMMISSION

Section 15.1. Organization and Procedures.

15.1.1.	Appointment: There is hereby established a Planning Commission which shall consist of the Mayor, one (1) member of Council and three (3) citizens of the Village to be appointed by the Mayor for terms of six (6) years each. Vacancies shall be filled by the same procedure. Each member shall serve until his successor is appointed and qualified. Members of the Commission shall be removed for nonperformance of duty, misconduct in office or other cause, by the Council, upon written charges having been filed with the Clerk-Treasurer of council and after a public hearing has been held regarding such charges, a copy of the charges having been served upon the member to be charged at least ten (10) days prior to the hearing, either personally or by registered mail or by leaving the same at his usual place of residence. The member shall be given an opportunity to be heard and answer all charges.

15. 1.2. Organization and Procedure.

a. The Planning Commission shall organize annually to elect a Chairman, ViceChairman and Secretary. It shall further adopt rules for its own government not inconsistent with law or with any other Ordinances of the Village to carry into effect the provisions of this Ordinance.

b. Meetings of the Commission shall be held at the call of the Chairman and at such other times as the Commission may determine. The Chairman, or in his absence, the Vice-Chairman, may administer oaths, and the Commission may compel the attendance of witnesses. All meetings of the Commission shall be open to the public. The Commission shall keep minutes of its proceedings showing the vote of each member upon each question, or, if absent or failing to vote, indicating such fact and shall keep records of its examination and other official actions, all of which shall be immediately filed with the Clerk-Treasurer of Council and shall be made a public record.

c. Three (3) members of the Commission shall constitute a quorum. The Commission shall act by motion and the concurring vote of at least three (3) members of the Commission shall be necessary to decide in favor of an applicant on any matter of which the Commission has original jurisdiction under this Ordinance.

d. The Commission may call upon the various departments of the Village for assistance in the performance of its duties and it shall be the duty of such departments to render such assistance to the Commission as may reasonably be required.

e. No Commission member shall participate in the process of decision, approval, disapproval or recommendation regarding any matter before the Commission in which he, a member of his family or any of his business associates has an interest.

Section 15.2. Conditional Use Permits.

Conditional use permits shall be required for certain types of main uses, so classified because of their uncommon or unique characteristics, infrequency of occurrence, large area requirements or for other reasons. Such use shall not be permitted by right. The conditional uses enumerated throughout this Ordinance may, however, be permitted under specific conditions in the district enumerated, or they may be prohibited by the Commission after making a determination for conformance with the following procedures and standards.

15.2.1. 	Application for conditional use permits shall be made by the owner, lessee, vendee, or developer of property and shall consist of such plans and/or drawings and/or a statement as necessary to fully describe all elements of the proposed use. Such data supplied with the permit application shall describe in detail the proposed use to the extent that the Commission can have no doubt as to the development of the proposed use and can determine the effect upon surrounding properties, and further can evaluate the effect upon traffic, fire hazards, public utilities and the public health, safety and welfare. Application for such permits shall be made to the Commission and it shall hold a public hearing thereon, notice of which shall be mailed to the property owners within two hundred feet (200~) feet of the proposed use, and further, notice of the aforesaid hearing shall be published in a newspaper of general circulation in the municipality at least fifteen (15) days prior to the hearing. A fee of fifty dollars ($50.00) plus two dollars ($2.00) for each property owner notified over ten (10) shall be paid at the time the application is filed and shall be payable to the Clerk-Treasurer to the credit of the General Revenue Fund.

15.2.2. 	Conditional uses shall not be expanded or modified unless a new permit is issued after following the aforesaid procedure.

Section 15.3. Standards for Evaluating Conditional Use Permits.

An application for a conditional use permit shall not be approved unless the Commission finds that it complies with the following conditions and standards:

15.3.1. 	To be approved in a residential district it shall be found that:

15.3.1-A.	 The proposed use would be properly located in relation to any adopted land use and street plan, particularly in proper relation to the secondary and local streets and pedestrian circulation pattern;

15.3.1-B. 	The proposed use would generate only a minimum of traffic through a residential neighborhood if located along a local street;

15.3.1-D. 	The location, design and operation of such use would not discourage or interfere with the appropriate development or impair the value of the surrounding residential district.

15.3.2. 	To be approved in a Business or Industrial District it shall be found that:

15.3.2-B. 	The proposed use would not be closer than appropriate in the particular situation to schools, churches and other places of assembly;

15.3.2-C. 	The location, size, intensity and layout of the proposed use and operation would conform to the noise, smoke, dust, odors, fumes, vibrations and/or glare performance standards of the districts;

15.3.2-D. 	The proposed use would form a harmonious part of the business or industrial district, considering such features as convenience of access and relationship of one use to another;

1 5.3.2-E. 	Because of its limited size, modern processes or equipment, the performance of the proposed use is such that it should properly be permitted in a less restrictive district than the district in which it is permitted by right; and,

15.3.2-F. 	The hours of operation of the proposed use is similar to a use permitted in the district and that the proposed use will not generate more traffic than normal for the district.

Section 15.4. Approval of Conditional Use Permits.

The Planning Commission shall make a determination based on the information available or it may request additional information. If a conditional use is approved, the Commission shall set forth any specific terms, conditions and safeguards that shall be required so that the proposed use will conform with the intent and standards of the district, and it shall instruct in writing the Zoning Inspector to issue a Conditional Use Permit, which shall be posted and openly displayed on the premises for which the Conditional Use Permit has been issued. The Council shall be notified of any such action.

After the effective date of this Ordinance an application for a Conditional Use Permit may be submitted by an owner for any existing use requiring such permit, and it shall be issued subject to approval based on the standards set forth in Section 15.3.

Section 15.5. Revocation of Conditional Use Permits.

The approval of the Conditional Use Permit shall become null and void if the conditional use is not carried out within a six-month period after date of approval. The Commission may revoke the Conditional Use Permit upon written evidence by any citizen or official of violation of this Ordinance and/or written terms and conditions upon which approval was granted. A Conditional Use Permit shall not be transferred to another owner, assignee or lessee of such use.

Section 15.6. Planning Commission.

The Planning Commission shall be the Platting Commission for the Village of Madison with all of the powers and duties set forth in Ohio Revised Code, Section 735.17 through 735.26.

ARTICLE 16 - BOARD OF ZONING APPEALS

Section 16.1. Organization and Procedures.

16.1.1. 	Appointment: There is hereby established a Board of Zoning Appeals, which shall consist of three (3) electors appointed by the Mayor, subject to confirmation by the Council. Vacancies shall be filled by the same procedure. Each member shall serve until his successor is appointed and qualified. Members of the Board shall be removed for nonperformance of duty, misconduct in office or other cause, by the Council, upon written charges having been filed with the Clerk Treasurer of Council and after a public hearing has been held regarding such charges, a copy of the charges having been served upon the member so charged at least ten (10) days prior to the hearing, either personally or by registered mail or by leaving the same at his usual place of residence. The member shall be given an opportunity to be heard and answer all charges.

16.1.2. Organization and Procedure:

a. The Board shall organize annually to elect a Chairman, a ViceChairman and Secretary. It shall further adopt rules for its own government not inconsistent with law or with any other ordinances of the Village to carry into effect the provisions of this Ordinance.

b. Meetings of the Board shall be held at the call of the Chairman and at such other times as the Board may determine. The Chairman, or in his absence, the Vice-Chairman, may administer oaths, and the Board may compel the attendance of witnesses. All meetings of the Board shall be open to the public. The Board shall keep minutes of its proceedings showing the vote of each member upon each question, or, if absent or failing to vote, indicating such fact and shall keep records of its examinations and other official actions, all of which shall be immediately filed with the Clerk-Treasurer of Council and shall be made a public record.

c. Two (2) members of the Board shall constitute a quorum. The Board shall act by motion and the concurring vote of at least two (2) members of the Board shall be necessary to reverse any order or determination of the Zoning Inspector or to decide in favor of an applicant on any matter of which the Board has original jurisdiction under this Ordinance or to grant any variance from the requirements stipulated in this Ordinance.

d. The Board may call upon the various departments of the Village for assistance in the performance of its duties, and it shall be the duty of such departments to render such assistance to the Board as may reasonably be required.

e. No Board member shall participate in the process of decision, approval, disapproval or recommendation regarding any matter before the Board in which he, a member of his family or any of his business associates has an interest.

Section 16.2. Powers of the Board of Zoning Appeals.

16.2.1. Exceptions and Interpretations.

a. 	The Board shall have the power to hear and decide, in accordance with the provisions of this Article, applications filed as herein before provided for special exceptions or for interpretation of the Zoning Ordinance or Map or for decisions upon other special questions on which the Board is authorized by this Article to pass. In considering an application for a special exception or interpretation of the Zoning Map, the Board shall give due regard to the nature and condition of all adjacent uses and structures; and, in authorizing a special exception, the Board may impose such requirements and conditions with respect to location, construction, maintenance and operation in addition to those expressly stipulated in this Article for the particular special exception as the Board may deem necessary for the protection of adjacent properties and the public interest.

16.2.2. Administrative Review and Variances.

a. Administrative Review: The Board shall have the power to hear and decide appeals, filed as herein before provided, where it is alleged by the appellant that there is error in any order, requirement decision, grant or refusal made by the Zoning Inspector or other official in the interpretation or of the provisions of this Ordinance.

b. Variances: The Board shall have the power to authorize, upon appeal in septic cases filed as herein before provided, such variances from the provisions or requirements of this Ordinance as will not be contrary to the public interest, but only in such cases where owing to special conditions pertaining to a specific piece of property, the literal enforcement of the provisions or requirements of this Ordinance would cause undue and unnecessary hardship.

c. Where, by reason of the exceptional narrowness, shallowness or unusual shape of a specific piece of property on the effective date of this Ordinance, or by reason of exceptional topographic condition or other extraordinary situation or condition of such piece of property or of the use or development of property immediately adjoining the piece of property in question, the literal enforcement of the requirements of this Ordinance would involve practical difficulties or would cause undue hardship unnecessary to carry out the spirit and purpose of this Ordinance the Board shall have power to authorize a variance from such strict application so as to relieve such hardship and so that the spirit and purpose of this Ordinance shall be observed and substantial justice done. In authorizing a variance the Board may attach thereto such conditions regarding the location, character and other features of the proposed structure or use as it may deem necessary in the interest of the furtherance of the purpose of the Ordinance and in the public interest. In authorizing a variance with attached conditions the Board shall require such evidence and guarantee or bond as it may deem to be necessary that the conditions attached are being and will be complied with.

d.  No such variance in the provisions or requirements of this Ordinance shall be authorized

by the Board unless the Board finds, beyond reasonable doubt, that all the following facts

and conditions exist:

(1 ) That there are exceptional or extraordinary circumstances or conditions applying to the property in question or to the intended use of the property that do not apply generally to other properties or classes of uses in the same zoning district;

(2) That such variance is necessary for the preservation and enjoyment of substantial property rights possessed by other properties in the same zoning district and in the same vicinity; and,

(3) That the authorizing of such variance will not be of substantial detriment to adjacent property and will not materially impair the purposes of this Ordinance or the public interest;

e. No grant of a variance shall be authorized unless the Board specifically finds that the condition or situation of the specific piece of property, or the intended use of said property for which variance is sought, one or the other in combination, is not of so general or recurrent a nature as to make reasonably practicable the formulation of a general regulation for such conditions or situation.

f. In exercising its power the Board may, in conformity with the provisions of State statutes and of this Ordinance, reverse or affirm, wholly or in part, or may modify the order, requirement, decision or determination appealed from and may make such order, requirement, decision or determination as ought to be made and to that end shall have all powers of the office from whom the appeal is taken.

16.2.3. 	Performance Requirements Procedure.

The Board shall have the power to authorize, upon application in specific cases filed as hereinafter provided, issuance of a zoning certificate for uses that are subject to performance requirements procedure under Article 8 of this Ordinance, as provided in the following:

a. 	Application: An application for a zoning certificate for a use subject to performance requirements shall be submitted in duplicate on a form prescribed by the Board. The applicant shall also submit in duplicate a plan of the proposed construction or development, including a description of the proposed machinery, processes and products, and specifications for the mechanisms and techniques to be used in restricting the emission of dangerous and objectionable elements as set forth in Article 8 in accordance with rules prescribed by the Board specifying the type of information required in such plans and specifications. The fee for such application shall include the cost of the special reports that may be required to process it, as set forth in Subsection b below.

b. 	Report by Expert Consultants: If, in its opinion, the proposed use may cause the emission of dangerous or objectionable elements, the Board may refer the application to one or more expert consultants qualified to advise as to 	whether a proposed use will conform to the applicable performance standards in Article 8 for investigation and report. Such consultant or consultants shall report as promptly as possible after his or their receipt of such application. A copy of such report shall be promptly furnished to the applicant.

c.	Review by Board: Within thirty (30) days after the Board has received the aforesaid application or the aforesaid report, if a report was required, or within such further period as agreed to by the applicant, the Board shall decide whether the proposed use will conform to the applicable performance standards and on such basis shall authorize or refuse to authorize issuance of a zoning certificate or require a modification of the proposed plan of construction or specifications, proposed equipment or operation. Any zoning certificate so authorized and issued shall be conditioned upon among other things (1) the applicants completed buildings and installations conforming in operation to the applicable performance standards and (2) the applicant paying the fees for services of the expert consultant or consultants deemed reasonable and necessary by the Board to advise the Board as to whether or not the applicants completed buildings and installation in operation will meet the applicable performance standards.

d.	Continued Enforcement: The Zoning Inspector shall investigate any purported violation of performance standards and if there is reasonable ground for the same, shall notify the Board of occurrence or existence of a probable violation thereof. The Board shall investigate the alleged violation, and for such investigation shall employ qualified experts. If, after public hearing on due notice, the Board finds that a violation occurred or exists, a copy of said findings shall be forwarded to the Council. The services of any qualified experts, employed by the Board to advise in establishing a violation, shall be paid by the violator if said violation is established, otherwise by the Village.

Section 16.3. Applications and Appeals.

16.3.1. 	An application in cases in which the Board has original jurisdiction under the provisions of this Ordinance, may be filed by any person aggrieved, including a tenant, or by a governmental officer, department, board or bureau. Such application shall be filed with the Zoning Inspector, who shall transmit same to the Board. A fee of fifty dollars ($50.00) plus $2.00 for each owner notified over 10 shall be paid to the Zoning Inspector at the time notice of appeal is filed, which the officer shall forthwith pay over to the Clerk-Treasurer to the credit of the General Revenue Fund.

16.3.2. 	An appeal to the Board may be taken by any person aggrieved or by an officer of the Village affected by any decision of the Zoning Inspector. Such appeal shall be taken within twenty (20) days after the decision by filing with the Zoning Inspector and with the Board a notice of appeal specifying the grounds thereof. The Zoning Inspector shall forthwith transmit to the Board all the papers constituting the record upon which the action appealed from was taken.

16.3.3. 	An appeal shall stay all proceedings in furtherance of the action appealed from, unless the Zoning Inspector shall certify to the Board of Zoning Appeals after the notice of appeal has been filed that by reason of facts stated in the certificate a stay would, in his opinion, cause imminent peril to life or property, in which case 	proceedings shall not be stayed otherwise than by a restraining order, which may be granted by the Board or by a court of competent jurisdiction, after notice to the officer from whom the appeal is taken and on just cause shown.

16.3.4. 	The Board may in conformity with the provisions of this Ordinance reverse or affirm, wholly or in part, or may modify the order, requirement, decision or determination appealed from and shall make such order, requirement, decision or determination as in its opinion ought to be made. To that end, the Board shall have all powers of the Zoning Inspector from whom the appeal is taken.

Section 16.4. Hearings.

16.4.1. 	The Board shall fix a reasonable time for the hearing of an appeal and shall give public notice thereof in a newspaper of general circulation in the Village one time at least fifteen (15) days prior to the hearing and by certified mail, return receipt requested, on all adjoining property owners and parties in interest at least fifteen (15) days prior to the hearing. Each application or notice of appeal shall be accompanied by the fee hereinafter specified. At this hearing, any party may appear in person or by an attorney.

16.4.2. 	The hearings of the Board shall be public.

16.4.3. 	Upon the day for hearing any application or appeal, the Board may adjourn the hearing in order to permit the obtaining of additional information or to cause such further notice as it deems proper to be served upon such other property owners as it decides may be substantially interested in said application or appeal. In these cases of an adjourned hearing, persons previously notified and persons already heard need not be notified of the time of resumption of said hearing unless the Board so decides.

Section 16.5. Decisions.

16.5.1. 	The Board shall decide all applications and appeals within thirty (30) days after the final hearing thereon.

16.5.2. 	A certified copy of the Board's decision shall be transmitted to all parties in interest. Such decision shall be binding upon the Zoning Inspector and observed by him, and he shall incorporate the terms and conditions of the same in the permit to the applicant or appellant whenever a permit is authorized by the Board.

    - A decision of the Board shall not become final until the expiration of fifteen (15) days from the date such decision is made unless the Board finds and determines that the immediate taking effect of such decision is necessary for the preservation of property or personal rights and shall so certify on the record.

Section 16.6. Appeals from Board of Zoning Appeals to Council.

16.6.1. 	Any person, firm or corporation, or any officer, department, board or agency of the Village, or any interested elector of the Village who has been aggrieved or affected by any decision of the Board of Zoning Appeals may appeal from such decision to the Council of the Village by filing a Notice of Appeal setting forth the facts of the case and the reasons for appeal with the Clerk-Treasurer of Council within fifteen (15) days from the date of the decision.

    - Council shall hold a public hearing on such appeal not less than thirty (30) days after such Notice of Appeal has been filed with its Clerk-Treasurer. Notice of the hearing shall be given as provided in Section 16.4.1. Council, by an affirmative vote of a majority of its members, shall decide the matter within fifteen (15) days after the final hearing thereon.

16.6.3.	The filing of a Notice of Appeal shall stay all proceedings in furtherance of the action appealed from unless the Board of Zoning Appeals has found and determined that its decision requires immediate effect because it is necessary for the preservation of property or personal rights and has so certified on the record.

(Amended by Ordinance No. 24-1999; Passed on May 17, 1999.  An Ordinance amending Ordinance No.

33-1996, as amended, Sections 16.6 and 16.5.3, so as to modify the time for filing and deciding appeals

from decisions of the Board of Zoning Appeals to Council.)

ARTICLE 17 - ENFORCEMENT, ZONING CERTIFICATES AND FEES

Section 17.1. Zoning Inspector.

17.1.1. 	It shall be the duty of the Zoning Inspector, who shall be appointed by the Council, to enforce this Ordinance. It shall also be the duty of all officials and employees of the Village to assist the Zoning Inspector by reporting new construction, reconstruction or land uses or apparent violations to him.

17.1.2. 	Appeal from the decision of the Zoning Inspector may be made to the Board of Appeals, as provided in Article 16.

Section 17.2. Zoning Certificates.

17.2.1.	It shall be unlawful for an owner to use or to permit the use of any structure, building or land or part thereof, hereafter created, erected, changed, converted or enlarged, in whole or in part, until a zoning certificate shall have been issued by the Zoning Inspector. It shall be the duty of the Zoning Inspector to issue a certificate, provided he is satisfied that the structure, building or premises and the proposed use thereof conform with all the requirements of this Ordinance. No permit for excavation or construction shall be issued by the Zoning Inspector unless the plans, specifications and the intended use conform to the provisions of this Ordinance.

17.2.2.	Under written request from the owner or tenant, the Zoning Inspector shall issue a zoning certificate for any building or premises existing at the time of enactment of this Ordinance certifying, after inspection, the extent and kind of use made of the building or premises and whether such use conforms to the provisions of this Ordinance.

Section 17.3. Conditions Under Which Zoning Certificates are Required.

A zoning certificate shall be required for any of the following, except as otherwise herein provided:

17.3.1 . 	Construction or structural alteration of any building, including accessory buildings;

17.3.2. 	Change in use of an existing building or accessory building to a use of a different classification;

17.3.3. 	Occupancy and use of vacant land;

17.3.4. 	Change in the use of land to a use of a different classification; and,

17.3.5. 	Any change in the use of a non-conforming use.

Section 17.4. Application and Issuance of Zoning Certificates.

17.4.1. 	Written application for a zoning certificate for the construction of a new building or for the alteration of an existing building shall be made at the same time as the application for a building permit. Said certificate shall be issued within ten (10) days after a written request for the same has been made to the Zoning Inspector 	or his agent, provided such construction or alteration is in conformity with the provisions of this Ordinance.

17.4.2. 	Written application for a zoning certificate for the use of vacant land or for a change in the use of land or of a building or for a change in a non-conforming use, as herein provided, shall be made to the Zoning Inspector. If the proposed use is in conformity with the provisions of this Ordinance, the certificate therefor shall be issued within ten (10) days after the application for same has been made.

17.4.3. 	Every application for a zoning certificate shall be accompanied by a plot plan drawn to scale and such other plans as may be necessary to show the use, location and/or type of buildings to be erected or alterations to be made.

Section 17.5. Fees for Zoning Certificates.

17.5.1. 	A fee, in accordance with the following schedule, shall accompany each application for a zoning certificate, and shall be deposited to the credit of the General Fund of the Village:

Building                                                       Fee

Single-family Dwelling                               $50.00

Accessory Building                                     $10.00 (Under 100 sq. ft.)

Accessory Building                                     $20.00 (Over 100 sq. ft.)

Two-family Dwelling                                  $70.00

Multiple Dwelling                                        $50.00 (First Unit Plus $20.00 for each

additional unit)

Addition or alteration to

existing dwelling                                          $25.00

Commercial or Industrial                             $0.03/sq. ft. with a minimum of $100.00

(Construction, addition,                                 and maximum of $500.00

alteration)

The fee for the application for a zoning certificate for the use of land not involving structures, including changes in the use of land, shall be $10.00. Additional fees shall be as follows:

Zoning Code:	$10.00

Zoning Map:	$10.00

Fence Permit:	$10.00

Pool Permit:	$20.00

Satellite Dish Unit Permit:	$10.00

Transmission Tower Permit:	$50.00

Conditional Use Permit:	$50.00 + $2.00 for each notified over 10

BZA Application:	$50.00 + $2.00 for each notified over 10

Rezoning Application:	$50.00 + $5.00 for each notified over 10

17.5.2. 	Every zoning certificate shall state that the building or the proposed use of a building or land complies with all provisions of law. A record of all zoning certificates shall be kept on file in the Office of the Zoning Inspector or his agent and copies shall be furnished on request to any person having proprietary or tenancy interest in the building or land affected.

Section 17.6. Zoning Certificates for Nonconforming Uses.

A zoning certificate shall be required for all lawful nonconforming uses of land or buildings created by adoption of the Ordinance. It shall be the duty of the Zoning Inspector to issue a certificate for a lawful non-conforming use upon request of the owner. Refusal of the Zoning Inspector to issue a certificate for such non-conforming use shall be evidence that said non-conforming use was either illegal or did not lawfully exist at the effective date of this Ordinance.

Section 17.7. Violations and Penalties.

It shall be unlawful to locate, erect, construct, reconstruct, enlarge change maintain or use any building or land in violation of any regulation in or any provisions of this Ordinance or any amendment or supplement thereto adopted by the Council. Unless otherwise provided for herein, any person, firm or corporation violating any regulation in or any provision of this Ordinance or any amendment or supplement thereto shall be deemed guilty of a misdemeanor, and upon conviction thereof, shall be fined not more than One Hundred Dollars ($100.00). Each and every day during which such illegal location, erection, construction, reconstruction, enlargement, change, maintenance or uses continues may be deemed a separate offense.

Section 17.8. Violations and Remedies.

In case any building is, or is proposed to be, located, erected, constructed, reconstructed, enlarged, changed, maintained or used, or any land is, or is proposed to be used in violation of this Ordinance or any amendment or supplement thereto, the Zoning Inspector, the legal representative of the Village, or any adjacent or neighboring property owner who would be specially damaged by such violation, in addition to other remedies provided by law, may institute injunction, mandamus, abatement or any other appropriate action, actions, proceeding or proceedings to prevent, enjoin, abate or remove such unlawful location, erection, construction, reconstruction enlargement, change, maintenance or use.

ARTICLE 18 - DISTRICT CHANGES AND ORDINANCE AMENDMENTS

Section 18.1. General.

Whenever the public necessity, convenience, general welfare, or good zoning practice requires, the Council may by Ordinance, after receipt of a recommendation thereon from the Planning Commission, and subject to the procedure provided by law, amend, supplement or change the regulations, district boundaries or classifications of property, now or hereafter established by this Ordinance or amendments thereof. It shall be the duty of the Commission to submit its recommendations regarding all applications or proposals for amendments or supplements to the Council.

Section 18.2. Initiation of Amendments.

A proposed amendment of the Zoning text or map may be initiated by the Planning Commission or Council. If initiated by Council it shall be referred to the Planning Commission. It may be initiated at the request of the owner of the property involved or any other person having an interest in the premises affected in which event the proposed change shall be in the form of a petition and accompanied by an accurate map drawn to scale of the property proposed to be changed and a legal description shall be subject to approval of the Village Engineer or Village Solicitor. The person or persons requesting the change shall also furnish to the Planning Commission the last known name and address of the owners of all property within 200 feet of the area proposed to be changed, as shown upon the records of Lake County. Where all property within 200 feet is under the same ownership as the property proposed to be changed, the names and address of owners of all property adjoining that included in the proposed change shall be furnished to the Planning Commission.

Section 18.3. Action by Planning Commission.

18.3.1. 	The Commission shall be allowed not more than forty-five (45) days after the first regular meeting, after the receipt of the proposed amendment to consider the proposal. A public hearing may be held at the discretion of the Commission or at the request of the owner of the property involved or any other person having an interest in the property, for any proposed amendment to the zoning text or map. If a public hearing is to be held, notice of the time, place and purpose of such hearing shall be given by:

a. 	Publication at least twice in a newspaper of general circulation in the Village. The first publication shall be not less than ten (10) days prior to the date of the hearing.

b.	Where the proposed amendment is to effect a change in the Zoning Map, written notice of the hearing shall be mailed by the Secretary of the Commission, by first class mail, at least five (5) days prior to the date of such hearing, to the owners of all property within two hundred (200) feet or adjoining property as prescribed in Section 18.2. The failure of delivery of such notice shall not invalidate the proceedings or findings of the Commission.

18.3.2. 	The Commission shall be allowed not less than ten (10) nor more than forty-five (45) days, unless extended by Council, after said hearing for submitting their recommendations or an amendment to the Zoning text or map to Council.

Section 18.4. Action by Council.

18.4.1. 	After receiving recommendations or a proposed amendment from the Commission or after the 45-day no decision period by the Commission, the Council shall advertise and hold a public hearing thereon. For all proposed text or map amendments, at least one (1) notice of said public hearing shall be given in a newspaper of general circulation in the Village at least thirty (30) days prior to the date of the hearing.

18.4.2. 	Where the proposed amendment is to effect a change in the Zone Map, written notice of the hearing shall be mailed by the Clerk-Treasurer of Council, by first class mail, at least twenty (20) days prior to the date of said hearing, to the owners of all property within two hundred (200) feet or adjoining property as described in Section 18.2. The failure of delivery of such notice shall not invalidate any such amendment.

18.4.3. 	The published and mailed notification shall set forth the time and place of the public hearing and a summary of the proposed amendment. During the thirty (30) days' advertising period the text or copy of the text of such Ordinance or amendment and the maps, plans and reports submitted by the Commission shall be on file, for public examination, in the office of the Clerk-Treasurer of Council or in such other office as designated by Council.

18.4.4 	After the public hearing the Council may adopt the proposal as recommended by the Commission by majority vote of its entire membership. If the Council modifies the proposal it may resubmit the proposed modification to the Commission for further consideration and approval. The Council may adopt the modified proposal without resubmitting it to the Commission and without the approval of the Commission by a five-sixths (5/6) vote of the full membership of the Council. The Clerk-Treasurer of the Council shall submit a copy of any action in regard to this Article to the Commission.

Section 18.5. Application Fees.

18.5.1. 	At the time that an application for a change of zoning districts is filed with the Planning Commission, as provided herein, there shall be deposited with the Clerk Treasurer the sum of Fifty Dollars ($50.00) as a fee to cover investigation, legal notice and other expenses incidental to the determination of such matter, such fee to be for one (1) lot or part of one (1) lot. An additional fee of Five Dollars ($5.00) shall be deposited for each additional lot or part of an additional lot which may be included in the request, such additional lot or part of a lot to be adjacent to each other. Such sums so deposited shall be credited by the Clerk-Treasurer to the General Fund.

ARTICLE 19 - ZONING DISTRICT MAP

The Zoning District Map attached hereto, incorporated herein and marked Exhibit "A," be and the same hereby is approved and adopted as the official Zoning District Map of the Village of Madison. The parcels of property identified by the legal descriptions attached hereto, incorporated herein and marked Exhibits "1 through 35," be and hereby are rezoned from their current zoning classification to a new zoning classification consistent with, and as designated on, the official Zoning District Map.

ARTICLE 20 - INTERPRETATION. PURPOSE AND CONFLICT

Section 20.1. General.

In interpreting and applying the provisions of this Ordinance they shall be held to the minimum requirements for the promotion of the public safety, health, convenience, comfort, morals, prosperity and general welfare. It is not intended by this Ordinance to interfere with or abrogate or annul any ordinance, rules, regulations or permits previously adopted or issued and not in conflict with any of the provisions of this Ordinance or which shall be adopted or issued pursuant to law relating to the use of buildings or premises and likewise not in conflict with this Ordinance; nor is it intended by this Ordinance to interfere with or abrogate or annul any easements, covenants or other agreements between parties; provided, however, that where this Ordinance imposes a greater restriction upon the use of buildings or premises or upon the height of buildings or requires larger open spaces or larger lot areas than are imposed or required by such other ordinances or agreements, the provisions of this Ordinance shall control.

ARTICLE 21 - SEPARABILITY

If any article, section, subsection, paragraph, sentence or phrase of this Ordinance is for any reason held to be invalid by a court of competent jurisdiction, such decision shall not affect the validity of the remaining portion of this Ordinance.

ARTICLE 22 - EFFECTIVE DATE

This Ordinance shall become effective on September 1, 1996.

ARTICLE 23 - REPEAL OF ORDINANCE NO. 1-1963. AS AMENDED

Section 23.1 General.

Ordinance No. 1-1963, as amended, be and hereby is repealed effective upon the date this new Zoning Ordinance takes effect.

ARTICLE 24 - OPEN MEETING

It is found and determined that all formal actions of this Council concerning and relating to the passage of this Ordinance were adopted in an open meeting of this Council, and that all deliberations of this Council and of any of its committees that resulted in such formal action were in meetings open to the public, in compliance with all legal requirements, including Section 121.22 of the Ohio Revised Code.

PASSED

ADOPTED

/s/ David G. Reed, Jr.

Mayor

ATTEST /s/ William Hufgard

Clerk-Treasurer

SUBDIVISION REGULATIONS

FOR

MADISON VILLAGE, OHIO

TABLE OF CONTENTS

SUBJECT							PAGE

I.  GENERAL PROVISIONS							95

SECTION 	1.  Authority						95

2.  Short Title						95

3.  Interpretation and Purpose				95

4.  Effective Date					95

5.  Scope						95

6.  Jurisdiction						96

7.  Authority and Administration			96

8.  Variances and Exceptions				97

9.  Amendments					97

10.  Definitions					97

11.  Separability					99

12.  Public Hearing					99

II.  PROCEDURE								100

SECTION	1.  Submission and Approval of

Preliminary Plan					100

2.  Preliminary Map Content and

Supplementary Information			101

3.  Approval of Final Plat				102

4.  Deed Transfers					105

III.  SUBDIVISION DESIGN STANDARDS				106

SECTION	1.  General						106

2.  Streets						106

3.  Blocks and Lots					109

4.  Building Lines and Easements			109

5.  Public Sites and Open Spaces			110

6.  Land Subject to Flooding or Containing

Poor Drainage Facilities				111

7.  Compatability of Adjacent Land Uses		112

8.  Variances						112

# TABLE OF CONTENTS - - continued

SUBJECT									PAGE

EXHIBITS	1.  Angles of Street Intersections

2.  Typical Block Design

3.  Horizontal Road Alignment

4.  Minimum Vertical Visibility

5.  Minimum Horizontal Visibility

IV.  MINIMUM REQUIRED IMPROVEMENTS				113

SECTION	1.  General 						113

2.  Street Improvements				113

3.  Sidewalks						114

4.  Drainage						114

5.  Water Supply					115

6.  Sanitary Sewers					116

7.  Monuments					116

8.  Utilities and other Improvements			116

9.  Improvements Bond and Insurance		118

10.  Provisions for Maintenance and Operations	119

11.  Variances						119

V.  ENFORCEMENT								120

SECTION	1.  Recording of Plat					120

2.  Sale of Land in Subdivision			120

3.  Permits						120

4.  Public Improvements				120

5.  Revision of Plat After Approval			120

6.  Fees						121

7.  Penalties						121

8.  Validity						122

SUBDIVISION REGULATIONS

FOR

MADISON VILLAGE, OHIO

A resolution prescribing minimum requirements for the design and development of new subdivisions and of resubdivisions within the incorporated area of Madison Village, providing for preliminary and final approval of all subdivisions plats, providing for the enforcement of these regulations and providing for changes and amendments.

Be it hereby resolved by the Planning Commission of Madison Village, Ohio:

ARTICLE I

GENERAL PROVISIONS

SECTION	1.  Authority

The Madison Village Planning Commission, hereinafter known as Commission, derives its authority by virtue of Chapter 711 and more specifically Sections 711.10 and 711.101 of the Revised Code.

## SECTION 	2.  Short Title

This resolution shall be known and may be cited as the "Subdivision Regulations of Madison Village, Ohio" and shall hereinafter be referred to as "these Regulations".

## SECTION	3.  Interpretation and Purpose

In their interpretation and application, the provisions of these Regulations shall be held to be the minimum requirements adopted for the protection of the public health, safety and welfare.  To protect the public, among other purposes, such provisions are intended to provide for beneficial growth of the Community environment, adequate municipal services, and adequate streets, highways, and public utilities.

## SECTION	4.  Effective Date

These Regulations shall be effective following adoption by the Council of Madison Village and the Commission.  Henceforth any other regulations previously adopted by the Council or the Commission shall be deemed to be repealed.

## SECTION	5.  Scope

These Regulations shall apply to any lot or lots forming a subdivision or part thereof.  It is not intended by these Regulations to repeal, abrogate, annul or in any way impair or interfere with existing provisions of other laws, regulations or ordinances, except those specifically repealed by these Regulations, or with private restrictions placed upon property by deed, covenant or other private agreement, or with restrictive covenants running with the land to which the Municipality is a party.  Where these Regulations impose a greater restriction upon land than is imposed or required by such existing provisions of law, ordinance, contract or deed, the provisions of these Regulations shall control.  These Regulations shall govern the development of all land uses and to develop a sound and wholesome community by regulating the design and construction of all new subdivisions or development projects.

## SECTION	6.  Jurisdiction

- It shall be unlawful for any person to subdivide any land within the Village Corporate limits unless it be by a plat complying with the regulations herein contained.  Plat shall be recorded and no lot or land shall be sold from any such plat until said plat has been approved as herein required.

- It shall be unlawful for any person to circumvent the provisions of this ordinance by lease or by sale of parcels, by metes and bounds.

3.  	The design and configuration of all subdivisions shall conform with the requirements of Article III hereof.  The subdivider shall submit plats in accordance with Article II of these Regulations.  All plats shall be approved as herein required and recorded by the Lake County Recorder.

4.  	The division of a tract into four (4) parcels or less fronting upon an existing street, and where the transfer is to be accomplished by the recording of a deed rather than a plat, requires the approval of the Commission before it can be recorded.  Such approval may be accomplished in the manner provided in Section 4 of Article II of these Regulations.

SECTION	7.  Authority and Administration

The Village Council and the Village Commission by virtue of the provisions of Chapter 711 of the Revised Code of the State of Ohio are authorized, subject to the holding of required public hearings, to adopt rules and regulations covering plats for subdivisions of land, and to approve, conditionally approve, or disapprove plats or subdivisions of land falling within their jurisdiction.  Administration of the rules and regulations contained herein shall be the responsibility of the Commission.

The Planning Commission, by virtue of the authority granted by the Statutes, Chapters 713 and 735 of the Revised Code of Ohio, or such statutes hereinafter in effect, may prepare plans for unsubdivided or improperly subdivided areas to show a pattern of future streets and public open spaces.  After the required public hearing has been held, such plans or portions thereof, may be adopted by the Commission, recorded, and thereafter no subdivision may be approved and no street may be improved or accepted, except that it conform substantially to such recorded plan.  No permit may be issued for construction of a building or structure on land shown within the right-of-way has been acquired or reserved, however such permit may not be withheld for more than ninety (90) days or such extended time as is mutually agreeable by the Village and the owner.

SECTION	8.  Variances and Exceptions

1.	Whenever the tract to be subdivided is of such unusual size or shape, or is surrounded by such development or unusual conditions that the strict application of the requirements of these Regulations would result in real difficulties, substantial hardship, or injustice, such requirements may be modified in a reasonable manner but at the same time, maintain the public welfare and interest of the Municipality and the surrounding area and that the general intent and spirit of these Regulations are enforced.

2.  	The requirements of these Regulations may also be modified and varied whenever the scope of a plat encompasses a complete neighborhood or community as long as such plat conforms to the Comprehensive Plan of Madison Village.  All land uses as indicated in the Comprehensive Plan shall be incorporated on the plat and must conform, in general, to the areas of land usage as delineated and adopted by the Commission and the Council. The Official Thoroughfare Plan and the Comprehensive Plan shall be the guide for all future development or redevelopment in Madison Village.  Any such variance requested must insure that adequate public spaces, circulation, recreations, light and air will be provided and the needs of the entire community when fully developed and occupied will be fully met.

SECTION	9.  Amendments

The Council may, after public hearing, amend, supplement or change these Regulations.  Notice shall be given of the time and place of such public hearing by publication in at least one (1) newspaper of general circulation published in the local area, at least fifteen (15) days prior to holding of said hearing.  The amendment or amendments shall be on file in the office of the Commission for public examination during said fifteen (15) days.

SECTION	10.  Definitions

1.  Block.  All that part of one side of a street which is between two intersecting streets.

2.  Clerk.  The Clerk of Council of Madison Village, Ohio.

3.  Council.  The Municipal Council (Legislative Authority) of Madison Village, Ohio.

4.  County.  Lake County, State of Ohio.

5.  Record Plat.  The Recorder of Lake County, Ohio, will file the final map, drawing, or chart on which the subdivider’s plan of subdivision is presented to the Planning Commission for approval, and which, if approved.

6.  Lot.  A parcel of land intended for transfer of ownership or building development, having its full frontage subject to the application of these Regulations.

7.  Master Plan.  The comprehensive plan (which may consist of several maps, data, etc.) or any portion thereof, made and adopted by the Planning Commission of Madison Village and which shows the general location and extent of physical facilities including major streets and main thoroughfares, parks, schools, and other public open spaces, and public building sites.

8.  Municipality.  The Municipality of Madison Village, Ohio.

9.  Mayor.  The Mayor of Madison Village, Ohio.

10.  Official Map.  The map established by the Planning Commission showing streets, highways, and parks theretofore laid out, adopted and established by law and any amendments thereto adopted by the Council and the Commission thereto resulting from the approval of subdivision plats by the Planning Commission and the subsequent filing of such approved plats.

- Official Thoroughfare Plan.  The officially adopted plan designating a system of principal arterial,

collector, or local streets.

- Owner.  Any individual, firm, association, syndicate, co-partnership, corporation, trust or any other

legal entity having sufficient proprietary interest in the land sought to be subdivided to commence and maintain proceedings to subdivide to commence and maintain proceedings to subdivide the same under these Regulations;  subdivider; developer.

- Planning Commission or Commission.  The Planning Commission of Madison Village, Ohio.

- Preliminary Plat.  The preliminary plan or drawing indicating the proposed layout of the subdivision including lots, streets, and utilities.

- Solicitor.  The Solicitor or legal advisor of Madison Village.

- Street.  A way for vehicular traffic, whether designated as a street, highway, thoroughfare, parkway, throughway, road, avenue, boulevard, lane, place or however otherwise designated.

- Arterial Street.  A principle or heavy traffic street or considerable continuity and used primarily as an interconnect between large districts or other communities.

- Collector Street.  A street which carries traffic from minor streets to arterial or major streets, including the principal entrance streets of a residential development and streets for circulation within such a development.

- Local Street.  A street used primarily for access to the abutting properties.

- Cul-de-Sac or Dead-end-Street.  A local street with only one outlet.

- Street Width.  The width of public right-of-way.

- Subdivider.  Any individual, firm, association, syndicate, co-partnership, corporation, trust or any other legal entity commencing proceedings under these Regulations to effect a subdivision of land hereunder for himself or for another;  owner;  developer.

- Subdivision.

- The division or subdivision of any parcel of land shown as a unit or as contiguous units on the last preceding tax roll, into two (2) or more parcels, sites or lots, any one of which is less than five (5) acres for the purposes of transfer of ownership, lease, or development now or in the future.  The division or partition of land into parcels of more than five (5) acres not involving any new streets or easements of access, and the sale or exchange of parcels between adjoining lot owners, where such sale or exchange does not create additional building sites, shall be exempted.

- The improvement of one or more parcels of land for residential, commercial or industrial structures or groups of structures involving the division or allocation of land for the opening, widening or extension of any street or streets, except private streets serving industrial structures;  the division or allocation of land as open spaces for common use by owners, occupants or lease holders or as easements for the extension and maintenance of public sewer, water, storm drainage or other public facilities.

SECTION 	11.  Separability

If any section, subsection, sentence, clause, phrase, or portion of these Regulations is for any reason held invalid or unconstitutional by any court of competent jurisdiction, such portion shall be deemed a separate, distinct and independent provision and such holding shall not affect the validity of the remaining portions hereof.

SECTION 	12.  Public Hearing

The Commission, on its own initiative or upon petition by any citizen or neighboring property owner, may, prior to acting on a preliminary plat of a subdivision, hold a hearing thereon at such time and upon such notice as required by law.

ARTICLE II

PROCEDURE

SECTION	1.  Submission and Approval of Preliminary Plan

- General.  Prior to the preparation and submission of the preliminary plat, the subdivider shall meet with the Commission and Municipal Officials to become familiar with subdivision requirements and with the proposals of the Official Thoroughfare Plan for Madison Village, Ohio, affecting the territory in which the proposed subdivision is located.

- Filing.  The subdivider shall prepare and file ten (10) copies of a preliminary plat, according to the standards and other requirements of these Regulations, and submit the filing fee (see Article V), not less than twenty (20) calendar days prior to the Commission’s next regularly scheduled meeting date.  The Preliminary Plat shall be considered officially filed only after it is examined by the Commission’s authorized agent and is found to contain all of the data as required by Section 2, Article II of these Regulations.

- Approval.  The Commission’s authorized agent shall forward copies of the officially filed preliminary plat to such officials, and agencies as may be necessary for the purpose of study and recommendation.  Any costs or fees incurred by the Village as part of the review beyond the amount paid for the review fee shall be paid by the subdivider.  After receipt of reports from such officials and agencies, the Commission shall determine whether the preliminary map shall be approved, approved with modifications or disapproved.  Notice of such action shall be supplied to the subdivider together with the reasons for disapproval.  If modifications are required, such modifications shall be included in a revised plan which shall be submitted to the Commission.

The Commission shall act on the preliminary plat within eighty-five (85) days after filing unless such time is extended by agreement with the subdivider or his agent.

When a preliminary plat has been approved by the Commission, the Chairman of the Commission shall affix his signature to the plat and attach thereto a notation that it has received preliminary approval and return one (1) copy to the subdivider for compliance with final approval requirements.  Approval of the preliminary plat by the Commission shall not constitute approval of the final plan of the subdivision by the Commission.

Preliminary approval shall confer upon the subdivider the right for a one (1) year period from the date of approval the guarantee that the general terms and conditions under which the preliminary approval was granted will not be affected by any change and/or amendments to these Regulations.

SECTION	2.  Preliminary Map Content and Supplementary Information

- Form.  The preliminary plat shall be clearly and legibly drawn.  The size of the map shall be on one (1) or more sheets twenty-four (24) inches by thirty-six (36) inches.  All subdivisions shall be drawn at a scale of one (1) inch equals one hundred (100) feet, unless otherwise required by the Commission.

- Coverage.  The preliminary plan shall include a sketch of all land intended for ultimate development, even though only a portion of the tract is to be initially recorded.

- Map Contents.  The preliminary plat shall contain the following information:

a.	Proposed name of the subdivision.  The name shall not duplicate, be the same in spelling or alike in pronunciation with any other recorded subdivision in Madison Village of Township.

b.	Location by section, town, range, township, county and state.

c.	Names and addresses of the subdivider, owner and registered engineer preparing the plan.

d.	Scale of the plan, north arrow, and date.

e.	Boundaries of the subdivision indicated by a heavy line and the approximate acreage.

f.	Location, widths, and names of existing or platted streets, railroad, rights-of-way, easements, parks, permanent buildings, section and corporation tract, town, range, township tract, town, range and township lines.

g.	Names of adjacent subdivisions and owners (as of last preceding tax roll) of adjoining parcels of unsubdivided land.

h.	Zoning districts, if any.

i.	Existing contours with intervals of not more than five (5) feet where the slope is greater than ten (10) percent and not more than two (2) feet where the slope is less than ten (10) percent.  Elevations are to be based on sea level datum if available.  Source of reference used shall be indicated.

j.	Drainage channels, wooded areas, power transmission poles and lines, proposed and/or existing sanitary sewers, storm drainage and water lines and any other significant items should be shown.

k.	Building setback lines with dimensions.

l.	When lots are located on a curve or when side lot lines are at angles other than ninety (90) degrees, the width at the building line shall be shown.

m.	Vicinity sketch.

n.	Wetland delineation, if any.

- Staging (phasing) plan.

- Supplementary Information.

a.	Statement of the proposed use of lots, stating type of residential buildings with number of proposed dwelling units;  type of business or industry;  so as to reveal the effect of the development on traffic, fire hazards, increase in school enrollment or congestion of population.

b.	Proposed covenants and restrictions.

c.	Water supply, sewage disposal, surface drainage and flood control.

d.	If any zoning changes are contemplated by the subdivider, the proposed zoning shall be outlined.

e.	After receiving notice of the approval of the preliminary plat and prior to the filing of the final plat, the subdivider shall present tot he Commission detailed engineering drawings of all utility and street improvements to be constructed in the proposed subdivision as required by the Village standards.  The Commission shall within ten (10) days after the filing of the above detailed plans, transmit copies of same to the Village Engineer for study and final recommendations.  The Commission, after receiving a report from the Village Engineer, shall notify the subdivider of any required changes or modifications for preparation of the final improvement plans and record plat.

SECTION	3.  Approval of Final Plat

- General.  The final plat shall have incorporated all changes or modifications required by the Planning Commission, otherwise it shall conform to the preliminary plat.  It shall constitute only that portion of the approved preliminary plat which the subdivider proposes to record and begin immediate development thereof.  It shall contain the information required by Paragraphs 6 and 7 of this Section and shall conform with the laws and regulations in effect at the time the final plat of same is approved.

- Preparation.  The final plat shall be prepared by a registered surveyor.

- Filing.

- Twelve (12) copies of the final plat shall be filed not later than twelve (12) months after the date of approval of the preliminary plan unless an extension in writing is requested by the developer and granted by the Commission.

- The final plat shall be considered officially filed after it is examined by the Commission’s authorized agent and is found to contain all of the data as required by Section 3, Paragraphs 6 and 7, Article II of those Regulations.  The final plat shall be filed not less than thirty (30) working days prior to the Commission’s regularly scheduled meeting at which it is to be considered.

- Approval.

- The Commission’s authorized agent shall forward copies of the final plan to the Village Engineer and Solicitor for the purpose of final approval.  After approval from the Village Engineer and Solicitor, the Commission shall take action on the final plat within sixty (60) days; otherwise said plat shall be deemed to have been approved.  The certificate of the Commission as to the date of the submission of the plat for approval, and the failure to take action within such time, shall be sufficient in lieu of the written endorsement or evidence of approval herein required.  If disapproved, the grounds for disapproval of the final plat shall be stated on the record of the Commission, including the reference to the regulation violated by the plat a copy of which shall be forwarded to the subdivider.  The subdivider shall make necessary correction and resubmit the final plat within thirty (30) days to the Commission for its final approval.

- The subdivider shall be notified of the final approval of the plat by the Commission who, in turn, shall then present the plat to Village Council for approval.

- Form.  The final plan shall be clearly and legibly drawn on mylar or tracing cloth.  The size of the plan shall be on one or more sheets twenty-four (24) inches by thirty-six (36) inches.  The final plan shall be drawn at a suitable scale no less than one (1) inch equals one hundred (100) feet.

- Plat Contents.  The final plat shall contain the following information.

- Name of the subdivision; location by tract, section, town, range, township, county, state; and the scale, date and northpoint.

- All plat boundaries with length of courses in feet and hundredths and bearings to not more than half minutes.

- Bearings and distances to the nearest established street lines, section corners or other recognized permanent monuments, which shall be accurately described on the plat.

- Municipal, township, county, or section lines accurately tied to the lines of the subdivision by distances and bearings.

- Lines of adjoining streets and alleys with their width and names.

- The radii, arcs, chords and chord bearings, point of tangency and central angles for all curvilinear streets and radii for rounded corners.

- All easements for rights-of-way provided for public services or utilities, and any limitations of such easements.

- All lot numbers and lines, with accurate dimensions in feet and hundredths, and with bearings.  The basis of bearings shall be stated on plat.

- Accurate location to all monuments.

- Location of all set-back lines from public right-of-way.

- Summary of any areas to be dedicated or reserved for public use with the purpose indicated thereon.

- Supplementary Information.

- Protective covenants shall be shown on the plat.

- If a zoning change is involved, certification from the Village Clerk shall be required indicating that the change has been approved and is in effect.

- Certification by a registered surveyor to the effect that the plan represents a survey made by him, and that all monuments shown thereon actually exist and that their location is correctly shown.

- An acknowledgement by the owner or owners of his or their adoption of the plat, and the dedication of streets and other public areas.

- The final plat shall be accompanied by the following:

- Evidence that all taxes against proposed roads and park lands have been paid and evidence that all encumbrances or liens of record against such proposed areas have been released.

- That all improvements have either been installed and approved by the Village Engineer or that a suitable performance bond insuring their installation has been accepted by the Village Council.

- Recording of Final Plat.  After the final plat has been approved by the Commission and Council and the necessary approvals endorsed in writing thereon, together with evidence of title, it may then be filed by the Village in the Office of the Recorder of Lake County, Ohio, as required by law.  All fees to be paid by developer.

SECTION 	4.  Deed Transfers

Whenever a division is to be made of a tract into parcels, as described in Section 6, Paragraph 4, Article I, fronting on an existing public street, and where the transfer is to be accomplished by the recording of a deed rather than a plat, the party or parties desiring to make the transfer shall submit a sketch thereof to the Commission’s authorized agent.  If the division conforms to the appropriate standards, the responsible official may stamp same “Approved, No Plat Required” and attach his signature, and the appropriate transfer may then be recorded.  In NO case where a division takes place, may the remaining portion of the tract be less than the minimum allowable lot size.  However, if the owner of the property does not agree with any of the requirements specified by the Engineer/Commission, he may then appeal to the Commission for approval for recording and such approval must be obtained before the transfer can be recorded.  No preliminary of final plat other than the rough sketch of the division shall be required and none of the improvements described in Article IV shall be required for the transfer.

ARTICLE III

SUBDIVISION DESIGN STANDARDS

SECTION	1.  General

The arrangement, character, extent, width and location of major, secondary and minor streets or highways shall conform with the Official Plan for Madison Village, Ohio and with the Provisions of these Regulations.  Streets not contained in the Official Thoroughfare Plan should conform to the recommendation of the Commission based on existing and planned streets, topography, public safety and convenience, and proposed uses of land.

Due regard shall be shown for the preservation of existing natural features, distinctive site characteristics, outstanding scenic, cultural or historic areas, and all such features shall be integrated into the plan to create functional variations in the neighborhood and more attractive building sites.

SECTION	2.  Streets

1.	Design and Arrangement

a.	The arrangement of streets in new subdivisions shall make provision for the continuation of the principal existing streets in adjoining areas (and/or their proper projection where adjoining land is not subdivided) insofar as they may be deemed necessary for public requirements.

b.	The street arrangement shall be such as to not cause hardship to owners of adjoining property as to zoning, access to pubic right-of-way, or use.  Residential streets shall be so designed as to discourage through traffic but offset streets should be avoided except as they are separated by not less than one hundred and fifty (150) feet at their access points on a collector street.

c.	The angle of intersection between minor streets and major streets should not be less than eighty (80) degrees.

d.	All other streets should intersect each other as near to a right angle as possible and no intersection of streets at angles of less than seventy (70) degrees shall be permitted.

e.	Residential streets shall be designed to discourage through traffic which may otherwise use secondary or major highways, and whose origin and destination are not within the subdivision.  Residential streets extending for considerable distance, parallel to any secondary or major street, should be avoided.

f.	Streets planned to serve multi-family, business or industrial areas shall connect directly to Major or Collector streets, so as to not generate large volumes of traffic on local residential streets.

2.	Alignment

a.	For main thoroughfare, profile grades shall be connected by vertical curves where the algebraic difference in grades is 0.5% or more.

b.	Minimum Horizontal-radii of centerline curvature:

Arterial Highways		475 ft.

Collector Streets		300 ft.

Local Streets 			200 ft.

Streets Shorter than 500 ft.	100 ft.

A minimum 50’ tangent shall be introduced between reverse curves.

c.	Visibility Requirements:	(Vertical and Horizontal)

(1)	Minimum visibility (measured 4 ½ ft. eye level to tail light 18” above ground level.)

Vertical		Horizontal

Arterial Streets		  500			      500

Collector Streets		  300			      300

Local Streets			  200			      100

3.	Street Width

a.	The width of all major thoroughfares shall conform to the width designated on the major thoroughfare plan of Madison Village.

b.	The minimum right-of-way widths for streets are as follows:

Type of Street or Way				Minimum Right-of-Way

Arterial highway or major thoroughfare

(see major thoroughfare plan)				Variable

Secondary or Collector Streets

(see major thoroughfare plan)				80 ft.

Minor or Local Urban Streets				60 ft.

Cul-De-Sacs						60 ft.

c.	Cul-de-Sac:  Each cul-de-sac shall be provided with a turn-around having a minimum right-of-way radius of sixty (60) feet and maximum length of seven hundred (700) feet measured from the centerline of the entering intersection and including the turn-around.  The outside of the road surface within the turn-around right-of-way shall have a minimum radius of fifty (50) feet.  The diameter of the median area shall be not less than fifty-two (52) feet.  Each cul-de-sac shall have a median area, the scope and design of which shall require the prior approval of the Planning Commission.  Said median area shall consist of open space, shall be aesthetically landscaped and shall be compatible with its surroundings.  The developer/subdivider shall provide, to the satisfaction of the Planning Commission, adequate and sufficient assurance that said median area will be continuously maintained in a good, proper and safe condition and in a reasonable state of repair.  The developer/subdivider shall provide, to the satisfaction of the Planning Commission, adequate and sufficient assurance that responsible persons or entities will continuously own, maintain and be responsible for said median area.  The assurances required herein may include, but are not limited to, (1) deposit of sufficient cash; (2) posting of adequate bond; (3) creation of homeowner or condominium association; (4) trust agreement appointing trustee to act for all owners; or, (5) common easements and deed restrictions.  (Amended by Ordinance No. 04-1999, Passed February 15, 1999:  An Ordinance amending Section 2.3(c) of Article III of the Madison Village Subdivision Regulations relating to Cul-de-Sacs.)

d.	Dead-End Streets:  Streets designated to be dead ended permanently shall not be permitted.  Any dead-end street of a temporary nature, if longer than two hundred (200) feet or fronted by existing lots, shall have a surface turning area equal to that of a permanent cul-de-sac.  Building on lots fronting on a temporary cul-de-sac is prohibited.

e.	Street Grades:  (Vertical Curves):  No street grade shall be less than 0.6 percent and shall not exceed the following with due allowances for reasonable vertical curves:

Main or arterial thoroughfares		6%

Secondary thoroughfares			8%

Local Streets					8%

Cul-de-Sacs					8%

f.	Street Names:  Where names are required for new streets, they shall meet with the approval of the Planning Commission and Village Council.

SECTION	3.  Blocks and Lots

- Block Design.

- No block shall be longer than fourteen hundred (1,400) feet or less than five hundred (500) feet, except in unusual circumstances.  Where a subdivision adjoins a major highway, the greater dimension of the block shall front along such major highway to minimize the number of points of ingress or egress.

- Blocks for commercial or industrial subdivisions shall be designed to accommodate the building sites and provide the yards, service drives, required off-street parking and other such required facilities for unit development rather than typical lot and street pattern.

2.	Lots

- The lot arrangement and design shall be such that all lots will provide satisfactory and desirable building sites, properly related to topography and the character of surrounding development.

- All side lines of lots shall be at approximate right angles to straight street lines and radial to curved street lines except where a variation to this rule will provide a better street and lot layout.  Lots with double frontage shall be avoided.

- No lot shall have less area or width than is required by the zoning regulations applying to the area in which it is located.

- In case of unusual soil conditions or other physical factors which may impair the health and safety of the neighborhood in which a subdivision may be located, the Planning Commission may require larger lot widths and lot areas.

- No corner lot shall have a width at the building line of less than seventy five (75) feet.  Either of the two (2) sides of a corner lot fronting on a street may be designated the front of a lot, provided rear yard shall always be opposite the frontage so designated.

- All corner lots shall have a curve with a minimum radius of thirty (30) feet joining in the two (2) sidelines of said right-of-way.

SECTION	4.  Building Lines and Easements

- Building Lines.  Building lines shall be established according to the front yard requirement set forth in the Municipal Zoning Ordinance.

- Easements.  Easements of at least ten (10) feet in width shall be provided on each side of all front lot lines for underground electric, gas, telephone, and CATV.  Easement s may also be required along or across lots where engineering design or special conditions may necessitate the installation of water and sewer lines outside public rights-of-way.

- Easement Along Streams.  Whenever any stream or drainage course is located in the area being subdivided, the subdivider shall provide an adequate easement along each side of the stream for the purpose of widening, deepening, relocating or protecting the stream for drainage or public use.  Areas within the Village 100 year floodway shall be within an easement.

SECTION 	5.  Public Sites and Open Spaces

All proposed residential plats submitted for approval under the provisions of these regulations shall allocate adequate open space areas in conformity with the Comprehensive Plan or as determined by the Planning Commission.

- Open spaces shall consist of no less than five (5) percent of the total land in the subdivision and shall be usable for such purposes as determined by the Planning Commission.

- In the event the 5% area equals less than one acre or where it is deemed to be in the best interest of the Village, a fee in lieu of the land may be substituted.

Where a fee is required in lieu of land area, that fee shall be paid to the Village and used for capital improvements of parks and recreation areas as directed by Village Council.  The fee shall be in a sum equal to the value of five (5) percent of the total area of the subdivision considered to be triple the current tax valuation of the undeveloped land.

- Land unusable for building sites, as determined by the Planning Commission may be designated as open space but in no case shall this designation constitute more than forty (40) percent of the open space requirement.

- If a subdivision is developed to a density of one dwelling unit or less per acres, open space may not be required by the Planning Commission.

- If the developer creates a subdivision to be developed in phases, the open space requirement shall be met for each phase however the developer may be permitted to Reconfigure and/or relocate the designated open space between phases to accommodate both an efficient lot layout and open space.  All modifications shall require a resubmittal of the Preliminary Plat and be approved by the Planning Commission.

- Optional means of providing open space:

- Residents associations with common deed:  Restricted open space may be held in fee, improved and maintained by an organization created by means of a homeowner or condominium association.  Such an organization shall include provisions for compulsory membership for all residents based upon the ownership or long-term lease of housing units.  Voting rights shall be distributed on an equitable basis.  The organization shall have a structure to permit effective participation of the residents.  Fees for the preservation, improvement and maintenance of the common open space shall be stated with adequate provision for collection.

- Trust agreement:  The developer may deed title to all common open space and recreational facilities to a fiduciary which, for a fee, acts as trustees for the benefit of all owners and occupants of the subdivision.  The trustee shall give easements across the open space and right to use the facilities to all owners and occupants of the subdivision.  Each owner shall receive a fee simple deed from the trustee, subject to the trustee’s right to charge and lien each property for its proportionate share of upkeep costs for the common facilities.

- Public Donations:  Common open space may be donated, in fee simple or by a perpetual conservation or preservation restriction subject to the acceptance by a governmental unit or agency.

- Common Easements:  Common open space may be provided by means of common easement.  Deed restrictions as finally approved and recorded shall be filed with the Commission.  Costs of preservation, improvement and maintenance shall be the responsibility of the owners of the property; however, an association of residents may be created to assess charges on an equitable basis.

- Dedication:  Dedication to public use.

SECTION	6.  Land Subject to Flooding or Containing Poor Drainage Facilities

The right is reserved to disapprove any subdivision which is subject to periodic flooding or which contains extremely poor drainage facilities.  If the subdivider agrees to make such improvements as will make the area completely safe for residential occupancy, the subdivision may be approved, subject, however, to the approval of the Village Engineer.

SECTION 	7.  Compatibility of Adjacent Land Uses

It shall be at the discretion of the Commission to permit noncompatible land uses to develop on adjacent properties.  Where such a condition arises, buffer zones shall be required to protect the land uses from infringement, one upon the other.  Buffer zones shall be established according to zoned land uses adjacent one to another and shall be as set forth in the Municipal Zoning Ordinance.

SECTION 	8.  Variances

Where topography and other physical conditions make full compliance with the subdivision design standards of Article III and undue hardship upon the owner, the Commission may refer the problem of compliance to the Village Engineer and the Council for recommendations as to permissible variances.

ARTICLE IV

MINIMUM REQUIRED IMPROVEMENTS

SECTION	1.  General

Prior to the granting of final approval, the subdivider shall have installed or shall have furnished adequate bond for the ultimate installation of the improvements listed and described in the Sections of this Article.  All of the required improvements listed and described in the Sections of this Article.  All of the required improvements shall be made in full compliance with the specifications for each of the various units of work as required by the Village Engineer.

The inspection authority shall rest with the Village Engineer except where otherwise authorized by law, agreement, or these regulations.  All inspection costs shall be borne by the subdivider.  Private streets are permitted in subdivisions provided they meet all of the requirements of these regulations except dedication of the right of way to public use.

SECTION 	2.  Street Improvements

All streets and thoroughfares shall be improved in accordance with the standards outlined or referred to in these Regulations.  Plans and specifications for street improvements shall be subject to approval by the Village Engineer.  All materials and construction procedures shall be in accordance with the current edition of “Construction and Materials Specifications” of the State of Ohio Department of Transportation.

- Width of Pavement.  The width of the pavement will vary, depending upon the character of the development served and the amount of traffic expected to utilize the street.  The following are the minimum street pavement widths:

Type of Street				Minimum Pavement Width

Arterial Streets				Variable

Collector Streets				     28’

Local Streets					     24’

- Base Course.  The developer may use any of the following base courses, based upon recommendations of the Village Engineer as to soil and traffic conditions:  aggregate, bituminous aggregate, asphaltic concrete, portland cement concrete, or an equivalent base course.  Thickness shall be subject to the approval of the Village Engineer, based upon the physical properties of the base course used and the physical properties of the roadbed as determined by geotechnical data provided by the developer.

- Surface Course.  Upon the expiration of a maintenance period for the base course, as established by the Village Engineer, the surface course shall be constructed using either asphaltic concrete or portland cement concrete.  Specific material and thickness specifications shall be subject to approval of the Village Engineer based upon traffic conditions:

- Concrete Pavement:  If the subdivider elects to construct streets totally out of portland cement concrete of if such pavement is required by the Village Engineer, thicknesses of seven (7) inches for local, collector streets, and nine (9) inches for major commercial and industrial streets shall be required.  The Village Planning Commission may require pavement of greater thickness, upon the recommendation of the Village Engineer based upon his evaluation of the subgrade, traffic, and projected wheel load conditions.

- Full-Depth Asphalt Pavement:  If streets to be constructed out of “full-depth” asphalt, an asphalt pavement in which asphalt-aggregate mixtures are used for all courses above the subgrade, careful inspection of the subgrade may be necessary, to determine pavement thicknesses.  Minimum, pavement depths shall be as follows:

Local Street				8”

Collector Street			9”

Major & Industrial Street		As per Engineer

The Village Planning Commission may require pavement of greater thickness upon the recommendation of the Village Engineer based upon his evaluation of the subgrade, traffic, and wheel load conditions.

- Curbs and Gutters.  Curbs and gutters shall be required on all streets.  Curbs, combined curbs and gutters shall be constructed of concrete in conformance with the current “Construction and Material Specifications” of the Ohio Department of Transportation or as specifically modified by the Village Engineer.

SECTION	3.  Sidewalks

Concrete sidewalks having a minimum width of five (5) feet and having minimum thickness of five (5) inches shall be installed along both sides of major streets or thoroughfares and along both sides of other streets.  Sidewalks at the driveway shall have a minimum thickness of six (6) inches.

SECTION	4.  Drainage

All necessary facilities, including underground pipe, inlets, catch basins, retention basins, or open drainage ditches, as determined by the Village Engineer, shall be installed to provide for the adequate disposal of subsurface and surface water and maintenance of natural drainage courses.  Surface water run off shall be controlled such that there shall be no greater burden on lower property owners after development than there was before development.  All storm drainage facilities within the subdivision shall connect to an adequate drainage course.

- Storm Sewers and Storm Water Drainage.  A drainage system adequate to serve the needs of the proposed new streets and the entire subdivision will be required in new subdivisions.  Where an adequate public storm sewer main is available at the plat boundary, the subdivider shall construct a storm sewer system and connect with such storm sewer main.  If such storm sewer systems are not accessible, adequate storm water drainage shall be provided by natural drainage channels with easements of adequate width as determined by the Village Engineer and approved by the Commission.  Where drainage ditches or storm sewers cross land not in the dedicated street right-of-way, appropriate easements shall be given to the Village for such drainage purposes.

- Culverts and Bridges.  When natural drainage channels intersect any street right-of-way, it shall be the responsibility of the subdivider to have satisfactory bridges and culverts designed and constructed.  Where culverts are required, the following minimum requirements shall be observed:

- All culverts, including replacement culverts, shall extend across the entire right-of-way width of the proposed road.  The location, type, and capacity shall be determined by the Village Engineer.  All culverts or bridges serving 100 year floodway shall be designed for the 100 year capacity.

- Standards and Specifications for Drainage Facilities.  The construction of storm water drainage improvements shall be subject to the approval of the Village.

SECTION	5.  Water Supply

A water supply, adequate to provide the required flow demands including fire protection, shall be available or proposed as part of the improvements.  The Planning Commission may require of the developer the construction of main extensions, interconnects, or booster stations to provide sufficient water supply based upon the recommendations of the Village Engineer.

All water supply improvements shall meet the criteria of the Village Engineer, Madison Fire Chief, and the Lake County Utility Department where applicable.

1.	Fire Protection.  Fire hydrants with two (2) and one half (2-1/2) inch outlets and one (1) large pumping connection shall be provided by the subdivider in all subdivisions and public water supplies.  The hydrants should be located between property lines and curbs with all outlets facing or parallel to the street.  Hydrants shall be placed at the corners of all blocks and at mid-blocks for blocks exceeding eight hundred (800) feet in length.  Hydrants shall also be required at the entrance and end of all cul-de-sacs four hundred (400) feet in length or longer.  Distance between hydrants shall not exceed three hundred fifty (350) feet.  The type and make of hydrant and control valves and the location and accessibility of the hydrant shall be approved by the Village Engineer.  The size of any water line serving any hydrant shall not be less than eight (8) inches in diameter and the line should be looped.  The size and location of water lines shall be approved by the Village Engineer.

SECTION 	6.  Sanitary Sewers

If a subdivision can reasonably be served by the extension of an existing public sanitary sewer, as determined by the Commission and in its sole discretion, the subdivider or developer shall provide a system of sanitary sewer mains and shall provide lateral connections for each lot.  In such cases septic systems shall not be permitted.

If required as determined by the Village Engineer, the developer may be required to install, replace, repair, or upgrade down stream sewer, pump stations, or force mains.

Where a public sanitary sewer is not reasonably accessible as determined by the Commission and in its sole discretion, the subdivider or developer shall provide a system of sewer mains and laterals whereby each lot may be served by the Village Wastewater Treatment Plant when sewer trunk lines are extended as provided for in the Comprehensive Sewerage Plan.  In such cases a septic system approved by the Lake County Health Department must also be installed, which may be used only until such time as a trunk line is extended to the subdivision.

All sewer systems must be approved and inspected by the Village Engineer.  Wherever main lines are installed, sewer and water mains shall be extended to the property lines of the subdivision.  Connections to public sanitary sewer lines shall be subject to the approval of and according to the specifications of the Village Engineer.

SECTION	7.  Monuments

Permanent monuments shall be accurately set and established at the intersections of all outside boundary lines of the plat;  at intersections of those boundary lines with all street lines at diagonally opposite corners of each street intersection;  at the beginning and end of all curves;  at points on curves where the radius or direction changes; and at such other points as are necessary to establish definitely all lines of the plat, except those outlining individual lots.  In general, permanent monuments shall be placed at all critical points necessary to correctly lay out any lot in the subdivision.  Monuments shall be in accordance with the minimum standards for Boundary Surveys  (ORC 4733 – 37) .

SECTION	8.  Utilities and other Improvements

- Utilities.  Electrical service, gas mains and other utilities shall be underground and shall extend to the property boundaries of the subdivision.

- House Numbers.  Each house shall have mounted on a conspicuous place thereon a “house number” which shall be visible from the street.  Said “house number” shall be in place before an occupancy permit will be issued.

- Street Identification Signs.  Street identification signs and traffic control devices with posts as required by the Planning Commission and the Village Engineer shall be provided by the developer and the cost of replacement, repair or maintenance shall be borne by the developer until the roadway is accepted for maintenance.

- Trees.  No less than one (1) tree with a minimum diameter of two (2) inches, measured one (1) foot above the ground shall be planted in each lot as approved by the Planning Commission.

- Sediment Control.  As part of the final improvement plans for a subdivision, the developer shall submit a plan that adequately provides for the control of sedimentation as a result of the project.  The plan shall include but not be limited to use of the following procedures and devices:

- Sedimentation basins at storm outlet locations.

- Silt fence and/or straw bales at locations susceptible to erosion.

- Temporary seeding requirements.

- Specific provisions for maintenance of erosion control devices.

- Staging plan to limit disturbed area(s).

- NPDES permit where required.

- Inclusion of a value for sedimentation control in the project bond.

All disturbed areas shall be final graded and seeded including right-of-way and easement areas prior to final release of escrow.

Developer shall remain responsible for the protection, cleaning, and repair of all sewers and streets during the total period of house construction.  Cleaning of catch basins and paved areas shall be required at any time deemed necessary by the Village Engineer.  If the cleaning does not occur within 48 hours of notice, the Village may cause the cleaning to be done, the cost for which shall be borne by the Developer.

- Street Lighting.  At the time of plat approval, the Developer shall submit a signed petition for the installation of street lights in the subdivision.

All costs for the installation of street lights shall be borne by the Developer.

7.	Subdivision Identification Signs.  Subdivision identification signs shall be required and shall be approved as set forth in Section 9.9 of the Madison Village Zoning Code.  (Amended by Ordinance No. 04-1999, Passed February 15, 1999:  An Ordinance amending Section 8, Article IV of the Madison Village Subdivision Regulations relating to Subdivision Identification signs.)

SECTION 	9.  Improvements Bond and Insurance

The subdivider may receive approval and acceptance of the plat provided the following conditions are met:

1a.	The subdivider shall deposit in a cash security with a bank or savings and loan in the amount of 100% of the approved estimated cost of the improvements plus the cost of maintaining said improvements until acceptance of said improvements by the Village which shall be valued at 10% of the construction cost.  The bank or savings and loan shall be directed to payout portions of the cash security towards the completion of said improvements only with the approval of the Village Engineer, who may require a balance to be maintained which in his opinion is sufficient to complete and maintain said improvements in accordance with the plans and specifications.  He shall authorize the release of any balance to the subdivider only after acceptance of said improvements by Council but in no case less than one year form final completion.

In the event said improvements are not constructed and/or not maintained, the Village shall have the option after thirty (30) days’ written notice to the subdivider to complete and maintain said improvements and to collect and receive from the bank or savings and loan a remaining balance of said deposit to be applied toward payment of costs and expenses of completing said improvements;  or

1b.	The subdivider may petition the Village to construct said improvements and access the total against the benefited lots.  This procedure may be followed upon condition that the subdivider furnishes the Village with a bond, with surety to the satisfaction of the Village Administrator, guaranteeing the payment of both the cost of design engineering and assessments as they fall due.  Said bond may provide that as houses are constructed and sold to the individual owners the amount of the bond shall be reduced not more than once each year in proportion to the amount of the assessment against such lot.  When houses have been completely constructed on seventy-five (75) percent of the lots the bond shall be released;  or

1c.	The subdivider may furnish corporate surety bond in the principal amount of the estimated cost of said improvements with surety to the satisfaction of the Village Administrator in such form as is approved by the Solicitor, guaranteeing completion and maintenance of said improvements according to plans, profiles and specifications and to the satisfaction of the Village Engineer.  Said bond shall remain in effect until said improvements have been completed and maintained for a period of one year minimum and/or until released by authority of Council.  Said bond shall provide that upon default of performance by the subdivider the Village may complete the same after thirty (30) days’ written notice.

- Progressive Installation.  The developer may apply for final approval and recording of only a portion of the entire subdivision.  Under such a progressive unit development procedure, the installation of requirements and sale or lease of lots may proceed only on that portion of the subdivision which has been approved and recorded.

- Failure to Complete Site Improvements  In the event the developer fails to complete the installation of all improvements according to the terms and conditions of the Agreement, the Village, upon proper notice, may complete same and appropriate such portion of money or bonds posted for the faithful performance of said work.  Any adjustments required in the elevation of sanitary manholes, catch basins and fire hydrants due to final grading shall be paid for by the subdivider.

SECTION	10.  Provisions for Maintenance and Operations

Where the subdivision contains sewers, sewage treatment plants, water supply systems, park areas, street trees or other physical facilities, necessary or desirable for the welfare of the area and which are of common use or benefit and which are of such character that the Village does not desire to maintain the, then provision shall be made by trust agreements, which are a part of the deed restrictions and which are acceptable to the Commission for the proper and continuous maintenance and supervision of such facility by the lot owners in the subdivisions.

SECTION 	11.  Variances

When topography and other physical conditions make full compliance with the minimum required improvements of Article IV an undue hardship upon the owner, the Commission may refer the problem of compliance to the Village Engineer and the Council for recommendations as to permissible variances.

ARTICLE V

ENFORCEMENT

SECTION	1.  Recording of Plat

No plat of any subdivision shall be entitled to be recorded in the office of the Recorder of Lake County or have any validity until it shall have been approved in the manner prescribed herein.  In the event any such unapproved plat is recorded, it shall be considered invalid and the Village Council shall institute proceedings to have the plat stricken from the records of Lake County, State of Ohio.

SECTION	2.  Sale of Land in Subdivision

No owner or agent of the owner of any land located within a subdivision shall transfer or sell any land by reference to, exhibition of, or by the use of a plan of plat of a subdivision before such plan or plat has been approved and recorded in the manner prescribed herein.  Any sale of transfer contrary to the provisions of this Section is void.  The description of such lot or parcel by metes and bounds in the instrument of transfer or other documents used in the process of selling or transferring shall not exempt the transaction from the provisions of these Regulations.

SECTION 	3.  Permits

Zoning or building permits shall not be issued for any structure on a lot in a subdivision for which a plat has not been approved and recorded in the manner prescribed herein.

SECTION 	4.  Public Improvements

The Village here defines its policy to be that they will withhold all public improvements of any nature, including the maintenance of streets and the furnishing of sewerage facilities and water service from all subdivisions which have not been approved, and from all areas dedicated to the public which have not been accepted by the Village Council in the manner prescribed herein.

SECTION	5.  Revision of Plat After Approval

No changes, erasures, modification or revisions shall be made in any plat or a subdivision after approval has been given by the Commission, and endorsed in writing on the plat, unless the said plat is first resubmitted to the Commission.

SECTION	6.  Fees

- At the time of submitting a preliminary plan, the subdivider shall pay a review fee and the amount of such fee shall be determined from the following schedule:

No. of Lots in Plat				Amount of Fee

2 – 10						$100.00

11 or more					$  10.00/lot

- The review fee shall be paid in legal tender or by check or money order made payable to the Village of Madison and deposited with the Clerk-Treasurer for transfer to the general fund.

- In the event that a plat is disapproved by the Commission before any physical inspection has been made of the site, then the Commission may – at its discretion – order that the developer be refunded an amount not to exceed one-half (1/2) of the fee paid to the Clerk-Treasurer.

- In addition tot he above review fee, the subdivider shall be charged actual cost for the review of detailed improvement plans for the cost of inspection of all improvements in the subdivision which fall under the jurisdiction of the Village Engineer.  The amount of the initial deposit and method of payment shall be as determined by the Village Administrator.

- The subdivider shall pay the actual cost of recording the plat(s) with the County Recorder.

SECTION 	7.  Penalties

- Whoever violates these Regulations adopted by Council or fails to comply with any order, pursuant thereto, shall forfeit and pay not less than one hundred ($100.00) dollars nor more  than one thousand ($1,000.00) dollars.  Such sum may be recovered with costs in a civil action brought in the Court of Common Pleas of the County.

- Whoever, being the owner or agent of the owner of any land within or without a municipal corporation, transfers any lot, parcel or tract of such land from or in accordance with a plat of a subdivision before such plat has been recorded in the office of the County Recorder, shall forfeit and pay the sum of not less than one hundred ($100.00) dollars nor more than five hundred ($500.00) dollars for each lot, parcel, or tract of land so sold.  The description of such lot, parcel, or tract by metes and bounds in the deed or transfer shall not serve to exempt the seller from the forfeiture provided in this Section.

- Any person who disposes of, offers for sale or leases for a time exceeding five (5) years any lot or any part of a lot I a subdivision before provisions of these Regulations are complied with, shall forfeit and pay the sum of not less than one hundred ($100.00) dollars nor more than five hundred ($500.00) dollars for each lot or part of a lot so sold, offered for sale or leased, to be recovered with costs in a civil action, in the name of the Village of Madison for the use of the Village.

- A County Recorder who records a plat contrary to the provisions of these Regulations shall forfeit and pay no less than one hundred ($100.00) dollars nor more than five hundred ($500.00) dollars, to be recovered with costs in a civil action by the Village Solicitor in the name and for the use of the Village.

SECTION	8.  Validity

If any section, subsection, sentence, clause, or phrase of these Regulations is for any reason held to be unconstitutional or void, such decision shall not affect the validity of the remaining portions of these Regulations.